| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0306 WG11 Number: m23181 |
| --- | --- |

| Title: | CE4: Cross-check report of subtest 2.2 for MediaTek on quantization matrices representation Method and syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianhua Zheng Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by HiSilicon for MediaTek proposal on quantization matrix representation method and syntax. The cross-check performance results match with those provided by the proponents and with those provided by other CE4 subtest2 crosschecker. The software provided by proponents has been investigated.

Without case4(random matrices), the proposed tools obtain 4.64x compression ratio against anchor with average reconstructed error 14.43; With case4(random matrices), the proposed tools obtain 4.93x compression ratio against anchor with average reconstructed error 13.72.

# Introduction

The proposal use implicitly derived method to generating 16x16 and 32x32 matrices from 4x4 and 8x8 matrices. The 4x4 and 8x8 matrices are hard coded in bitstream, and the 16x16 and 32x32 matrices are implicitly generated and are not really coded, hence have coding error in large size QM coding. The proposed method is a lossy method.

The configure setting are:

ScalingList                   : 2                    # ScalingList 0 : off, 1 : default, 2 : file read

ScalingListFile               : scaling_list_asymmetry1.txt    # Scaling List file name.

The macro setting are:

#define SCALING_LIST                  1 //JCTVC-G880/JCTVC-G1016 quantization matrices

#define SCALING_LIST_OUTPUT_RESULT    1 //JCTVC-G880/JCTVC-G1016 quantization matrices

#define SCALING_LIST_CODE_EVERY_FRAME 0 //code quantization matrices all frames

#define MTK_QM              1

#if MTK_QM

#define MTK_SCALING_LIST_SIZE_NUM   2

#define MTK_QM_INTERPOLATE  1

#define MTK_QM_REPEAT       0 //1

#define MTK_QM_ADAPTIVE     0 //1

#define MTK_QM_HVS_DEFAULT  0

# Simulation Results

The implementation is on HM 5.0-qmatrix branch software. Tests were conducted on CE4 subtest2 test1 condition[2]. Only first frame is loaded with one set of matrices for each runs. In simulation, totally 6 set of symmetric matrices and 6 set of asymmetric matrices are used, which are provided by Sony and Qualcomm. In those sets of matrices, case4 are random matrices, which included one set of random symmetric matrices and one set of asymmetric matrices. Whether or not to put case4 (random matrices) in tool evaluation was deeply discussed in CE4 reflector, three voices to remove the case4, and one voice to keep it in QMs test set. Since it is not got unified agreement, in the following CE simulation, case4 ( random matrices)is kept in test set, which is for the purpose of test the robustness of the methods.

The crosscheck report will include the summary results with/without case4 random matrices individually.

## TEST1 results

Table 1  Subtest2.2 test 1(symmetry QMs)

| subtest2.2(test1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][1] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][2] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][3] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][4] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][5] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [1][0] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][3] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][4] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][5] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [2][0] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][1] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][2] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][3] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][4] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][5] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [3][0] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [3][1] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Total Bit | 2026 | 1260 | 2576 | 2552 | 1428 | 2110 |
| MaxError | 19 | 14 | 66 | 37 | 62 | 103 |
| AvgError | 1.998031 | 3.453248 | 19.981545 | 4.866142 | 18.825787 | 33.987451 |

Table 2  Subtest2.2 test 1(asymmetry QMs)

| subtest2.2(test1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][1] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][2] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][3] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][4] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][5] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [1][0] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][3] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][4] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][5] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [2][0] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][1] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][2] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][3] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][4] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [2][5] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [3][0] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Matrix [3][1] Bit | 0 | 0 | 0 | 0 | 0 | 0 |
| Total Bit | 2400 | 1456 | 2860 | 3714 | 1684 | 2430 |
| MaxError | 10 | 11 | 86 | 129 | 55 | 134 |
| AvgError | 2.18873 | 2.782726 | 17.217766 | 15.425443 | 15.570866 | 28.31939 |

## Summary results

The average coding performance of the proposed coding tools is summarized in this part. The results are reported as w/o case4 (random matrices) in obtaining the average performance. The compression ratio is obtained by the ratio of anchor generated bits and the proposal generated bits.

### without case4(random matrices)

Table 3  Subtest2.2, symmetry QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 9595.05 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(symmetry QMs) |  |
|  | Total Bit | 1880.00 |
|  | MaxError | 52.80 |
|  | AvgError | 15.65 |
|  | compression ratio(anc/pro) | 4.86 |

Table 4  Subtest2.2, asymmetry QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 9536.21 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(asymmetry QMs) |  |
|  | Total Bit | 2166.00 |
|  | MaxError | 59.20 |
|  | AvgError | 13.22 |
|  | compression ratio(anc/pro) | 4.41 |
|  | compression ratio(anc/pro) | 5.07 |

Table 5  Subtest2.2, All QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 9339.40 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(for all QMs) |  |
|  | Total Bit | 2023.00 |
|  | MaxError | 56.00 |
|  | AvgError | 14.43 |
|  | compression ratio(anc/pro) | 4.64 |

### with case4(random matrices)

Table 6  Subtest2.2, symmetry QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 11908.10 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(symmetry QMs) |  |
|  | Total Bit | 1992.00 |
|  | MaxError | 50.17 |
|  | AvgError | 13.85 |
|  | compression ratio(anc/pro) | 5.08 |

Table 7  Subtest2.2, asymmetry QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 11963.76 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(asymmetry QMs) |  |
|  | Total Bit | 2424.00 |
|  | MaxError | 70.83 |
|  | AvgError | 13.58 |
|  | compression ratio(anc/pro) | 4.79 |

Table 8  Subtest2.2, All QMs

| (Subtest2.0, anchor) | Average(anchor) | Average(QMs) |
| --- | --- | --- |
|  | Total Bit | 11158.00 |
|  | MaxError | 0.00 |
|  | AvgError | 0.00 |
| subtest2.2(test1) | Average(for all QMs) |  |
|  | Total Bit | 2208.00 |
|  | MaxError | 60.50 |
|  | AvgError | 13.72 |
|  | compression ratio(anc/pro) | 4.93 |

More detailed coding results can be found in the Excel attachment.

# Conclusion

We have cross-checked the proposed tools, studied the algorithm and verified their coding simulation results. Without case4(random matrices), the proposed tools obtain 4.64x compression ratio against anchor with average reconstructed error 14.43; With case4(random matrices), the proposed tools obtain 4.93x compression ratio against anchor with average reconstructed error 13.72.

# References

[1] X. Zhang, S. Liu, S. Lei “Method and syntax for quantization matrices representation” in JCTVC-G152, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[2] K.Sato, “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-G1204, 7th Meeting: Geneva, CH, 21-30 November, 2011