| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-11 February, 2012 | Document: JCTVC-H308 |
| --- | --- |

| Title: | CE2: Subset 4.4: Test Results on Harmonization of JCTVC-G283, JCTVC-G442 and JCTVC-G980 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1,5Xin Zhao, 1Jicheng An, 1Xun Guo, 1Shawmin Lei North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2Liwei Guo, 2Xianglin Wang, 2Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA 3Akira Minezawa, 3Kazuo Sugimoto, 3Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN 4Atsuro Ichigaya, 4Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN 5Siwei Ma, 5Wen Gao School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, jijing.an, xun.guo, shawmin.lei}@mediatek.com {liweig, xianglin} @qualcomm.com {Sekiguchi.Shunichi@eb, Sugimoto.Kazuo@ak, Minezawa.Akira@ds}.MitsubishiElectric.co.jp ichigaya.a-go@nhk.or.jp {swma, wgao}@pku.edu.cn |
| Source: | 1MediaTek Inc., 2Qualcomm Inc., 3Mitsubishi Electric Corporation, 4NHK(Japan Broadcasting Corporation), 5Peking University |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of subtest 4.4 defined in CE2 document. Two tests, depending on whether 32x32 chroma transform is used, are conducted in this subtest. When 32x32 chroma transform is used, the proposed method reportedly achieves chroma BD-rate reductions of 2.4%, 1.7% and 1.2% for AI-HE, RA-HE and LD-HE, respectively, and chroma BD-rate reductions of 2.2%, 1.6% and 1.3% for AI-LC, RA-LC and LD-LC, respectively. When 32x32 chroma transform is not used, the proposed method reportedly achieves chroma BD-rate reductions of 2.4%, 1.6% and 0.9% for AI-HE, RA-HE and LD-HE, respectively, and chroma BD-rate reductions of 1.9%, 1.4% and 1.1% for AI-LC, RA-LC and LD-LC, respectively. It is also reported that the run time is almost the same with HM5.0.

# Introduction

In HEVC, RQT is applied to allow flexible transform unit (TU) sizes inside each coding unit (CU), and the optimal RQT depth is decided at encoder and signaled in the bitstream. In current HM5.0, for a chroma block, the transform depth of corresponding luma block is simply reused, i.e., luma and chroma components share the same transform depth. However, since chroma components generally have smoother textures than luma, it is not appropriate to always apply the same RQT depth of luma on chroma. To resolve this issue, Core Experiment 2 (CE2) [1] was established at Geneva meeting, and three proposals [2]-[4] were included in the description of CE2. In the subtest 4.4, a harmonized solution of the three proposals is defined, which contains two tests described in the following section.

# Proposed harmonization

In subtest 4.4, two user-defined parameters, max_transform_hierarchy_depth_inter_chroma and max_transform_hierarchy_depth_intra_chroma, are proposed, which define the maximum RQT depth for chroma in inter and intra coding, respectively. Since the maximum transform depth is no longer shared between luma and chroma, the original max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra in HM5.0 are also modified and used only for luma. As shown in Table 1, the proposed modifications in SPS are highlighted in red. In addition, chroma 32x32 transform is also introduced to enhance the coding performance.

Table 1 Proposed user-defined parameters in the SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter_luma | ue(v) |
| max_transform_hierarchy_depth_intra_luma | ue(v) |
| max_transform_hierarchy_depth_inter_chroma | ue(v) |
| max_transform_hierarchy_depth_intra_chroma | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| … | … |
| } |  |

For brevity, let us denote the prefix “max_transform_hierarchy_depth_” as “MaxDepth_” in the following texts. When the chroma RQT depth is smaller than luma, the RQT depth of the current chroma block, namely DepthC, is derived using the maximum RQT depth of chroma and RQT depth of current luma block, namely DepthL, as shown in Table 2.

Table 2 Derivation of chroma RQT depth

|  | DepthC |
| --- | --- |
| Inter | DepthC = min( DepthL, MaxDepth_inter_chroma ) |
| Intra | DepthC = min( DepthL, MaxDepth_intra_chroma ) |

# Simulations and results

The proposed method is implemented on top of HM 5.0. Simulations are performed under common test conditions defined in [5]. There are two test cases in subtest 4.4, depending on whether 32x32 chroma transform is used:

TEST 0: MaxDepth_intra_chroma =0, MaxDepth_inter_chroma =1, no 32x32 chroma transform;

TEST 1: MaxDepth_intra_chroma =0, MaxDepth_inter_chroma =1, with 32x32 chroma transform.

The BD-rate reductions for TEST 0 and TEST 1 are reported in Table 4 and 5, respectively. Both of the tests show good coding performance for chroma components. For TEST 0, the proposed method achieves 2.4%, 1.6% and 0.9% BD-rate reductions for AI-HE, RA-HE and LD-HE, respectively, and 1.9%, 1.4% and 1.1% BD-rate reductions for AI-LC, RA-LC and LD-LC, respectively. For TEST 1, the proposed method achieves 2.4%, 1.7% and 1.2% BD-rate reductions for AI-HE, RA-HE and LD-HE, respectively, and 2.2%, 1.6% and 1.3% BD-rate reductions for AI-LC, RA-LC and LD-LC, respectively.

Table 4 Experimental results of TEST 0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -3.0% | -3.9% | 0.0% | -1.4% | -2.3% |
| Class B | 0.0% | -3.0% | -3.2% | 0.0% | -2.1% | -2.7% |
| Class C | 0.0% | -1.3% | -1.4% | 0.0% | -1.1% | -1.4% |
| Class D | 0.0% | -0.7% | -0.7% | 0.0% | -0.7% | -0.7% |
| Class E | 0.1% | -5.0% | -4.3% | 0.0% | -4.1% | -3.5% |
| Class F | 0.1% | -1.7% | -1.6% | 0.0% | -0.8% | -0.8% |
| Overall | 0.0% | -2.4% | -2.5% | 0.0% | -1.8% | -2.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.2% | -3.2% | 0.0% | -1.4% | -2.5% |
| Class B | 0.0% | -2.5% | -2.6% | 0.0% | -2.0% | -2.5% |
| Class C | -0.1% | -0.9% | -1.0% | 0.0% | -0.8% | -1.0% |
| Class D | 0.0% | -0.7% | -0.3% | 0.0% | -0.2% | -0.9% |
| Class F | 0.0% | -1.2% | -1.2% | 0.0% | -0.7% | -0.7% |
| Overall | 0.0% | -1.6% | -1.6% | 0.0% | -1.1% | -1.7% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B | 0.0% | -1.4% | -1.3% | 0.0% | -1.2% | -0.9% |
| Class C | 0.0% | -0.4% | -0.3% | 0.0% | -0.3% | -0.5% |
| Class D | -0.1% | 0.1% | -1.0% | -0.1% | -0.2% | -0.6% |
| Class E | 0.1% | -1.7% | -1.4% | -0.1% | -3.3% | -2.9% |
| Class F | -0.1% | -1.2% | -1.2% | 0.0% | -1.7% | -1.2% |
| Overall | 0.0% | -0.8% | -1.0% | 0.0% | -1.1% | -1.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -3.6% | -2.9% |
| Class B | 0.0% | -2.6% | -2.8% |
| Overall | 0.0% | -3.0% | -2.8% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 5 Experimental results of TEST 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.4% | -3.1% | 0.0% | -1.5% | -2.4% |
| Class B | -0.1% | -3.0% | -3.1% | 0.0% | -2.5% | -3.2% |
| Class C | -0.1% | -1.3% | -1.2% | 0.0% | -1.2% | -1.4% |
| Class D | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.7% |
| Class E | 0.0% | -5.7% | -4.8% | 0.0% | -5.1% | -4.3% |
| Class F | 0.0% | -1.6% | -1.6% | 0.0% | -1.0% | -1.0% |
| Overall | -0.1% | -2.5% | -2.4% | 0.0% | -2.1% | -2.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -2.0% | -2.7% | 0.0% | -1.5% | -2.7% |
| Class B | -0.1% | -2.9% | -2.8% | 0.0% | -2.4% | -3.1% |
| Class C | 0.0% | -1.0% | -0.9% | 0.0% | -0.9% | -1.1% |
| Class D | 0.0% | -0.7% | -0.4% | 0.0% | -0.2% | -0.8% |
| Class F | -0.1% | -1.4% | -1.4% | 0.1% | -1.0% | -0.8% |
| Overall | -0.1% | -1.7% | -1.7% | 0.0% | -1.3% | -1.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B | 0.0% | -1.4% | -1.5% | 0.0% | -1.5% | -1.5% |
| Class C | 0.0% | -0.3% | -0.5% | 0.0% | -0.2% | -0.6% |
| Class D | -0.1% | -0.2% | -0.9% | -0.1% | -0.1% | -0.5% |
| Class E | -0.1% | -3.2% | -2.4% | 0.0% | -3.7% | -4.2% |
| Class F | -0.1% | -1.2% | -1.4% | 0.1% | -1.5% | -1.6% |
| Overall | 0.0% | -1.2% | -1.2% | 0.0% | -1.2% | -1.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -6.4% | -5.7% |
| Class B | 0.0% | -3.0% | -2.9% |
| Overall | 0.0% | -4.5% | -4.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

In this contribution, maximum chroma RQT depth is proposed to be added in SPS to allow separate transform depth for luma and chroma. Two test cases defined in CE2 subtest 4.4 are tested. Experimental results show that the proposed method can achieve around 1%~5% BD-rate reduction for chroma components.

# References

[1] K. Sugimoto, X. Guo and X. Wang, “Description of Core Experiment 2 (CE2): Chroma RQT depth”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1202r2, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[2] X. Zhao, X. Guo, X. Li, S. Lei, Siwei Ma, Wen Gao, “Residue Quad Tree Depth for Chroma in Intra coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G283, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] A. Minezawa, K. Sugimoto, Shun-ichi Sekiguchi, “Improvement to chroma TU specification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G442, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[4] L. Guo, X. Wang, M. Karczewicz, “Evaluation of Limiting Chroma Transform Depth in RQT on HM4.0”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G980, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[5] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).