#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter_luma | ue(v) |
| max_transform_hierarchy_depth_intra_luma | ue(v) |
| max_transform_hierarchy_depth_inter_chroma | ue(v) |
| max_transform_hierarchy_depth_intra_chroma | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| … | … |

#### 7.4.2.1 Sequence parameter set RBSP semantics

log2_min_pcm_coding_block_size_minus3 + 3 specifies the minimum size of I_PCM coding blocks.

The variable Log2MinIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3. The variable Log2MinIPCMCUSize should be equal or less than Log2MaxCUSize.

max_transform_hierarchy_depth_intra_luma specifies the maximum hierarchy depth for luma transform blocks of coding blocks coded in intra prediction mode.

max_transform_hierarchy_depth_inter_luma specifies the maximum hierarchy depth for luma transform units of coding units coded in inter prediction mode.

max_transform_hierarchy_depth_intra_chroma specifies the maximum hierarchy depth for chroma transform blocks of coding blocks coded in intra prediction mode.

max_transform_hierarchy_depth_inter_chroma specifies the maximum hierarchy depth for chroma transform units of coding units coded in inter prediction mode.

chroma_pred_from_luma_enabled_flag equal to 1 specifies the intra chroma prediction process using the reconstructed luma block is applied according to the intra chroma prediction mode.

loop_filter_across_slice_flag equal to 1 specifies the in-loop filtering operations are performed across slice boundary; otherwise, the in-loop operations are slice-independent and not across slice boundary. The in-loop filtering operations include deblocking filter, sample adaptive offset, and adaptive loop filter.

### 7.3.8	Transform tree syntax

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize ) { if( PredMode != MODE_INTRA && trafoDepth≤getTrafoDepthC(trafoDepth+ split_transform_flag[ x0 ][ y0 ][ trafoDepth ]) ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| … | … |
| } |  |

### 7.3.9	Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx, bChroma ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| … | … |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx, bChroma ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx, bChroma ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx, bChroma ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx, bChroma ) |  |
| } else { |  |
| { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else |  |
| scanIdx = 3 |  |
| if( (!split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && cIdx==0) ¦¦ |  |
| (trafoDepth≤getTrafoDepthC(trafoDepth+ split_transform_flag[ x0 ][ y0 ][ trafoDepth ]) cIdx!=0)){ |  |
| residual_coding( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## 6.7	Derivation process for chroma transform depth

Input to this process is the luma transform depth trafoDepthL.

Output of this process is the luma transform depth trafoDepthC.

if (PredMode  ==  MODE_INTRA){

uiMaxTrModeC = max_transform_hierarchy_depth_intra_chroma-1

trafoDepthC = PartMode == SIZE_2Nx2N ? uiMaxTrModeC : 1

}

else{

trafoDepthC = max_transform_hierarchy_depth_inter_chroma-1

}

trafoDepthC = min(trafoDepthL, trafoDepthC);

### 8.3.3	Decoding process for intra blocks

……

Output of this process is:

–	a modified reconstructed picture before deblocking filtering.

Depending split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] && cIdx ==0 is true or trafoDepth == getTrafoDepthC(trafoDepth+ split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && cIdx !=0 is true , the following ordered steps apply:

……

#### 8.4.3.2 Decoding process for chroma residual blocks

……

The variable splitChromaFlag is derived as follows:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than Log2MinTrafoSize + 1, splitChromaFlag is set equal to 1.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to Log2MinTrafoSize + 1), splitChromaFlag is set equal to 0.

–	getTrafoDepthC(trafoDepth+ split_transform_flag[ x0 ][ y0 ][ trafoDepth ]

Depending splitChromaFlag, the following applies: