| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0311 |
| --- | --- |

| Title: | CE10 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-H0310 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qian Huang, Jicheng An, Xun Guo, and Shawmin Lei No. 2, Kexueyuan South Rd., Raycom Infotech Park, Beijing, China 100190 | Email: | {qian.huang, jicheng.an, xun.guo, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of chroma deblocking filter in CE10 Subtest 1. In specific, chroma deblocking filter with dependent filtering decision on luma and 8x8 filtering unit is tested and proposed. It is reported that an average BD-Rate reduction of 1.1% can be achieved for chroma. For low delay cases, an average BD-Rate reduction of 0.1% can also be achieved for luma. The run time is reported to be similar to HM5.0. It is also reported that the subjective quality is almost the same as that of HM5.0.

# Introduction

In HM5.0 deblocking filter, filtering decision is only applied on luma boundaries. In order to further improve both objective performance and subjective quality, we have proposed a chroma filtering decision method in our previous proposals JCTVC-F118 [1], then a simplified chroma filtering decision method in JCTVC-F120 [2]. This contribution is based on JCTVC-F120. However, in order to reduce complexity, 8x8 filtering unit is used in this contribution instead of previous 4x4 unit.

The rest of this contribution is organized as follows. Section 2 describes the proposed algorithm. Section 3 shows and analyzes the experimental results. And section 4 draws our conclusion.

# Algorithm Description

The proposed chroma filter actually is a simplification of that in JCTVC-G286. The major difference is that chroma boundary would re-use the filtering decision of corresponding luma boundary. Figure 1 gives the flowchart of the proposed method. Beta_Luma and Beta_Chroma are the threshold values derived from the beta table for luma and chroma, respectively.

For each 8x8 luma boundary, the luma pixel differences d1 and d2 of its two sub-boundaries are computed as follows:

d1 = |p20 - 2*p10 + p00| + |q20 - 2*q10 + q00| + |p23 - 2*p13 + p03| + |q23 - 2*q13 + q03|,         (1)

d2 = |p24 - 2*p14 + p04| + |q24 - 2*q14 + q04| + |p27 - 2*p17 + p07| + |q27 - 2*q17 + q07|,         (2)

where the luma samples are depicted in Figure 2.

The corresponding chroma samples will be filtered if the below expression is true:

(d < Beta_Chroma) && (On the 8x8 chroma grid),                  (3)

where d is either d1 or d2, depending on the chroma position. For the 4:2:0 format, d1 is used for the first two chroma lines whereas d2 is used for the last two chroma lines.

Note that Beta_Luma is exactly the same as in HM 5.0, and Beta_Chroma is defined according to the slice type:

- For I slices, Beta_Chroma is defined as a fixed value 128.

- For B/P slices, Beta_Chroma is the same as Beta_Luma.

Figure 1. Flowchart of the proposed algorithm

|  | p30 | p20 | p10 | p00 | q00 | q10 | q20 | q30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p31 | p21 | p11 | p01 | q01 | q11 | q21 | q31 |  |
|  | p32 | p22 | p12 | p02 | q02 | q12 | q22 | q32 |  |
|  | p33 | p23 | p13 | p03 | q03 | q13 | q23 | q33 |  |
|  | p34 | p24 | p14 | p04 | q04 | q14 | q24 | q34 |  |
|  | p35 | p25 | p15 | p05 | q05 | q15 | q25 | q35 |  |
|  | p36 | p26 | p16 | p06 | q06 | q16 | q26 | q36 |  |
|  | p37 | p27 | p17 | p07 | q07 | q17 | q27 | q37 |  |

Figure 2. An example of 8x8 luma boundary

# Experimental Results

## Objective Performance

Our simulations are based on HM5.0 software and the common test conditions in G1200 [3]. An average BD-Rate reduction of 1.1% is observed for chroma, and an average BD-Rate reduction of 0.1% is observed for luma in low delays cases. Moreover, the run time of the proposed method is almost the same to that of HM5.0.

Table 1. Performance compared to HM 5.0

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | -0.4% | -0.3% | 0.0% | -0.4% | -0.3% |  |  |  |
| Class C | 0.0% | -0.4% | -0.4% | 0.0% | -0.2% | -0.1% |  |  |  |
| Class D | 0.0% | -0.5% | -0.4% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |  |  |  |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |  |  |  |
| Class F | 0.0% | -0.8% | -0.8% | 0.0% | -0.6% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99.43% |  |  | 99.45% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -1.0% | -0.9% | 0.0% | -1.3% | -1.1% | 0.0% | -1.6% | -1.4% |
| Class B | 0.0% | -1.2% | -1.2% | 0.0% | -1.7% | -1.5% | 0.0% | -1.4% | -1.2% |
| Class C | 0.0% | -1.2% | -1.2% | 0.0% | -1.6% | -1.4% |  |  |  |
| Class D | 0.0% | -1.3% | -1.2% | 0.0% | -1.6% | -1.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -1.2% | -1.2% | 0.0% | -1.6% | -1.4% | 0.0% | -1.5% | -1.3% |
|  | 0.0% | -1.2% | -1.1% | 0.0% | -1.6% | -1.4% | 0.0% | -1.5% | -1.3% |
| Class F | -0.1% | -1.6% | -1.5% | 0.0% | -1.6% | -1.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98.98% |  |  | 99.28% |  |  | 100.73% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.9% | -0.6% | 0.0% | -2.3% | -1.7% |  |  |  |
| Class C | -0.1% | -1.3% | -1.2% | 0.0% | -1.2% | -1.0% |  |  |  |
| Class D | -0.1% | -1.1% | -0.9% | -0.1% | -1.0% | -1.0% |  |  |  |
| Class E | -0.1% | -1.1% | -2.0% | -0.1% | -4.6% | -3.1% |  |  |  |
| Overall | -0.1% | -1.1% | -1.1% | -0.1% | -2.2% | -1.6% |  |  |  |
|  | -0.1% | -1.1% | -1.1% | -0.1% | -2.1% | -1.6% |  |  |  |
| Class F | -0.1% | -1.5% | -1.0% | -0.1% | -1.8% | -1.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99.22% |  |  | 99.89% |  |  |  |  |  |

## Subjective Performance

An informal subjective evaluation shows that the proposed technique has similar subjective quality to HM5.0 in general. However due to the proposed filtering decision and the filtering of inter chroma blocks, we has less chroma artifacts than HM5.0. Here are some examples:

In Figure 3, (a), (b) and (c) are the original picture, HM 5.0 reconstructed picture and G287 reconstructed picture for BasketballDrillText_832x480_LDHE_QP37_#81. It can be seen that G287 has less color artifacts around the basketball.

(a) Original                   (b) HM 5.0                   (c) Proposed

Figure 3 Subjective results on BasketballDrillText_832x480_LDHE_QP37_#81

In Figure 4, (a), (b) and (c) are the original picture, HM 5.0 reconstructed picture and G287 reconstructed picture for RaceHorses_832x480_RAHE_QP37_#31. The color of the horse diffuses in HM 5.0, which does not exist in G287.

(a) Original                   (b) HM 5.0                   (c) Proposed

Figure 4 Subjective results on RaceHorses_832x480_RAHE_QP37_#31

In Figure 5, (a), (b) and (c) are the original picture, HM 5.0 reconstructed picture and G287 reconstructed picture for RaceHorses_832x480_RAHE_QP37_#34. The proposed method has better grass details than HM 5.0.

(a) Original                   (b) HM 5.0                   (c) Proposed

Figure 5 Subjective results on RaceHorses_832x480_RAHE_QP37_#34

# Conclusion

The contribution proposes a simplified solution based on an earlier contribution [2]. Specifically, the filtering will be performed only on 8x8 chroma grid. For common test conditions, the average BD-Rate reduction is 1.1% for chroma and the average decoder running time is slightly reduced compared with that of HM 5.0. In addition, the subjective quality is almost the same as HM5.0.

# References

- A. Norkin, K. Andersson, R. Sjöberg, Q. Huang, J. An, X. Guo and S. Lei, “CE12: Ericsson's and MediaTek's deblocking filter,” JCTVC-F118, JCT-VC meeting, Torino, IT, Jul. 2011.

- Q. Huang, J. An, X. Guo and S. Lei, “Simplification of chroma deblocking filter in JCTVC-F118,” JCTVC-F120, JCT-VC meeting, Torino, IT, Jul. 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, JCT-VC meeting, Geneva, CH, Nov. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).