| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0312 |
| --- | --- |

| Title: | Non-CE4: Clipping of Transformed Coefficients before De-quantization |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Xiang Li *Xun Guo #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | x.li@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

In current HEVC WD5 (JCTVC-G1103), there is potential overflow during de-quantization process when quantization matrices are enabled. To avoid such overflow, an adaptive clipping of transformed coefficients before de-quantization is proposed in this contribution. When 8-bit video input is further assumed, a very simple clipping method is suggested.

# Introduction

In current HEVC (working draft 5 and test model 5), de-quantization process is formulated as (1), namely

coeffQ = ((level*IQ[QP%6] << (QP/6)) + offset)>>(M-1+DB),    offset = 1<<(M-2+DB),	(1)

where coeffQ and level denote de-quantized and quantized transformed coefficients, respectively; IQ(x)=g(x)={40,45,51,57,64,72}, x=0,…,5 indicates de-quantization step; QP represents quantization parameter, M=log2(N); N stands for transform size; DB=B-8, B is the bit-depth of input video. Note that QP/6 indicates the integer part of QP/6 hereafter.

It is assumed that the dynamic range of level is 16 bits (including 1 bit for sign). Since IQ(x)<=72, QP<=51, the dynamic range of IQ(x) is 7 bits and the operation “<<(QP/6)” needs 8 bits at most. Consequently, the dynamic range of “(level*IQ[QP%6]) << (QP/6)” is 16+7+8=31 bits. Therefore in 32-bit de-quantization, (1) will never cause overflow.

However, when quantization matrix is introduced, the de-quantization is modified from (1) to (2)-(4), i.e.,

iShift = M-1+DB+4;									(2)

if (iShift > QP/6)

coeffQ[i][j] = (level[i][j]*W[i][j]*IQ[QP%6] + offset)>>(iShift-QP/6),  offset  = 1<<(iShift-QP/6-1),												(3)

else

coeffQ[i][j] = (level[i][j]*W[i][j]*IQ[QP%6]) << (QP/6-iShift), 				(4)

where W denotes quantization matrix; [i][j] indicates the position of transformed coefficient in a transform unit (i=0…nW-1, j = 0..nH-1, nW and nH are the width and height of the transform).

Considering that W is an 8-bit array, the dynamic range of (4) is 16+8+7+3=34 bits when QP=51, M=2 and DB=0. In 32-bit de-quantization, (4) may cause overflow, e.g. transmission error of level values, which is not desired in practice since it may cause coding failure especially at decoder.

# Proposed Method

Let x be the dynamic range of transformed coefficient “level”. To avoid overflow in (4), x has to meet the following condition:

x+8+7+(QP/6-(M-1+DB+4)) <=32,								(5)

which leads to

x<=20+M+DB-QP/6									(6)

Consequently, level shall be clipped as in (7):

level = max( -2x-1, min(2x-1-1, level))								(7)

To avoid the overflow generally, level is clipped as follows according to (6):

if (20+M+DB-QP/6>=16)

level = max( -215, min(215-1, level))

else

level = max( -2x-1, min(2x-1-1, level))

When the bit-depth of source video is assumed as 8 bits (DB=0), (6) is simplified to

x<=22-QP/6.

Consequently, the proposed clipping is only dependent on QP, i.e.,

if (QP<=42)

level = max( -215, min(215-1, level));

else

level = max( -221-QP/6, min(221-QP/6-1, level)).

When the bit-depth of source video is 10 bits or higher, namely DB>=2, the condition in (6) is always met. In this case, 16-bit clipping, namely level=max( -215, min(215-1, level)), is always used.

# Simulation results

The proposed method was implemented on top of HM-5.0 branch of quantization matrix (SVN revision number 1722). The simulation environments were set according to JCTVC-G1200 except that default quantization matrices were enabled (--ScalingList=1).

Table 1 summarizes the simulation results. The anchor is HM-5.0 with default quantization matrices (--ScalingList=1). Because there is no dynamic range overflow under common test conditions, identical coding performance to the anchor was obtained.

Table 1 Simulation results of proposed method

# Conclusions

In this contribution, an adaptive clipping method in de-quantization process of video and image compression is proposed. Based on transform size, source bit-depth and quantization parameter, this method clips transformed coefficients before de-quantization. The proposed method has no impact on coding performance under common test conditions because there is no overflow. However, the potential overflow in 32-bit de-quantization, which may be caused by transmission error of level values, can be avoided by the proposed method.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).