| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0314 WG11 Number: m23189 |
| --- | --- |

| Title: | CE4: Subtest 2.4 Layered quantization matrices representation and compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yunfei Wang(Tsinghua) Jianhua Zheng(HiSilicon) Yun He(Tsinghua) Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

A layered quantization matrices representation and compression approach is proposed to improve the quantization matrices compression ratio. It can be configured as lossless coding or lossy coding with a large range of compression ratio.

Without case4 (random matrices), the proposed tools obtain 1.87 x compression ratios against the HM5-QM anchor, 4.5x compression ratios with average error 1.74, 10.87x compression ratios with average error 2.87, and 20.26x compression ratios with average error 4.23, respectively.

With case4(random matrices), the proposed tools obtain 1.96 x compression ratios against the HM5-QM anchor, 4.47x compression ratios with average error 0.54, 9.94x compression ratios with average error 1.38, and 17.59x compression ratios with average error 2.83, respectively.

The main features of this approach are,

- only one pass processing need to compress the quantization matrices, so the encoding time and design complexity to compress quantization matrix are very low.

- controlling the number of coded layers, it is easily shift from matrix lossless coding to matrix lossy coding with high compression ratio.

- If a small error affordable, the lossy coding approach will reach high compression ratio.

-The proposed approach is comparable with both lossless QM coding and lossy QM coding

# Introduction

## Layered Coding structure

In this layered coding structure, different elements in quantization matrix are coded in different matrix layers. A sample layered 8x8 matrix is shown in Fig.1. The numbers in the matrix in Fig.1 stand for the layer numbers. As in Fig.1, five layers are predefined and the layer number starts from 0 to 4. And the maximum layer numbers are predefined for different size of matrix. In this proposal, the maximum layer number is predefined as 3 for 4x4 matrix, 4 for 8x8 matrix, 5 for 16x16 matrix and 6 for 32x32 matrix, respectively. Hence the maximum layer number of NxN is determined by log2N+1.

Fig. 1 Example of matrix layers for 8x8 matrix

In the layered coding structure, lower layered elements are coded before higher layered elements. The current element is predicted from coded elements in the previous and current layer and the residue of the predicted value and the original element is coded. Since the residue is coded in matrix bitstream, the matrix element can be reconstructed using the predicted value and residue in lossless way. If compression ratio is required to increase, the residue in higher layer is not coded to reduce the coded bits. Hence the elements in higher layers are reconstructed in a lossy way using the predicted value instead of coding the residues in bitstream.

The largest number of layer to be coded is determined by QMSizeNCodeLevels and configured in encoder. QMSizeNCodeLevels need to be notified to the decoder and coded in bitstream. For an element (i, j) belongs to Layer k,

if k<=QMSizeNCodeLevels, the residue value is coded, so the reconstructed value is obtained by the predicted value and the residue value, and the original element can be reconstructed with lossless;

if k>QMSizeNCodeLevels, residues are not coded, so the reconstructed value of element equals to the prediction value of the element, which is predicted from coded elements in the previous and current layer.

## Prediction

For an element (i, j) belongs to Layer k (k>0), the prediction value of current element is interpolated by the reconstruction value of two reference elements R1 and R2. If this element is vertical interpolated, R1 is the nearest coded element which is above it in the same column, and R2 is the nearest coded element which is below it in the same column; if this element is vertical interpolated, R1 is the nearest coded element which is left to it in the same row, and R2 is the nearest coded element which is right to it in the same row.

### Coding Order within a layer

For Layer k (k>1), the coding can be classified into 2 stages:

Stage 1: vertical stage. For each element in layer k, if it can be vertical interpolated, then element is vertical interpolated by reference elements R1 and R2, (R1 and R2 are the nearest coded elements). The vertical interpolation within this stage can be paralleled.

Stage 2: horizontal stage. The rest elements are predicted by horizontal interpolation. The horizontal interpolation within this stage can be paralleled.

Fig. 2 Illustration of vertical stage and horizontal stage.

### Interpolation

If the distance of the current element to R1 equals to the distance of the current element to R2, the prediction value of the current element is conducted as:

( R1 + R2 +1 ) >>1

Otherwise, the interpolation is weighted by the distance.

## Copy mode

There are totally 24 matrices in the quantization matrices set for one frame. In case the user does not design all the 24 matrices, or the matrices are too much alike, the copy mode can be used. If a matrix is determined as using this mode, only one bit is signaled for the matrix, and the matrix is derived directly from its reference matrix.

Down sampling may be required. In this case, the elements with even indexes are extracted directly from the reference matrix.

## Symmetry mode

One bit of flag is signaled to determine if the matrix is symmetric. In this mode, half of the elements, the up-right triangle, are coded, as same as in [3].

## Residue Coding

For residue from Layer k, if k<=4, the residue is code by signed exp-golomb code; otherwise, the residue is expected to be small, so it is coded by a run-length code.

## Differential coding

Because the difference between two adjacent matrix sets in temporal axis is normally small, the differential coding is employed. If there are coded quantization matrix sets, use the nearest matrix to be the reference matrix. The differential matrix is obtained via reference matrix, and the values in the differential matrix are coded. The coding method is the same as above. The reconstructed differential matrices are then added to the reference matrices to get the reconstructed matrices. If it is lossy coding, the lossy elements should be interpolated after the previous process is done.

# Simulation Results

The implementation is on HM 5.0-qmatrix branch software. Tests were conducted on CE4 subtest2 test1 condition[2]. Only first frame is loaded with one set of matrices for each runs. In simulation, totally 6 set of symmetric matrices and 6 set of asymmetric matrices are used, which are provided by Sony and Qualcomm. In those sets of matrices, case4 are random matrices, which included one set of random symmetric matrices and one set of asymmetric matrices. Whether or not to put case4 (random matrices) in tool evaluation was deeply discussed in CE4 reflector, three voices to remove the case4, and one voice to keep it in QMs test set. Since it is not got unified agreement, in the following CE simulation, case4 ( random matrices)is kept in test set, which is for the purpose of test the robustness of the methods.

The simulation will include the summary results with/without case4 random matrices respectively.

## TEST1 results

Table 1  Subtest2.4, lossless(symmetry)

| (subtest2.4),(lossless) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 57 | 47 | 69 | 57 | 57 | 69 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 65 | 37 | 65 | 65 | 65 | 65 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 185 | 151 | 247 | 233 | 151 | 185 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 141 | 87 | 187 | 221 | 87 | 141 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 501 | 308 | 575 | 854 | 575 | 308 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 318 | 212 | 342 | 815 | 342 | 212 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 1173 | 884 | 1942 | 3119 | 1173 | 1942 |
| Matrix [3][1] Bit | 902 | 433 | 1757 | 3289 | 902 | 1757 |
| Total Bit | 3367 | 2184 | 5209 | 8678 | 3377 | 4704 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |

Table 2  Subtest2.4, lossy opt1(symmetry)

| (subtest2.4),(lossy opt1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 37 | 33 | 47 | 35 | 37 | 47 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 45 | 31 | 47 | 33 | 45 | 47 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 92 | 78 | 126 | 106 | 78 | 92 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 74 | 44 | 88 | 90 | 44 | 74 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 224 | 138 | 252 | 284 | 252 | 138 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 148 | 90 | 158 | 254 | 158 | 90 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 538 | 382 | 801 | 962 | 538 | 801 |
| Matrix [3][1] Bit | 352 | 180 | 654 | 962 | 352 | 654 |
| Total Bit | 1535 | 1001 | 2198 | 2751 | 1529 | 1968 |
| MaxError | 23 | 11 | 46 | 27 | 15 | 23 |
| AvgError | 0.55315 | 0.255659 | 1.026821 | 3.752461 | 0.55315 | 0.687992 |

Table 3  Subtest2.4, lossy opt2(symmetry)

| (subtest2.4),(lossy opt2) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 37 | 33 | 47 | 35 | 37 | 47 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 45 | 31 | 47 | 33 | 45 | 47 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 1 | 39 | 57 | 1 | 39 | 41 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 39 | 29 | 47 | 41 | 29 | 39 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 1 | 58 | 96 | 1 | 96 | 58 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 72 | 44 | 76 | 100 | 76 | 44 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 228 | 168 | 330 | 304 | 228 | 330 |
| Matrix [3][1] Bit | 158 | 78 | 272 | 286 | 158 | 272 |
| Total Bit | 606 | 505 | 997 | 826 | 733 | 903 |
| MaxError | 21 | 14 | 58 | 30 | 24 | 29 |
| AvgError | 1.102116 | 0.617618 | 2.716781 | 5.234006 | 1.568652 | 1.675197 |

Table 4  Subtest2. 4, lossy opt3(symmetry)

| (subtest2.4),(lossy opt3) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 37 | 33 | 47 | 35 | 37 | 47 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 45 | 31 | 47 | 33 | 45 | 47 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 1 | 39 | 57 | 1 | 39 | 41 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 39 | 29 | 47 | 41 | 29 | 39 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 1 | 31 | 45 | 1 | 45 | 31 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 41 | 31 | 43 | 43 | 43 | 31 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 88 | 68 | 128 | 112 | 88 | 128 |
| Matrix [3][1] Bit | 68 | 42 | 104 | 100 | 68 | 104 |
| Total Bit | 345 | 329 | 543 | 391 | 419 | 493 |
| MaxError | 20 | 14 | 58 | 32 | 29 | 47 |
| AvgError | 2.375984 | 0.964321 | 5.092766 | 5.805856 | 2.820374 | 3.589075 |

Table 5  Subtest2.4, lossless(asymmetry)

| (subtest2.4),(lossless) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 81 | 69 | 95 | 111 | 81 | 95 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 97 | 51 | 101 | 151 | 97 | 101 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 313 | 229 | 427 | 577 | 229 | 313 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 293 | 133 | 375 | 555 | 133 | 293 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 668 | 392 | 882 | 2239 | 882 | 392 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 697 | 392 | 770 | 2265 | 770 | 392 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 1762 | 1426 | 2866 | 8947 | 1762 | 2866 |
| Matrix [3][1] Bit | 1673 | 1027 | 3535 | 9072 | 1673 | 3535 |
| Total Bit | 5609 | 3744 | 9076 | 23942 | 5652 | 8012 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |

Table 6  Subtest2.4, lossy opt1(asymmetry)

| (subtest2.4),(lossy opt1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 48 | 44 | 56 | 72 | 48 | 56 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 60 | 36 | 62 | 76 | 60 | 62 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 156 | 124 | 208 | 210 | 124 | 156 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 156 | 72 | 186 | 204 | 72 | 156 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 342 | 212 | 400 | 696 | 400 | 212 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 346 | 158 | 360 | 728 | 360 | 158 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 738 | 484 | 1167 | 2460 | 738 | 1167 |
| Matrix [3][1] Bit | 744 | 395 | 1363 | 2545 | 744 | 1363 |
| Total Bit | 2615 | 1550 | 3827 | 7016 | 2571 | 3355 |
| MaxError | 8 | 4 | 16 | 105 | 7 | 8 |
| AvgError | 0.396161 | 0.193652 | 0.794045 | 11.776329 | 0.393947 | 0.53937 |

Table 7  Subtest2.4, lossy opt2(asymmetry)

| (subtest2.4),(lossy opt2) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 48 | 44 | 56 | 72 | 48 | 56 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 60 | 36 | 62 | 76 | 60 | 62 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 1 | 56 | 86 | 1 | 56 | 64 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 62 | 40 | 68 | 82 | 40 | 62 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 1 | 106 | 174 | 208 | 174 | 106 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 160 | 78 | 164 | 218 | 164 | 78 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 330 | 240 | 474 | 658 | 330 | 474 |
| Matrix [3][1] Bit | 338 | 136 | 542 | 726 | 338 | 542 |
| Total Bit | 1025 | 761 | 1651 | 2066 | 1235 | 1469 |
| MaxError | 10 | 7 | 32 | 112 | 10 | 16 |
| AvgError | 0.976378 | 0.477854 | 2.220226 | 15.337598 | 1.137303 | 1.323327 |

Table 8  Subtest2. 4, lossy opt3(asymmetry)

| (subtest2.4),(lossy opt3) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 13 | 13 | 13 | 13 | 13 | 13 |
| Matrix [0][0] Bit | 48 | 44 | 56 | 72 | 48 | 56 |
| Matrix [0][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][3] Bit | 60 | 36 | 62 | 76 | 60 | 62 |
| Matrix [0][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][0] Bit | 1 | 56 | 86 | 84 | 56 | 64 |
| Matrix [1][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][3] Bit | 62 | 40 | 68 | 82 | 40 | 62 |
| Matrix [1][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [1][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][0] Bit | 1 | 48 | 68 | 76 | 68 | 48 |
| Matrix [2][1] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][2] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][3] Bit | 66 | 44 | 66 | 78 | 66 | 44 |
| Matrix [2][4] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [2][5] Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [3][0] Bit | 158 | 118 | 208 | 210 | 158 | 208 |
| Matrix [3][1] Bit | 152 | 60 | 230 | 210 | 152 | 230 |
| Total Bit | 573 | 471 | 869 | 913 | 673 | 799 |
| MaxError | 11 | 7 | 32 | 113 | 20 | 19 |
| AvgError | 1.973917 | 0.883366 | 4.799705 | 16.664124 | 2.825049 | 2.935778 |

## Summary results

The average coding performance of the proposed coding tools is summarized in this part. The results are reported as w/o case4 (random matrices) in obtaining the average performance. The compression ratio is obtained by the ratio of anchor generated bits and the proposal generated bits.

### without case4(random matrices)

Table 9  Subtest2.4, symmetry QMs

| Average(symmetry QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 9595.05 | 3768.20 | 1646.20 | 748.80 | 425.80 |
| MaxError | 0.00 | 0.00 | 23.60 | 29.20 | 33.60 |
| AvgError | 0.00 | 0.00 | 0.62 | 1.54 | 2.97 |
| compression ratio(anc/pro) | 1.00 | 2.43 | 5.50 | 12.12 | 21.21 |

Table 10  Subtest2.4, asymmetry QMs

| Average(asymmetry QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 9536.21 | 6418.60 | 2783.60 | 1228.20 | 677.00 |
| MaxError | 0.00 | 0.00 | 8.60 | 15.00 | 17.80 |
| AvgError | 0.00 | 0.00 | 0.46 | 1.23 | 2.68 |
| compression ratio(anc/pro) | 1.00 | 1.50 | 3.44 | 7.76 | 13.97 |

Table 10  Subtest2.4, All QMs

| Average(for all QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 9339.40 | 5093.40 | 2214.90 | 988.50 | 551.40 |
| MaxError | 0.00 | 0.00 | 16.10 | 22.10 | 25.70 |
| AvgError | 0.00 | 0.00 | 0.54 | 1.38 | 2.83 |
| compression ratio(anc/pro) | 1.00 | 1.96 | 4.47 | 9.94 | 17.59 |

### with case4(random matrices)

Table 11  Subtest2.4, symmetry QMs

| Average(symmetry QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 11908.10 | 4586.50 | 1830.33 | 761.67 | 420.00 |
| MaxError | 0.00 | 0.00 | 24.17 | 29.33 | 33.33 |
| AvgError | 0.00 | 0.00 | 1.14 | 2.15 | 3.44 |
| compression ratio(anc/pro) | 1.00 | 2.33 | 5.54 | 13.26 | 24.36 |

Table 12  Subtest2.4, asymmetry QMs

| Average(asymmetry QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 11963.76 | 9339.17 | 3489.00 | 1367.83 | 716.33 |
| MaxError | 0.00 | 0.00 | 24.67 | 31.17 | 33.67 |
| AvgError | 0.00 | 0.00 | 2.35 | 3.58 | 5.01 |
| compression ratio(anc/pro) | 1.00 | 1.42 | 3.45 | 8.47 | 16.17 |

Table 13  Subtest2.4, All QMs

| Average(for all QMs) | Average(anchor) | lossless | lossy opt1 | lossy opt2 | lossy opt3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit | 11158.00 | 6962.83 | 2659.67 | 1064.75 | 568.17 |
| MaxError | 0.00 | 0.00 | 24.42 | 30.25 | 33.50 |
| AvgError | 0.00 | 0.00 | 1.74 | 2.87 | 4.23 |
| compression ratio(anc/pro) | 1.00 | 1.87 | 4.50 | 10.87 | 20.26 |

More detailed coding results can be found in the Excel attachment.

Without case4 (random matrices), the proposed tools obtain 1.87 x compression ratios against the HM5-QM anchor, 4.5x compression ratios with average error 1.74, 10.87x compression ratios with average error 2.87, and 20.26x compression ratios with average error 4.23, respectively.

With case4(random matrices), the proposed tools obtain 1.96 x compression ratios against the HM5-QM anchor, 4.47x compression ratios with average error 0.54, 9.94x compression ratios with average error 1.38, and 17.59x compression ratios with average error 2.83, respectively.

The main feature of this approach are,

- only one pass processing need to compress the quantization matrices, so the encoding time and design complexity to compress quantization matrix are very low.

- controlling the number of coded layers, it is easily shift from matrix lossless coding to matrix lossy coding with high compression ratio.

- If a small error affordable, the lossy coding approach will reach high compression ratio.

-The proposed approach is comparable with both lossless QM coding and lossy QM coding

# Conclusion

A layered quantization matrices representation and compression approach is proposed to improve the quantization matrices compression ratio. It can be configured as lossless coding or lossy coding with a large range of compression ratio. It is suggested to enable lossy QM coding and have layer coding structure in HM design.

# References

[1] Y. Wang, J. Zheng, Y. He, “Layered quantization matrices compression” in JCTVC-G530, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[2] K.Sato, “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-G1204, 7th Meeting: Geneva, CH, 21-30 November, 2011

[3] J.Tanaka, Y.Morigami,T.Suzuki, “Enhancement of quantization matrix coding for HEVC” in JCTVC-F475, Torino, IT, July. 2011.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, Tsinghua University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).