| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0316 |
| --- | --- |

| Title: | Non-CE9: Simplification of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *#Liang Zhao, #Xun Guo, #Shawmin Lei, £Siwei Ma, *Debin Zhao, £Wen Gao #North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 * Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 £School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {liang.zhao, xun.guo, shawmin.lei}@mediatek.com {swma, dbzhao, wgao}@jdl.ac.cn |
| Source: | # MediaTek, *Harbin Institute of Technology, £Peking University |  |  |

_____________________________

# Abstract

In HM5.0, two redundancy checking processes are performed when the AMVP candidate list is constructed. In the first one, comparisons are made among spatial and temporal candidates to remove duplicate motion vectors. In the second one, if there is only one candidate available, zero checking is used to identify whether the candidate is zero or not. If not, zero motion vector is added in the candidate list. This contribution simplifies the AMVP candidate construction by exempting temporal candidate from comparison in the first one, and removing the zero checking in the second one. It is reported that there is almost no BD-rate change on average compared with HM5.0. Class F reportedly shows 0.1% BD-rate reduction in low delay B configurations. It is also reported that the run time is almost the same as that of HM5.0.

# Introduction

In HM5.0, five spatial candidates of AMVP are classified into 2 categories, i.e. top category (B-1, Bn and Bn+1) and left category (Am and Am+1), as shown in Figure 1. In each category, the first available candidate along the search order is added to the AMVP candidate list. After the spatial candidates are derived, a temporal candidate is added to the AMVP candidate list. Therefore, the candidate list contains 2 spatial candidates and 1 temporal candidate at most. After that, a redundancy checking process is performed to remove the duplicate motion vectors by made comparisons among all the candidates. If only one candidate is available, a zero checking process is used to identify whether the candidate is zero motion vector or not. If not, zero motion vector is added in the end of the candidate list. Otherwise, there is only one candidate for AMVP, i.e. zero motion vector.

Figure 1 Spatial candidates for AMVP

# Proposed method

In order to simplify the candidate generation process for AMVP, two modifications are proposed.

Firstly, it is proposed to compare only spatial AMVP candidates and exempt the temporal one from the first redundancy checking process. After this change, the maximum checking operations can be reduced from 3 to 1. Meanwhile, this modification can also enhance the error resilience of AMVP when reference frame is lost and temporal candidate is not available at decoder. In such case, the first redundancy checking process cannot be performed at decoder in HM5.0

Secondly, it is proposed to directly add a zero motion vector in the end of AMVP candidate list when only one candidate is available so that zero checking can be removed. The zero checking is actually redundant, because the results of AMVP would be the same no matter whether it is performed or not.

# Experimental results

The proposed method has been implemented on top of the HM5.0 software. Simulations have been conducted using the common test conditions in [1]. As shown in Table 1, the coding performance change is very minor, compared with HM5.0, by using proposed method. For low delay B configurations, some coding gain is observed for class F sequences.

Table 1 Experimental results of proposed method

# Conclusions

In this contribution, two modifications were proposed to simplify AMVP candidate generation. Firstly, the temporal motion vector is exempted from the redundancy checking process. Secondly, zero checking is removed in case only one candidate is available. It is reported that the coding performance and run time of the proposed method are very similar with that of HM5.0.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, November, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).