| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-11 February, 2012 | Document: JCTVC-H0317 |
| --- | --- |

| Title: | CE2 Subtest 4.1: Report on Chroma RQT Depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1,2Xin Zhao, 1Jicheng An, 1Xun Guo, 1Shawmin Lei North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2Siwei Ma, 2Wen Gao School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, jijing.an, xun.guo, shawmin.lei}@mediatek.com {swma, wgao}@pku.edu.cn |
| Source: | 1MediaTek Inc., 2Peking University |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of subtest 4.1 defined in CE2 document. It is proposed to use separate maximum RQT depth configuration for chroma, and by setting the maximum chroma RQT depth configuration as minimum, it is reported that with the proposed method achieves chroma BD-rate reductions of 2.4%, 1.3% and 0.0% for AI-HE, RA-HE and LD-HE, respectively, and chroma BD-rate reductions of 1.9%, 1.4% and 0.9% for AI-LC, RA-LC and LD-LC, respectively. It is also reported that the run time is almost the same with HM5.0.

# Introduction

In HEVC, RQT is applied to allow flexible transform unit (TU) sizes, and the optimal RQT structure is decided at encoder and signaled in the bitstream. In current HM5.0, for a chroma block, the transform depth of corresponding luma block is simply reused, i.e., luma and chroma components share the same transform depth. However, it is not appropriate to always apply the same RQT depth of luma on chroma because chroma components generally have smoother textures than luma. To resolve this issue, Core Experiment 2 (CE2) [1] was established at Geneva meeting, and three proposals [2]-[4] were included in the description of CE2. And in the subtest 4.4, a harmonized solution of the three proposals is also defined.

# Proposed harmonization

In both subtest 4.1 and 4.4, user-defined parameters, max_transform_hierarchy_depth_inter_chroma and max_transform_hierarchy_depth_intra_chroma, are proposed, which define the maximum RQT depth for chroma in inter and intra coding, respectively. Since the maximum transform depth is no longer shared between luma and chroma, the original max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra in HM5.0 are also modified and used only for luma. As shown in Table 1, the proposed modifications in SPS are highlighted in red. In addition, in subtest 4.1, 32x32 transform is disabled for chroma.

Table 1 Proposed user-defined parameters in the SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter_luma | ue(v) |
| max_transform_hierarchy_depth_intra_luma | ue(v) |
| max_transform_hierarchy_depth_inter_chroma | ue(v) |
| max_transform_hierarchy_depth_intra_chroma | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| … | … |
| } |  |

For brevity, let us denote the prefix “max_transform_hierarchy_depth_” as “MaxDepth_” in the following texts. When the chroma RQT depth is smaller than luma, the RQT depth of the current chroma block, namely DepthC, is derived using the maximum RQT depth of chroma and RQT depth of current luma block, namely DepthL, as shown in Table 2.

Table 2 Derivation of chroma RQT depth

|  | DepthC |
| --- | --- |
| Inter | DepthC = min( DepthL, MaxDepth_inter_chroma ) |
| Intra | DepthC = min( DepthL, MaxDepth_intra_chroma ) |

For the special case of setting both MaxDepth_inter_chroma and MaxDepth_intra_chroma as 0, the proposed method employs an implicit setting of chroma RQT depth as:

DepthC = min( DepthL, SIZE_2Nx2N ? 0 : 1 )

The same software used in Subtest 4.4 is employed for running the Subtest 4.1, but different configuration as shown below is used in Subtest 4.1:

- 32x32 transform is disabled for chroma

- MaxDepth_inter_chroma and MaxDepth_intra_chroma are set as 0

# Simulations and results

The proposed method is implemented on top of HM 5.0. Simulations are performed under common test conditions defined in [5]. The test configuration defined in subtest 4.1 is used for running the simulations:

MaxDepth_intra_chroma =0, MaxDepth_inter_chroma =0, no 32x32 chroma transform;

The BD-rate reductions are reported in Table 4. Tests results show good coding performance for chroma components. The proposed method achieves 2.4%, 1.3% and 0.0% BD-rate reductions for AI-HE, RA-HE and LD-HE, respectively, and 1.9%, 1.4% and 0.9% BD-rate reductions for AI-LC, RA-LC and LD-LC, respectively.

Table 4 Experimental results of Subtest 4.1 in CE2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -3.0% | -3.9% | 0.0% | -1.4% | -2.3% |
| Class B | 0.0% | -3.0% | -3.2% | 0.0% | -2.1% | -2.7% |
| Class C | 0.0% | -1.3% | -1.4% | 0.0% | -1.1% | -1.4% |
| Class D | 0.0% | -0.7% | -0.7% | 0.0% | -0.7% | -0.7% |
| Class E | 0.1% | -5.0% | -4.3% | 0.0% | -4.1% | -3.5% |
| Class F | 0.1% | -1.7% | -1.6% | 0.0% | -0.8% | -0.8% |
| Overall | 0.0% | -2.4% | -2.5% | 0.0% | -1.8% | -2.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -2.1% | -3.1% | 0.0% | -1.4% | -2.5% |
| Class B | 0.0% | -2.1% | -2.2% | 0.0% | -1.9% | -2.5% |
| Class C | -0.1% | -0.7% | -0.8% | 0.0% | -0.7% | -1.0% |
| Class D | -0.1% | -0.4% | -0.1% | 0.0% | -0.4% | -0.9% |
| Class F | 0.0% | -1.2% | -1.2% | 0.0% | -0.8% | -0.7% |
| Overall | -0.1% | -1.2% | -1.4% | 0.0% | -1.1% | -1.7% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | -0.7% | -1.3% |
| Class C | -0.1% | 0.2% | 0.1% | 0.0% | 0.0% | -0.4% |
| Class D | -0.1% | 0.9% | 0.2% | -0.1% | 0.3% | -0.5% |
| Class E | 0.0% | -1.7% | -0.5% | 0.0% | -3.4% | -2.4% |
| Class F | -0.2% | -0.6% | -0.8% | 0.1% | -0.6% | -0.6% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | -0.8% | -1.1% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -3.3% | -2.6% |
| Class B | 0.0% | -2.2% | -2.4% |
| Overall | 0.0% | -2.7% | -2.5% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

In this contribution, maximum chroma RQT depth is proposed to be added in SPS to allow separate transform depth for luma and chroma. The results for setting the maximum chroma RQT depth as minimum are tested. Experimental results show that the proposed method can achieve around 1%~4% BD-rate reduction for chroma components.

# References

[1] K. Sugimoto, X. Guo and X. Wang, “Description of Core Experiment 2 (CE2): Chroma RQT depth”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1202r2, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[2] X. Zhao, X. Guo, X. Li, S. Lei, Siwei Ma, Wen Gao, “Residue Quad Tree Depth for Chroma in Intra coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G283, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] A. Minezawa, K. Sugimoto, Shun-ichi Sekiguchi, “Improvement to chroma TU specification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G442, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[4] L. Guo, X. Wang, M. Karczewicz, “Evaluation of Limiting Chroma Transform Depth in RQT on HM4.0”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G980, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[5] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).