| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0318 |
| --- | --- |

| Title: | CE Subtest 4.1: Cross-check Report of Samsung’s Higher granularity of quantization parameter scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Li Xun Guo North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 | Email: | x.li@mediatek.com xun.guo@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of Samsung’s CE Subtest 4.1 proposal: Higher granularity of quantization parameter scaling. The simulation results matched the data provided by the proponent. The code is in line with the proposed algorithm. The revisions on modules other than quantization process may be regarded as necessary changes which are resulted from modifications in quantization process.

# Simulation results

Simulations defined in CE4 description JCTVC-G1204 were conducted. All results matched the data provided by the proponents.

First, the proposed method was tested under common test conditions (JCTVC-G1200), where the tested QPs are equivalent to those of HM-5.0 anchor. As shown in Table 1, almost the same coding performance to the anchor was obtained by the proposed method.

Second, quantization setting with QP+1 was tested, where the QP matches that of HM-5.0 anchor. The simulation results are summarized in Table 2. In general, the coding performance of the proposed method is similar to that of the anchor. The average rate difference between the anchor and the proposed method is moderate (4.8%-9.7%), which indicates that medium-granularity quantization can be achieved by the proposed method.

Table 1 Equivalent QPs to HM-5.0 anchor

# Algorithm analysis

The code of JCTVC-H0336 was reviewed. It is in line with the algorithm proposed in JCTVC-H0336. In addition to quantization, modules of RDO lambda derivation and deblocking were also revised. In general, those modifications may be regarded as straightforward changes which are resulted from the modifications in quantization process.

Table 2 QP+1 (QP is equivalent to that in HM-5.0 anchor)

# Conclusions

JCTVC-H0336 is verified in this report. The simulation results matched the data provided by the proponent. The code is in line with the proposed algorithm. The revisions on modules other than quantization process may be regarded as necessary changes which are resulted from modifications in quantization process.