| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0319 |
| --- | --- |

| Title: | CE2 Subtest 4.2: Crosscheck for Mitsubishi/NHK’s Fixed TU Depth for Chroma in JCTVC-H0235 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xin Zhao and Xun Guo North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 | Tel: Email: | {x.zhao, xun.guo} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the cross-check results on Mitsubishi/NHK’s proposal of fixed TU depth for chroma. It is reported that the cross-check results match those provided by the proponents of G442 and H0235. It is also reported that the software implementation is in line with Mitsubishi/NHK’s proposal.

# Introduction

In JCTVC-G442, fixed TU depth for chroma is used, which is independent from TU depth of luma, and 32x32 TU for chroma is also introduced. In the implementation of JCTVC-G442 on HM5.0, The chroma TU depth is fixed to 0 for CU with 2Nx2N PU, and is fixed to 1 for the rest cases.

The implementation provided by Mitsubishi/NHK, is inline with their algorithm description in JCTVC-H0235. The quad-tree structure for chroma TU is separated from luma, 32x32 transform is introduced for chroma, and the deblocking process is unchanged as it was in HM5.0.

# Simulation Results

We have studied the software and proposal documents provided by Mitsubishi/NHK, and also verified the results provided by them. The test conditions as specified in CE2 [2] are followed. Tests were conducted on Intra, Random Access and Low Delay configurations for both high-efficiency and low-complexity settings [3].

The results are summarized in Table 1.

Table 1 Crosscheck results of BD-Rate performance of G442 on HM5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.4% | -3.1% | 0.0% | -1.5% | -2.4% |
| Class B | -0.1% | -3.0% | -3.1% | 0.0% | -2.5% | -3.2% |
| Class C | -0.1% | -1.3% | -1.2% | 0.0% | -1.2% | -1.4% |
| Class D | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.7% |
| Class E | 0.0% | -5.7% | -4.8% | 0.0% | -5.1% | -4.3% |
| Class F | 0.0% | -1.6% | -1.6% | 0.0% | -1.0% | -1.0% |
| Overall | -0.1% | -2.5% | -2.4% | 0.0% | -2.1% | -2.4% |
|  | -0.1% | -2.5% | -2.4% | 0.0% | -2.2% | -2.4% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -1.8% | -2.7% | 0.0% | -1.4% | -2.7% |
| Class B | -0.1% | -2.6% | -2.7% | 0.0% | -2.5% | -3.2% |
| Class C | 0.0% | -0.8% | -0.7% | 0.0% | -0.7% | -0.9% |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.7% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -1.5% | -1.5% | 0.1% | -1.2% | -1.0% |
| Overall | -0.1% | -1.4% | -1.5% | 0.0% | -1.3% | -1.9% |
|  | -0.1% | -1.4% | -1.4% | 0.0% | -1.3% | -1.9% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.1% | -1.2% | -0.1% | -2.2% | -3.1% |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | -0.4% | -1.2% |
| Class D | 0.0% | 0.8% | 0.3% | -0.1% | 0.3% | -0.9% |
| Class E | -0.2% | -5.5% | -4.1% | 0.0% | -6.1% | -5.3% |
| Class F | -0.1% | -1.4% | -1.7% | 0.2% | -1.5% | -2.1% |
| Overall | -0.1% | -1.2% | -1.1% | 0.0% | -1.8% | -2.5% |
|  | -0.1% | -1.1% | -1.1% | 0.0% | -1.9% | -2.4% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -6.9% | -6.1% |
| Class B | 0.0% | -2.8% | -2.8% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.0% | -4.6% | -4.3% |
|  | 0.0% | -4.6% | -4.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

We have cross-checked Mitsubishi/NHK’s proposal JCTVC-H0235, studied the algorithm and software, and confirmed their results.

# References

[1] A. Minezawa, K. Sugimoto, Shun-ichi Sekiguchi, “Improvement to chroma TU specification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G442, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[2] K. Sugimoto, X. Guo and X. Wang, “Description of Core Experiment 2 (CE2): Chroma RQT depth”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1202r2, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.