| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0326-r1 |
| --- | --- |

| Title: | Non-CE6: Simplification of intra chroma mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Houqiang Li Bin Li Li Li Jinlei Zhang (USTC) Haitao Yang Haoping Yu (Huawei) | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | University of Science and Technology of China Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This proposal presents two simplified intra chroma mode coding schemes. Scheme 1 applies FL binarization and by-pass coding to the four remaining modes (Ver, Hor, DC, and Planar). It is asserted that scheme 1 aligns with the mode coding design for luma component and can reduce decoding complexity. About 0.2% ~ 0.4% bitrate saving for chroma components with AI_HE and AI_LC configurations are also reported. Furthermore, scheme 2 keeps only one remaining mode, i.e. DC mode for intra chroma prediction. By reducing chroma prediction mode, it is reported that the encoding time is reduced by about 7%~8% with acceptable performance loss.

# Introduction

In HM5.0, there are 6 chroma intra prediction modes, DM, LM, Ver, Hor, DC and Planar. For the mode signalling, TU code is used for binarization, and then the binarized bins are coded with context adaptation. DM mode specifies that the chroma component just take the mode of luma component as its own mode. It can be seen as an efficient way of coding chroma mode, just like the most probable mode mechanism for luma component, because the chroma and luma texture share the same directionality in many cases. LM mode is optional and is enabled in HE configuration as specified in the current common test condition. When enabled, LM mode is also frequently selected. Beside DM and LM modes, the four remaining modes (Ver, Hor, DC, and Planar) are in general evenly distributed. So TU code may not be the best choice for coding chroma modes.

Furthermore, the four remaining modes can be seen as the so-called potential duplicate modes as described in JCTVC-G423, since there is a chance that one of them is the same as the luma mode and hence the same as the DM mode. So G423 proposes use only DM and LM modes for chroma intra prediction to remove this redundancy. However it is commented during the previous meeting that an encoder may need one or more explicit modes besides the two implicit modes (DM and LM modes) in certain kinds of implementations for parallel processing. So based on the proposal in G423, we add back DC mode to provide a better solution.

# Solution

## Fix length and bypass coding of intra chroma remaining mode

For intra luma mode coding in HM5.0, the remaining mode is encoded as Fix Length (FL) and bypass coding (except one exception). We propose to apply similar techniques to intra chroma mode, where the modes except DM and LM are treated as remaining mode, using FL (cMax=3) and bypass coding. The syntax element of intra_chroma_pred_mode is also split into three syntax elements, namely intra_chroma_DM_flag, intra_chroma_LM_flag, intra_chroma_rem_mode. Although one syntax element is split into three, no more decoding complexity is introduced. Actually, as the intra chroma remaining modes are coded by FL and BP, the decoding complexity is also reduced a little.

Note that, in HM-5.0 intra chroma mode coding, the first bin is used to represent whether it is DM or not. If it is 1, the second bin is conditionally parsed (depends on whether LM is enabled or not). If it is not DM or LM, the other modes (remaining modes) are also parsed as TU. Here in the proposed method, the first bin is also used to represent whether it is DM or not. If it is 0 (note the semantics of this bin is revered, and also the CABAC initialized probability is also reversed, by setting newInitValue = 2x119 – origInitValue, this kind of modification has no impact on the coding performance), the second bin is conditionally parsed (depends on whether LM is enabled or not). If it is not DM or LM, an FL code (max=3) is parsed to decide the chroma mode form the four remaining modes.

## Reduce the number of intra chroma modes

As mentioned in the introduction section, we propose to keep only DM, LM and one explicit mode for intra chroma mode. The explicit mode is DC mode, and when luma is also DC mode (DC and DM are duplicated modes in this case), Planar mode is used as explicit chroma mode. Because only one explicit mode is added back, we don’t have to explicitly signal this mode information. With the proposed method, a decoder firstly parse one bin, if it equals 1 DM mode is decided. Otherwise, the decoder further parse one bin, if it equals 1, LM mode is decided. Otherwise, the explicit mode is decided.

# Experimental results

The two intra configurations specified in the current common test condition are followed for the simulation. HM5.0 is used as the comparison basis.

- Fix length and bypass coding of intra chroma remaining mode

- Reduce the number of intra chroma modes
With DM mode, the chroma components always share the same mode with luma component. So it is natural for an encoder to perform joint mode decision (JMD) for luma and chroma when searching for the best mode. The results with JMD enabled for both proposed method and HM5.0 are also provided here for more information.

  - Proposed method vs. HM5.0

  - HM5.0 (JMD) vs. HM5.0

  - Proposed method (JMD) vs. HM5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.4% | -0.2% | -1.3% | -1.1% |
| Class B | -0.2% | -0.5% | -1.0% | -0.4% | -1.3% | -1.5% |
| Class C | -0.2% | -0.5% | -0.4% | -0.5% | -1.8% | -1.8% |
| Class D | -0.2% | -0.4% | -0.6% | -0.4% | -1.5% | -1.8% |
| Class E | -0.3% | -1.5% | -1.5% | -0.4% | -2.0% | -2.2% |
| Class F | 0.1% | -0.9% | -0.8% | -0.3% | -1.5% | -1.5% |
| Overall | -0.2% | -0.6% | -0.8% | -0.4% | -1.5% | -1.6% |
|  | -0.2% | -0.6% | -0.8% | -0.4% | -1.6% | -1.7% |
| Enc Time[%] | 112% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Thanks to I2R for the crosschecking efforts.

# Conclusion

As results show, the proposed chroma intra mode coding method has nearly no influence in terms of coding performance. The proposed scheme 1 even has a little coding gain. On the other hand, the spirit and method of FL coding of remaining modes align with the current design for luma intra mode coding. We suggest adopting one of the proposed schemes to the developing standard.

# Text Modification

The text modification is based on JCTVC-G1103_d8.

## Scheme 1: simplification of intra chroma remaining mode coding.

7.3.7

| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| intra_chroma_DM_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_chroma_DM_flag[ x0 ][ y0 ]) { |  |
| if( chroma_pred_from_luma_enabled_flag ) |  |
| intra_chroma_LM_flag[ x0 ][ y0 ] | ae(v) |
| Else |  |
| intra_chroma_LM_flag[ x0 ][ y0 ]=0 |  |
| if( !intra_chroma_LM_flag[ x0 ][ y0 ] ) |  |
| intra_chroma_rem_mode[ x0 ][ y0 ] | ae(v) |
| } |  |

7.4.7

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

intra_chroma_DM_flag[ x0 ][ y0 ] specifies whether the intra prediction mode for chroma samples is direct mode (DM) or not. The array indices x0, y0 specify the location ( x0,  y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The detailed intra prediction mode for chroma samples is specified in 8.3.2.

intra_chroma_LM_flag[ x0 ][ y0 ] specifies whether the intra prediction mode for chroma samples is linear mode (LM) or not. The array indices x0, y0 specify the location ( x0,  y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The detailed intra prediction mode for chroma samples is specified in 8.3.2.

intra_chroma_rem_mode[ x0 ][ y0 ] specifies remaining intra prediction modes for chroma samples. The array indices x0, y0 specify the location ( x0,  y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The detailed intra prediction mode for chroma samples is specified in 8.3.2.

8.3.2

The chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-1 or Table 8-2 below with intra_chroma_pred_mode, intra_chroma_DM_flag[ xB ][ yB ], intra_chroma_LM_flag[ xB ][ yB ], intra_chroma_rem_mode[ xB ][ yB ], IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

- If intra_chroma_DM_flag[ xB ][ yB ] is equal to 1, IntraPredModeC[ xB ][ yB ] is set to IntraPredMode[ xB ][ yB ].

- Else if intra_chroma_DM_flag[ xB ][ yB ] is equal to 0, and intra_chroma_LM_flag[ xB ][ yB ] is equal to 1, IntraPredModeC[ xB ][ yB ] is set to LM.

- Otherwise (both intra_chroma_DM_flag[ xB ][ yB ] and intra_chroma_LM_flag[ xB ][ yB ] are equal to 0). IntraPredModeC[ xB ][ yB ] is derived as specified in Table 8-4 with intra_chroma_rem_mode[ xB ][ yB ] and IntraPredMode[ xB ][ yB ] as inputs.

- Table 8-1 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 0 | 0 | 1 |
| 2 | 2 | 2 | 7 | 1 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | LM | LM | LM | LM | LM |
| 5 | 0 | 1 | 2 | 3 | X |

- Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 1 | 1 | 1 |
| 2 | 2 | 2 | 7 | 2 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | 0 | 1 | 2 | 3 | X |

Table 8-3 – Specification of IntraPredModeC according to the values of intra_chroma_rem_mode[ xB ][ yB ] and IntraPredMode[ xB ][ yB ]

| intra_chroma_rem_mode[ xB ][ yB ] | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 0 | 0 | 1 |
| 2 | 2 | 2 | 7 | 1 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |

9.2.1.1

Table 9-7

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_DM_flag | Error: Reference source not found14 | 0 | 2 | 4 |
|  | intra_chroma_LM_flag | Error: Reference source not found14 | 1 | 3 | 5 |
|  | intra_chroma_rem_mode | Na | na | na | na |

Table 9-12

Table 9-4 – Values of variable initValue for intra_chroma_pred_mode ctxIdx

| Initialisation variable | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 53 | 103 | 85 | 87 | 101 | 87 |

Table 9-54 – Values of variable intra_chroma_DM_flag and intra_chroma_LM_flag ctxIdx

| Initialisation variables | intra_chroma_DM_flag and intra_chroma_LM_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| value | 185 | 135 | 153 | 151 | 137 | 151 |

9.2.2

Table 9-31

| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_DM_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found14 | 0 |
|  | P |  | 0 | Error: Reference source not found14 | 2 |
|  | B |  | 0 | Error: Reference source not found14 | 4 |
| intra_chroma_LM_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found14 | 1 |
|  | P |  | 0 | Error: Reference source not found14 | 3 |
|  | B |  | 0 | Error: Reference source not found14 | 5 |
| intra_chroma_rem_mode | All | FL, cMax = 3 | na | na | na, (use Decode Bypass) |

9.2.3

Table 9-37

| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 0 | 1 | 1 | 1 | na |
|  |  | 4 | 0 | 1 | 1 | 1 | na |
| intra_chroma_DM_flag | Error: Reference source not found14 | 0 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
|  |  | 4 | 0 | na | na | na | na |
| intra_chroma_pred_mode | Error: Reference source not found14 | 1 | 0 | na | na | na | na |
|  |  | 3 | 0 | na | na | na | na |
|  |  | 5 | 0 | na | na | na | na |

## Scheme 2: reduce number of intra chroma mode

Similar to the text above, only two modifications.

- The text related to intra_chroma_rem_mode is not necessary, as when the number of intra chroma mode is reduced, intra chroma remaining mode is not to be signaled.

- 8.3.2

The chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-1 or Table 8-2 below with intra_chroma_pred_mode, intra_chroma_DM_flag[ xB ][ yB ], intra_chroma_LM_flag[ xB ][ yB ], IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

- If intra_chroma_DM_flag[ xB ][ yB ] is equal to 1, IntraPredModeC[ xB ][ yB ] is set to IntraPredMode[ xB ][ yB ].

- Else if intra_chroma_DM_flag[ xB ][ yB ] is equal to 0, and intra_chroma_LM_flag[ xB ][ yB ] is equal to 1, IntraPredModeC[ xB ][ yB ] is set to LM.

- Otherwise (both intra_chroma_DM_flag[ xB ][ yB ] and intra_chroma_LM_flag[ xB ][ yB ] are equal to 0), IntraPredModeC[ xB ][ yB ] is derived as

  - If IntraPredMode[ xB ][ yB ] is 3, IntraPredModeC[ xB ][ yB ] is set to 0.

  - Otherwise (IntraPredMode[ xB ][ yB ] is not 3), IntraPredModeC[ xB ][ yB ] is set to 3.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).