| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0327-r1 |
| --- | --- |

| Title: | Simplification of MVP index coding and AMP mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Houqiang Li Bin Li (USTC) Haitao Yang Haoping Yu (Huawei) | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | University of Science and Technology of China Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes by-pass coding of the bin for MVP index and the bins for AMP mode info. The two simplifications reportedly show 0.0% BD-rate change on average.

# Proposed method

- Fix on MVP index coding in the working draft. In the previous version of HM, we have more than two MVPs, and the MVP is signaled as Truncated Unary (TU). But now, we only have at most two MVPs, and there is no necessary to signal it as TU, using Fix Length (FL) is a better method. This modification has no impact on the performance, just a cleanup (and/or fix) of the WD and the software. We noticed that this aspect has already been fixed in the lated WD text in JCTVC-G1103_d8.

- Bypass coding of MVP index, the context of MVP index is not necessary anymore.

- Bypass coding of AMP bins. There are two bins for AMP, the first bin is used to represent whether the current PU is AMP or not. And when the current PU is AMP, the second bin is used to represent the detailed PU size.

# Experimental results

The inter configurations specified in the current common test condition are followed for the simulation. HM5.0 is used as the comparison basis.

- Bypass MVP index.

- Bypass AMP bins.

- Combination of bypass MVP index and AMP bins.

As AMP is not used for LC, the combined results of LC should be the same with the MVP index only simplification result.

Thanks to Panasonic for the crosschecking efforts.

# Text Modification

The text modification is based on JCTVC-G1103_d8.

- Fix on MVP index coding.

Note that this has already been fixed in the latest WD text.

- Bypass coding of MVP index

Table 9-37

| mvp_l0_flag | Error: Reference source not found | 0 | 0 na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mvp_l0_flag, mvp_l1_flag, mvp_lc_flag | Error: Reference source not found | 1 | 0 na | na | na | na | na |

- Bypass coding of AMP bins.

Table 9-37

| part_mode | Error: Reference source not found | 2 | 0 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 0 | 1 | 0 na | 1 na | na |
|  |  | 6 | 0 | 1 | 0 na | 1 na | na |

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).