| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0331 |
| --- | --- |

| Title: | CE8.c.1: Crosscheck for TI's necessity of sign bits for SAO offsets in JCTVC-H0434 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Chih-Ming Fu, Yu-Wen Huang MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com Yu-Wen Huang +886-3-5670766 ext. 26297 {chihming.fu, yuwen.huang}@mediatek.com |
| Source: | Samsung Electronics Co. Ltd. and MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, performance and visual quality are verified for CE8.c.1, which is sign removal in edge offset coding. Both normative and non-normative solutions were testes. Average luma BD-rate change in verified tests is 0.0% for both normative and non-normative design, but there is some chroma BD-rate increase. The variation of encoding and decoding time compared to anchor is under noise level.

# Introduction

It is known that sample adaptive offset (SAO) proposed in JCTVC-E049 [1] and JCTVC-F057 [2] improves both objective and subjective quality of reconstructed signal. But in very rare cases as was pointed out in JCTVC-G680 [3] it might cause salt and pepper noises because current HM-5.0 encoder only optimizes PSNR without considering subjective quality.

Salt and pepper noise appears when edge offset (EO) operates as sharpening filter making minimums and maximums extremely sharp. In [3] it was suggested to allow SAO operate as smoothing filter only and totally restrict sharpening effect in edge correction. It can be done as encoder-only modification or with syntax change (since EO becomes unsigned value after suggested restriction). Here we verify tests for both variants.

In JCTVC-G222 [4] and later in JCTVC-H0114 [5] statistics of edge offsets was analyzed in HM-4.0 and HM-5.0. It was show that expected value for EO is non zero and so EO syntax in HM is not optimal. Both JCTVC-G222 and JCTVC-H0114 suggest EO syntax modification utilizing information about non-zero EO expectation. Additionally in JCTVC-H0114 it was suggested to constrain EO into predetermined range. This resolves salt and pepper noise as well since sharpening effect of EO is restricted and allows fixed length coding for EO.

Similar to JCTVC-G680, constrained EO in JCTVC-H0114 can be tested as non-normative modification. The difference from JCTVC-G680 will be only range of EO.

JCTVC-G680 was implemented on top of HM-4.0, and JCTVC-H0434 [6] is its version on top of HM-5.0.

# Simulation results

S/w was provided by proponent in time according to CE8 description. Implementation matches JCTVC-H0434. Additionally proponent kindly provided list of cases where JCTVC-H0434 improves visual quality. We confirm visual quality improvement and compare visual quality of verified test and alternative solution in JCTVC-H0114.

We tested the provided JCTVC-H0434 s/w according to JCTVC-G1200 [7]. Summary of test results in comparison with JCTVC-H0114 is shown in Tables 1-2. EO range is different in JCTVC-H0434 and JCTVC-H0114 according to internal bit-depth, so we report 8-bits (Table 1) and 10-bits (Table 2) simulations result separately. We report average across mandatory JCTVC-G1200 tests of luma and chroma BD-rates.

Table 1. Summary of test results for 8 bits internal depth.

|  | Encoder-only |  |  |  | Normative syntax change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JCTVC-H0434 |  | JCTVC-H0114 |  | JCTVC-H0434 |  | JCTVC-H0114 |  |
|  | 0EO(0)8 |  | -1EO(0)3 |  | 0EO(0)8 |  | -1EO(0)3 |  |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| AI-HE | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% |
| RA-HE | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | -0.4% |
| LD-HE | 0.0% | 0.3% | 0.0% | 0.2% | 0.0% | 0.3% | -0.1% | -0.5% |
| All | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% | -0.1% | -0.4% |

Table 2. Summary of test results for 10 bits internal depth.

|  | Encoder-only |  |  |  | Normative syntax change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JCTVC-H0434 |  | JCTVC-H0114 |  | JCTVC-H0434 |  | JCTVC-H0114 |  |
|  | 0EO(0)32 |  | -5EO(0)11 |  | 0EO(0)32 |  | -5EO(0)11 |  |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| RA-HE | 0.0% | 0.6% | 0.0% | 0.0% | 0.0% | 0.7% | 0.0% | 0.1% |

Additionally to JCTVC-G1200 common test condition, we tested LC case with SAO enabled both for anchor and modification. Details of test results can be found in Tables 3, 4 (appendix) and excel spreadsheets attached to this cross-check report.

Our observations are:

- For encoder only change, JCTVC-H0114 performs better than JCTVC-H0434, especially for Inter test cases. So total restriction of SAO sharpening effect is not beneficial performance wise.

- Normative changes of EO syntax in JCTVC-H0434 do not provide performance improvement (sometimes causes performance degradation).

- Maximum performance drop observed in verified test was 0.6% for luma and 5.0% for chroma.

- Maximum performance drop observed in verified test was 0.1% for luma and 1.9% for chroma

.

# Conclusions

Samsung and MediaTek confirm visual quality improvement proposed by TI. We found no objective performance improvement (even some degradation) due to the proposed normative changes. We also provide alternative solution which also resolves salt and pepper noise problem and in opposite verified test shows objective performance improvement as well.

# References

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), “CE13: Sample Adaptive Offset with LCU-Independent Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E049, March 2011.

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek), S. Park, B. Jeon (LGE), A. Alshin, E. Alshina (Samsung), “Sample Adaptive Offset for Chroma,” Document of Joint Collaborative Team on Video Coding, JCTVC-F057, July 2011.

- W.-S. Kim, D.-K. Kwon (TI), “Non-CE8: Method of visual coding artifact removal for SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G0680, November 2011.

- C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), “Non-CE8: Offset coding in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G222, November 2011

- E. Alshina, A. Alshin (Samsung),I. S. Chong, M. Karczewicz(Qualcomm), Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek) “Non-CE8: Sample adaptive offset with constrained ranges of edge offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0114, February 2012

- W.-S. Kim, D.-K. Kwon, “CE8 Subset c: Necessity of sign bits for SAO offsets,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0434, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Appendix.

Table 3. Performance test results for proposed non-normative changes.

|  | All Intra HE |  |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% |  | 0.5% | 0.7% | 0.0% | 0.0% | 0.0% | 0.0% | 1.0% | 1.2% |
| Class B | 0.0% |  | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% |  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% |  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% |  | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U |  | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.8% |  | 0.4% | 0.0% | 0.6% | 1.1% | 0.0% | 1.3% | 1.2% |
| Class B | 0.0% | 0.3% |  | 0.3% | 0.1% | 0.8% | 0.6% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.2% |  | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% |  | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.1% |  | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% |
| Overall | 0.0% | 0.3% |  | 0.2% | 0.0% | 0.5% | 0.5% | 0.0% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y |  | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% |  | 0.6% | 0.8% | 0.1% | 1.1% | 0.9% | 0.1% | 0.5% | 0.5% |
| Class C | 0.0% |  | 0.2% | 0.4% | 0.1% | 0.8% | 0.7% | 0.0% | 0.4% | 0.3% |
| Class D | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | -0.1% |
| Class E | 0.1% |  | 0.0% | 0.3% | 0.4% | 0.4% | 0.6% | 0.1% | -0.1% | 0.7% |
| Class F | -0.1% |  | 0.6% | 0.0% | 0.2% | 1.0% | 1.2% | 0.0% | 0.6% | 0.3% |
| Overall | 0.0% |  | 0.3% | 0.3% | 0.1% | 0.7% | 0.7% | 0.0% | 0.3% | 0.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U |  | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% |  | 0.4% | 0.1% | 0.7% | 0.4% | 0.0% | 0.5% | 0.2% |
| Class C | 0.0% | 0.5% |  | 0.3% | 0.1% | 0.8% | 1.1% | 0.0% | 0.2% | 0.2% |
| Class D | 0.0% | 0.1% |  | 0.0% | -0.1% | 0.4% | -0.1% | 0.1% | -0.1% | 0.0% |
| Class E | 0.1% | -0.2% |  | 1.0% | 0.3% | 0.5% | 0.4% | 0.1% | 0.0% | 1.1% |
| Class F | 0.0% | 0.6% |  | 0.3% | 0.0% | 0.9% | 0.6% | -0.1% | 0.7% | 0.4% |
| Overall | 0.0% | 0.3% |  | 0.4% | 0.1% | 0.7% | 0.5% | 0.0% | 0.3% | 0.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  | 100% |  |  |

Table 4. Performance test results for proposed non-normative changes.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.5% | 0.7% | 0.0% | 0.0% | 0.0% | 0.0% | 1.0% | 1.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.8% | 0.3% | 0.0% | 0.7% | 1.0% | 0.0% | 1.5% | 1.2% |
| Class B | 0.0% | 0.2% | 0.2% | 0.1% | 0.8% | 0.6% | 0.0% | 0.2% | 0.0% |
| Class C | -0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% | 0.1% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.5% | 0.4% | 0.0% | 0.4% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.3% | 0.1% | 1.2% | 0.8% | 0.0% | 0.4% | 0.5% |
| Class C | 0.0% | 0.4% | 0.5% | 0.1% | 0.8% | 0.7% | 0.0% | 0.2% | 0.3% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | -0.1% | 0.8% | 0.3% | 0.2% | 0.5% | 0.1% | -0.9% | 0.7% |
| Class F | -0.1% | 0.3% | 0.2% | 0.0% | 0.8% | 0.8% | -0.2% | 0.4% | 0.2% |
| Overall | 0.0% | 0.3% | 0.3% | 0.1% | 0.7% | 0.5% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.2% | 0.0% | 0.6% | 0.5% | 0.0% | 0.4% | 0.0% |
| Class C | 0.0% | 0.3% | 0.1% | 0.0% | 0.7% | 0.9% | 0.0% | 0.3% | 0.1% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | 0.5% | -0.6% | 0.0% | -0.4% | 0.6% |
| Class E | 0.1% | -0.1% | 0.8% | 0.4% | 0.1% | 0.1% | 0.0% | -0.4% | 0.6% |
| Class F | 0.0% | 0.7% | 0.4% | -0.1% | 0.6% | 0.6% | -0.1% | 0.9% | 0.4% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.5% | 0.3% | 0.0% | 0.2% | 0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 101% |  |  |