| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0332 |
| --- | --- |

| Title: | CE8.d.1: Crosscheck for TI’s reduced number of band offsets in SAO in JCTVC-H0436 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {chihming.fu, yuwen.huang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the modified band offset proposed by TI in JCTVC-H0436. In Test 1, the number of transmitted band offsets is reduced from 16 to 8, so the buffer size to store offsets is reduced by half. The measured BD-rate is 0.0% for luma and -0.4% for chroma. In Test 2, the ranges of the first and last bands are extended. The BD-rate becomes 0.0% for both luma and chroma. The crosschecker would like to point out that the buffer size can be reduced by adding level constraints on the number of offsets per picture, as proposed in JCTVC-G218, JCTVC-H0272, JCTVC-H0273, and JCTVC-H0274. When the buffer size is reduced by half by adding the constraint for the SAO in HM-5.0, no coding efficiency loss is observed.

# Verification

TI provided the HM source codes of their proposed codec described in JCTVC-H0436 [1]. The verification is carried out by setting the following parameters in the provided codes:

#define SAO_NBO

HE-AI, HE-RA, HE-LDB, and HE-LDP were simulated. The simulation includes encoding and decoding.

# Results

The BD-rates from our experimental results match exactly those provided by TI except for run times. Our run time data are not accurate. Conceptually, the two methods in JCTVC-H0436 may not have noticeable impact on run times. The detailed results are listed in JCTVC-H0332_Test1.xls and JCTVC-H0332_Test2.xls.

# Conclusion

The results presented by TI in JCTVC-H0436 are confirmed except for run times.

# References

- W.-S. Kim, D.-K. Kwon, “CE8 Subset d.1: Reduced number of band offsets in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0436, February 2011.