| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0345r1 WG11 Number: m23220 |
| --- | --- |

| Title: | Constrained Tile for parallel decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Kyoko Uchibayashi Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In order to entropy decode bit streams in parallel, decoder needs to search the beginning of each parallel processing units. In this contribution, 1-bit flag in slice header is proposed to indicate whether the slice starts at the top-left position of tile for easier implementation of tile level parallel processing. Furthermore, a set of restrictions on tile and slice for the same purpose is also proposed as a starting point of profile discussion.

# Introduction

In HEVC, independent tile can be processed needs no data from other tile with disabling cross-tile loop filtering [1][2][3]. When tiles can be fully independent, the multi-core decoder can be performed in parallel. However, if the tile is not started at the top-left position of tile, extra storage is needed for buffering data of slice header and parallel processing unit seeking will reduce the processing throughput. Therefore, 1-bit flag in slice header is proposed to remove seeking process with a set of restrictions on tile.

# Proposal

## Start tile in slice flag

1-bit flag in slice header is proposed to indicate whether the slice starts at the top-left position of tile for easier implementation of tile level parallel processing as shown in Table 1.

The top-left position of tile is necessary in slice header in order to set the data of slice header for tile decoding[4].

Table 1. Modified slice_header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| start_tile_in_slice_flag | u(1) |
| entropy_slice_flag | u(1) |
| … |  |
| } |  |

start_tile_in_slice_flag  equal to 1 specifies the slice starts at the top-left position of tile; otherwise, t the slice does not starts at the top-left position of tile.

## A set of restriction on tiles and slices.

To enable the parallel decoding of tiles for easier implementation, a set of restriction is as follows.

- tile_boundary_independence_flag equal to 1

- loop_filter_across_slice_flag equal to 0

- the top-left position of tile is equal to the beginning of slice

In detail:

  - tile_locations_idc equal to 0

  - It is not necessary to code the marker to indicate the top-left position of tile (0x000 002 in JCTVC-F594).

  - If start_tile_in_slice_flag equal to 1, tile_idx_minus_1 in tile header can be coded in slice header.

The decoder can search the starting point of tile by only parsing SPS, PPS and slice header.

# Results

The software used to generate anchor data was the HM-5.1. It was further modified to insert slice header at the first (top-left ) position of tile within the bit stream. We use Tiles configuration showing Table-1 and other settings are shown as follows. Other encoder configuration is set to common test conditions specified in JCTVC-G1200 [5].

- Slice Mode: 1.

- Slice Argument: 1500.

The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table-1 Tile configuration and other setting change

| Tile configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |  |
|  |  | horizontally | vertically | horizontally |  | vertically |
|  | A | 3 | 3 | 15 |  | 8 |
|  | B | 2 | 2 | 15 |  | 8 |
| SliceMode |  |  |  |  | 0 |  |
| LFCrossTileBoundaryFlag |  |  |  |  | 0 |  |
| TileLocationInSliceHeaderFlag |  |  |  |  | 0 |  |
| TileMarkerFlag |  |  |  |  | 0 |  |

Reference: HM-5.1 with inserting slice header at the first (top-left ) position of tile

Tested: Reference with inserting “start_tile_in_slice_flag” for each slice header.

It is tested only for Class A and B, because it is important for HD and larger size for tile level parallel processing.

Table-2  signaling “start_tile_in_slice_flag” within slice header

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 99.9% |  |  | 99.4% |  |  |  |  |  |
| Dec Time[%] | 100.1% |  |  | 100.1% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.0% |  |  | 99.6% |  |  | 100.0% |  |  |
| Dec Time[%] | 100.0% |  |  | 99.6% |  |  | 99.7% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.1% |  |  | 100.5% |  |  |  |  |  |
| Dec Time[%] | 100.3% |  |  | 100.4% |  |  |  |  |  |

# Conclusion

This contribution proposes 1-bit flag in slice header and a set of restriction for tiles and slices. For HD and larger size, it is assumed to use tile for parallel processing, proposed methods show no loss on average in all test conditions. This should be discussed during profile/level definitions should be included in CD.

# References

- A. Fuldseth, M. Horowitz, S. Xu A. Segall, M. Zhou, “Tiles,” JCTVC-F335, Torino, July 2011.

- C.-Y. Tsai, C.-W. Hsu, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei and A. Fuldseth, “AHG4: Non-cross-tiles loop filtering for independent tiles”, JCTVC-G194, Geneva, ,Nov. 2011.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding” JCTVC-G1103, Geneva, Nov. 2011.

- K. Misra, A. Segall, “New results for parallel decoding for Tiles”, JCTVC-F594, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).