| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0347 |
| --- | --- |

| Title: | AHG16 Unification of SDIP and NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | X. Cao, Y. He (Tsinghua) X. Peng (USTC) C. Lai, X.Zheng, L. Liu, J. Zheng (HiSilicon) J. Xu (Microsoft) H. Yang, J. Song, H. Yu (Huawei) J. Lim, B. Jeon (LGE) J. Sole, L.Guo, X. Wang, M. Karczewicz (Qualcomm) J. Xu, E. Maani, A. Tabatabai (Sony) X. Zhang, S. Liu, S. Lei (MediaTek) | Email: | laichangcai@huawei.com jzxu@microsoft.com jaehyun.lim@lge.com joels@qualcomm.com haoping.yu@huawei.com ali.tabatabai@am.sony.com shan.liu@mediatek.com |
| Source: | Tsinghua, USTC, HiSilicon, Microsoft, Huawei, LG, Qualcomm, Sony, MediaTek |  |  |

_____________________________

# Abstract

This contribution reports the results of the unification of SDIP and NSQT. SDIP with 2NxN/Nx2N and 2NxhN/hNx2N PU were tested with the same non square TUs which are the same with NSQT. Results with or without 8x2/2x8 TU are also reported. For all intra HE case, when 2x8/8x2 TU are used, 1.8% and 1.7% gain can be achieved with 128% and 130% encoding time increase for SDIP with 2NxN/Nx2N PU and SDIP with 2NxhN/hNx2N PU respectively. Without 2x8/8x2 TU, for SDIP with 2NxN/Nx2N PU and SDIP with 2NxhN/hNx2N PU, the gain is 1.0% and 1.0%, with 117% and 119% encoding time increase. No obvious decoding time increase was observed.

# Introduction

AHG16 was set up to unify the SDIP and NSQT in last meeting based on the basis software of G558+G754. This document described the unification activity and results.

# Description of AHG16

## Simple introduction of SDIP at last meeting

In the last meeting, SDIP introduced two new Intra partition types 2NxhN and hNx2N for CUs smaller than 64x64, where h means half or 0.5. These new partitions were called as SDIP PUs and could have 32x8, 16x4, and 8x2 sizes for the 2NxhN case and, similarly, 8x32, 4x16, and 2x8 for the hNx2N case. For convenience, all non-square PUs and TUs are summarized in Table 1. The existing intra prediction algorithms are all used for non-square prediction blocks. In particular, 18 prediction modes are supported in 2x8/8x2 PU, 35 modes are supported in 4x16/16x4 and 8x32/32x8 PU, planar mode, MDIS, MDCS and 4x4 MD-DCT/DST were harmonized with non square transform units, the deblocking filtering is also used for the edges belong to 8x8 samples. For SDIP blocks in 32x32 and 16x16 CU, chroma is coded with non square transform units, the first six available modes from a maximum of ten candidate modes, i.e. four luma intra prediction modes, LM, PLANAR, VER, HOR, DC and VER+8, compose the chroma intra prediction mode set.

| Table 1. Non square PU/TU in SDIP at last meeting CU size | Non-square PUs | Non-square TUs | Intra Mode number for non-square PU |
| --- | --- | --- | --- |
| 64x64 | - | - | - |
| 32x32 | 32x8/8x32 | 32x8->32x2 /8x32->2x32 | 35 |
| 16x16 | 16x4/4x16 | 16x4/4x16 | 35 |
| 8x8 | 8x2/2x8 | 8x2/2x8 | 18 |

## SDIP with 2NxN/Nx2N and 2NxhN/hNx2N PU type

In this unification, both PU and TU type are unified with inter slices based on SDIP.

- 2NxN/Nx2N PU are used in SDIP, for comparison and as the basis software of AHG16, SDIP with 2NxhN/hNx2N is also supported by a macro in the AHG16 software.

- The same non square transform units used in NSQT (32x8/8x32, 16x4/4x16) are also used in SDIP, which means 2x32/32x2 TU are removed from SDIP. For smallest (8x8) CU, either 8x2/2x8 or 4x4 transforms can be applied to 8x4/4x8 PU.

- The non square transform and non square coefficients coding in HM5 for NSQT are straight forward used in SDIP, transform reordering and coefficient mapping are not need any more.

Table 2 summarized the key features of SDIP with 2NxN/Nx2N PU. Note that 8x4/4x8 PU and 8x2/2x8 TUs can be switched OFF by a macro. When 2x8/8x2 TU is switched ON, 2x8/8x2 TU is also used in NSQT of inter.

Table 2. SDIP with 2NxN/Nx2N PU

| CU size | SDIP with 2NxN/Nx2N PU | Non-square TUs | Intra Mode number for non-square PU |
| --- | --- | --- | --- |
| 64x64 | - | - | - |
| 32x32 | 32x16/16x32 | 32x8/8x32 | 35 |
| 16x16 | 16x8/8x16 | 16x4/4x16 | 35 |
| 8x8 | 8x4/4x8 | 8x2/2x8 | 35 |
|  |  | 4x4 | 35 |

Table 3 summarized the key features of SDIP with 2NxhN/hNx2N PU. The difference of table 2 and table 3 is SDIP use different non square PU type, different supported intra mode for non square PU in 8x8 CU and different PU type signaling method, in Table2, 2NxN/Nx2N PU type in SDIP is signaled as like in inter to signal to 2NxN/Nx2N PU type and share the same partition size contexts, in Table 3, SDIP with 2NxhN/hNx2N PU, the PU type is signaled by its own contexts without line buffers.

Table 3. SDIP with 2NxhN/hNx2N PU

| CU size | SDIP with 2NxhN/hNx2N PU | Non-square TUs | Intra Mode number for non-square PU |
| --- | --- | --- | --- |
| 64x64 | - | - | - |
| 32x32 | 32x8/8x32 | 32x8/8x32 | 35 |
| 16x16 | 16x4/4x16 | 16x4/4x16 | 35 |
| 8x8 | 8x2/2x8 | 8x2/2x8 | 18 |

# Test results

## Encoder fast search description

In SDIP, the non square PU type maybe skipped according to the coding results of square PU, as illustrated in Figure 1. The coding information of 2Nx2N PU type is used to guide the coding of non square PUs. Square PU is coded as the same as HM5.0, and the bits of 2Nx2N PU type was compared with a threshold before encoding the non square PU types. Non square PU types will be skipped if the bits of 2Nx2N satisfy the conditions to skip the non square PU. The threshold T is calculated as formula (1) when QP is not equal to 0:

T = a *(64.0 / ((QP < 32)? QP: 64))                                        (1)

where variable “a” is set to 5, 15 and 300 for 8x8, 16x16 and 32x32 CU respectively.

For each individual PU, the intra prediction mode is decided by two steps in HM5. First, a set of M candidate modes were selected from total N modes (M<= N) by minimize the HAD based RD cost, where N is the intra mode number in Table 1, then the final mode was selected from the M candidate modes and the most probable modes by calculating the actual rate distortion cost. In Case 1 of SDIP, we set the M as 2 for non square PUs in 16x16 and 32x32 CU, and 3 for non square PU in 8x8 CU. In addition, at the first step of non square PUs in a CU, not all of the N modes will be tested but only the following modes will be tested: the M candidate modes selected in 2Nx2N and the modes whose number is not larger than 9.

When 4x4 transform is used for 8x4/4x8 intra PU, fast search also is enabled for the encoder complexity reduction. The early skip procedure is first applied. If the current CU is 8x8 and the RD cost of NxN is larger than the RD cost of 2Nx2N, the 8x4/4x8 PU checking will be skipped. In the HAD loop, the “number is not larger than 9” is changed to “number is not larger than 3”. Besides that, The number of candidate modes M is set as 2 for all non-square PUs. During the final R-D mode decision, the best modes that are selected by Intra NxN search are added as the additional candidates.

## Test Results

The results are tested in the common test conditions, both HE and loco configurations are tested for all intra, random access and low delay settings. For purpose of easy comparison, results in HE are firstly illustrated and results of intra loco and inter settings will be shown at the last part of this document. Totally four cases were tested as follows:

case 1. SDIP with 2NxN / Nx2N PU type with 2x8 / 8x2 transform;

case 2: SDIP with 2NxhN / hNx2N PU type with 2x8 / 8x2 transform;

case 3: SDIP with 2NxN / Nx2N PU type without 2x8 / 8x2 transform;

- Disable 4x8/8x4 intra PU in 8x8 CU

- Enable 4x8/8x4 intra PU with 4x4 transform in 8x8 CU

case 4: SDIP with 2NxhN / hNx2N PU type without 2x8 / 8x2 transform;

Note that case3 and case4 disabled non square PU (4x8/8x4 or 2x8/8x2) and TU (2x8/8x2) in 8x8 CU.

Table 4 SDIP with 2x8/8x2 TU (left: case1, right: case2)

|  | All Intra HE case1 |  |  | All Intra HE case2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.3% | -3.6% | -4.1% | -1.4% | -3.8% | -4.3% |
| Class B | -1.5% | -3.6% | -3.7% | -1.4% | -3.6% | -3.8% |
| Class C | -1.9% | -2.7% | -2.9% | -1.8% | -2.5% | -2.7% |
| Class D | -1.8% | -2.5% | -2.5% | -1.7% | -2.3% | -2.2% |
| Class E | -2.3% | -6.7% | -6.4% | -2.2% | -6.8% | -6.4% |
| Overall | -1.8% | -3.6% | -3.7% | -1.7% | -3.6% | -3.7% |
|  | -1.8% | -3.7% | -3.7% | -1.7% | -3.6% | -3.7% |
| Class F | -4.8% | -5.2% | -5.2% | -5.9% | -6.0% | -6.2% |
| Enc Time[%] | 128% |  |  | 130% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 5. SDIP without 2x8/8x2 TU (left: case3a, right: case4)

|  | All Intra HE case3a |  |  | All Intra HE Case4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -3.3% | -4.1% | -1.0% | -3.6% | -4.3% |
| Class B | -1.0% | -3.2% | -3.4% | -1.0% | -3.3% | -3.5% |
| Class C | -0.7% | -2.0% | -2.2% | -0.8% | -2.1% | -2.4% |
| Class D | -0.6% | -1.5% | -1.4% | -0.6% | -1.6% | -1.5% |
| Class E | -1.8% | -7.1% | -6.5% | -1.9% | -7.2% | -6.6% |
| Overall | -1.0% | -3.2% | -3.3% | -1.0% | -3.3% | -3.4% |
|  | -1.0% | -3.2% | -3.3% | -1.0% | -3.4% | -3.4% |
| Class F | -0.7% | -1.4% | -1.5% | -0.9% | -1.4% | -1.7% |
| Enc Time[%] | 117% |  |  | 119% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 6. SDIP without 2x8/8x2 TU (left: case3b, right: case4)

|  | All Intra HE case3b |  |  | All Intra HE Case4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.1% | -3.4% | -4.0% | -1.0% | -3.6% | -4.3% |
| Class B | -1.1% | -3.3% | -3.4% | -1.0% | -3.3% | -3.5% |
| Class C | -1.0% | -2.2% | -2.3% | -0.8% | -2.1% | -2.4% |
| Class D | -0.9% | -1.8% | -1.7% | -0.6% | -1.6% | -1.5% |
| Class E | -2.0% | -6.9% | -6.4% | -1.9% | -7.2% | -6.6% |
| Overall | -1.2% | -3.3% | -3.4% | -1.0% | -3.3% | -3.4% |
|  | -1.2% | -3.3% | -3.4% | -1.0% | -3.4% | -3.4% |
| Class F | -1.0% | -1.6% | -1.7% | -0.9% | -1.4% | -1.7% |
| Enc Time[%] | 120% |  |  | 119% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

The results of all intra loco and inter configurations are listed at the section 6 (Additional Results) for giving the additional information.

# Conclusion

Based on the good performance and unification with NSQT, we recommend adopting unified SDIP with the better harmonized PU types into HEVC.

# References

# Additional results (AI loco and inter)

Table7 Case 1. SDIP with 2NxN / Nx2N PU type with 2x8 / 8x2 transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.3% | -3.6% | -4.1% | -1.7% | -3.2% | -4.0% |
| Class B | -1.5% | -3.6% | -3.7% | -2.0% | -3.7% | -4.1% |
| Class C | -1.9% | -2.7% | -2.9% | -2.3% | -2.8% | -3.2% |
| Class D | -1.8% | -2.5% | -2.5% | -2.1% | -2.5% | -2.5% |
| Class E | -2.3% | -6.7% | -6.4% | -3.0% | -7.7% | -7.3% |
| Overall | -1.8% | -3.6% | -3.7% | -2.2% | -3.8% | -4.1% |
|  | -1.8% | -3.7% | -3.7% | -2.2% | -3.9% | -4.1% |
| Class F | -4.8% | -5.2% | -5.2% | -5.5% | -5.0% | -5.4% |
| Enc Time[%] | 128% |  |  | 139% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.8% | -3.3% | -3.9% | -1.1% | -2.7% | -3.6% |
| Class B | -0.9% | -3.1% | -3.1% | -1.2% | -3.0% | -3.2% |
| Class C | -1.1% | -2.4% | -2.2% | -1.2% | -2.0% | -2.3% |
| Class D | -0.9% | -2.2% | -2.1% | -1.0% | -1.5% | -2.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.9% | -2.7% | -2.7% | -1.1% | -2.3% | -2.7% |
|  | -0.9% | -2.8% | -2.7% | -1.1% | -2.3% | -2.7% |
| Class F | -3.7% | -4.4% | -4.6% | -4.5% | -4.5% | -4.8% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -2.5% | -2.1% | -0.5% | -1.5% | -1.5% |
| Class C | -0.6% | -2.4% | -2.1% | -0.5% | -0.8% | -1.1% |
| Class D | -0.6% | -2.9% | -2.5% | -0.4% | -0.5% | -1.4% |
| Class E | -0.7% | -3.8% | -5.6% | -1.2% | -5.9% | -5.5% |
| Overall | -0.5% | -2.8% | -2.9% | -0.6% | -1.9% | -2.1% |
|  | -0.5% | -2.7% | -2.8% | -0.6% | -2.0% | -2.1% |
| Class F | -2.7% | -3.7% | -4.3% | -3.0% | -3.3% | -3.6% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table8 Case 2. SDIP with 2NxhN / hNx2N PU type with 2x8 / 8x2 transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.4% | -3.8% | -4.3% | -1.8% | -3.8% | -4.7% |
| Class B | -1.4% | -3.6% | -3.8% | -1.9% | -4.0% | -4.6% |
| Class C | -1.8% | -2.5% | -2.7% | -2.2% | -3.0% | -3.4% |
| Class D | -1.7% | -2.3% | -2.2% | -2.0% | -2.4% | -2.5% |
| Class E | -2.2% | -6.8% | -6.4% | -2.9% | -8.3% | -7.8% |
| Overall | -1.7% | -3.6% | -3.7% | -2.2% | -4.1% | -4.4% |
|  | -1.7% | -3.6% | -3.7% | -2.2% | -4.1% | -4.4% |
| Class F | -5.9% | -6.0% | -6.2% | -6.6% | -5.9% | -6.3% |
| Enc Time[%] | 130% |  |  | 142% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.8% | -3.3% | -3.8% | -1.2% | -3.1% | -4.4% |
| Class B | -0.8% | -3.1% | -3.1% | -1.2% | -3.3% | -3.5% |
| Class C | -1.0% | -2.5% | -2.4% | -1.3% | -1.9% | -2.4% |
| Class D | -0.9% | -2.1% | -1.9% | -1.0% | -1.4% | -1.7% |
| Class E |  |  |  |  |  |  |
| Overall | -0.9% | -2.7% | -2.7% | -1.2% | -2.4% | -2.8% |
|  | -0.9% | -2.7% | -2.7% | -1.2% | -2.4% | -2.9% |
| Class F | -4.4% | -5.0% | -5.2% | -5.5% | -5.1% | -5.4% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -2.2% | -2.3% | -0.5% | -1.8% | -1.5% |
| Class C | -0.5% | -2.2% | -2.2% | -0.4% | -0.9% | -1.2% |
| Class D | -0.5% | -2.8% | -2.2% | -0.3% | -0.5% | -1.5% |
| Class E | -0.6% | -3.7% | -5.5% | -1.1% | -7.4% | -7.0% |
| Overall | -0.5% | -2.6% | -2.8% | -0.5% | -2.3% | -2.4% |
|  | -0.5% | -2.6% | -2.8% | -0.5% | -2.3% | -2.5% |
| Class F | -3.1% | -3.9% | -4.5% | -3.9% | -4.2% | -4.2% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table9 Case 3a. SDIP with 2NxN / Nx2N PU type without 2x8 / 8x2 transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -3.3% | -4.1% | -1.0% | -2.9% | -3.9% |
| Class B | -1.0% | -3.2% | -3.4% | -1.1% | -3.3% | -3.9% |
| Class C | -0.7% | -2.0% | -2.2% | -0.7% | -2.1% | -2.5% |
| Class D | -0.6% | -1.5% | -1.4% | -0.6% | -1.5% | -1.5% |
| Class E | -1.8% | -7.1% | -6.5% | -2.2% | -8.1% | -7.3% |
| Overall | -1.0% | -3.2% | -3.3% | -1.1% | -3.4% | -3.6% |
|  | -1.0% | -3.2% | -3.3% | -1.1% | -3.4% | -3.6% |
| Class F | -0.7% | -1.4% | -1.5% | -0.7% | -1.2% | -1.6% |
| Enc Time[%] | 117% |  |  | 123% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.6% | -2.6% | -3.3% | -0.6% | -2.5% | -3.6% |
| Class B | -0.6% | -2.1% | -2.2% | -0.7% | -2.8% | -3.1% |
| Class C | -0.4% | -1.4% | -1.4% | -0.4% | -1.5% | -1.8% |
| Class D | -0.2% | -1.5% | -0.8% | -0.3% | -0.6% | -1.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -1.8% | -1.8% | -0.5% | -1.8% | -2.3% |
|  | -0.4% | -1.8% | -1.7% | -0.5% | -1.9% | -2.3% |
| Class F | -0.5% | -1.1% | -1.3% | -0.6% | -1.1% | -1.5% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -0.4% | -0.3% | -1.3% | -1.5% |
| Class C | -0.1% | -0.7% | -0.6% | -0.2% | -0.6% | -0.7% |
| Class D | -0.1% | -0.2% | -0.6% | -0.2% | 0.0% | -1.0% |
| Class E | -0.4% | -4.8% | -4.3% | -0.9% | -6.8% | -6.0% |
| Overall | -0.2% | -1.3% | -1.2% | -0.4% | -1.8% | -2.0% |
|  | -0.2% | -1.3% | -1.2% | -0.4% | -1.9% | -2.0% |
| Class F | -0.5% | -0.6% | -0.9% | -0.6% | -2.0% | -1.8% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table10 Case 3b. SDIP with 2NxN / Nx2N PU type without 2x8 / 8x2 transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.1% | -3.4% | -4.0% |  |  |  |
| Class B | -1.1% | -3.3% | -3.4% |  |  |  |
| Class C | -1.0% | -2.2% | -2.3% |  |  |  |
| Class D | -0.9% | -1.8% | -1.7% |  |  |  |
| Class E | -2.0% | -6.9% | -6.4% |  |  |  |
| Overall | -1.2% | -3.3% | -3.4% |  |  |  |
|  | -1.2% | -3.3% | -3.4% |  |  |  |
| Class F | -1.0% | -1.6% | -1.7% |  |  |  |
| Enc Time[%] | 120% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table11 Case 4. SDIP with 2NxhN / hNx2N PU type without 2x8 / 8x2 transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.0% | -3.6% | -4.3% | -1.1% | -3.2% | -4.4% |
| Class B | -1.0% | -3.3% | -3.5% | -1.2% | -3.5% | -4.2% |
| Class C | -0.8% | -2.1% | -2.4% | -0.8% | -2.3% | -2.7% |
| Class D | -0.6% | -1.6% | -1.5% | -0.6% | -1.5% | -1.6% |
| Class E | -1.9% | -7.2% | -6.6% | -2.3% | -8.7% | -7.9% |
| Overall | -1.0% | -3.3% | -3.4% | -1.2% | -3.6% | -3.9% |
|  | -1.0% | -3.4% | -3.4% | -1.2% | -3.6% | -3.9% |
| Class F | -0.9% | -1.4% | -1.7% | -0.9% | -1.5% | -1.9% |
| Enc Time[%] | 119% |  |  | 126% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.7% | -2.7% | -3.4% | -0.8% | -2.6% | -4.2% |
| Class B | -0.6% | -2.2% | -2.3% | -0.8% | -2.9% | -3.5% |
| Class C | -0.5% | -1.5% | -1.5% | -0.5% | -1.6% | -1.8% |
| Class D | -0.3% | -1.2% | -1.0% | -0.3% | -0.8% | -1.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -1.8% | -1.9% | -0.6% | -2.0% | -2.6% |
|  | -0.5% | -1.9% | -1.8% | -0.6% | -2.0% | -2.6% |
| Class F | -0.7% | -1.3% | -1.5% | -0.7% | -1.4% | -1.8% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -0.5% | -0.3% | -1.3% | -1.5% |
| Class C | -0.1% | -0.6% | -0.3% | -0.2% | -0.6% | -0.7% |
| Class D | 0.0% | -0.6% | -0.3% | -0.2% | 0.0% | -1.0% |
| Class E | -0.4% | -3.8% | -5.2% | -0.9% | -6.8% | -6.0% |
| Overall | -0.2% | -1.2% | -1.3% | -0.4% | -1.8% | -2.0% |
|  | -0.2% | -1.2% | -1.3% | -0.4% | -1.9% | -2.0% |
| Class F | -0.4% | -0.5% | -1.3% | -0.6% | -2.0% | -1.8% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Changes to WD text

The related changes to WD can be found in the attached document of this proposal, for SDIP with 2NxhN/hNx2N PU, please see “H0347_AHG16_SDIP_2NxhN_spec_text.doc”, for SDIP with 2NxN/Nx2N PU, please see “H0347_AHG16_SDIP_2NxN_spec_text.doc”.

# Patent rights declaration(s)

Huawei, Microsoft, HiSilicon, Tsinghua, USTC, LG Electronics, Qualcomm, Sony and MediaTek may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).