| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0349 |
| --- | --- |

| Title: | Simplification on WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jongbum Choi Jeonghoon Park | Email: | tammy.lee@samsung.com jb79.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes (1) Remove non-flush mode (2) No adaptation on synchronization for copying initial CABAC probabilities. There are two modes of non-flush and flush modes to determine CABAC reset at the end of each LCU line and at the first LCU in a thread, CABAC probabilities are copied from the specified LCU indicated by synchronization offsets. The main purpose of non-flush mode and   adaptive synchronization is performance improvement over flush mode and non-adaptive synchronization. However, according to test results, flush mode has only 0.2% loss over non-flush mode and synchronization with the LCU above and to the right by 2 LCUs has 0.0% improvement over the one with the LCU above and to the right by 1 LCU.

# Introduction

In JCTVC-F274 [1], wavefront parallel processing (WPP) was proposed and adopted in order to enable parallel encoding and decoding. WPP consists of synchronizing the CABAC probabilities of the first LCU in each line from the specified LCU of the line above. Synchronization offset to get position of the specified LCU is signalled named as entropy_coding_synchro. It is to give flexibility to find and use the better probability in the first LCU in each line. But it also increases latency for starting the first LCU as entropy_coding_synchro is assigned by the higher value because the LCU cannot be started until the LCU of the above to the right by the number of LCUs specified by entropy_coding_synchro is finished to have its CABAC probabilities (Figure 1).

Delay

Delay

Delay

Delay

Cabac synchro by 1

Cabac synchro by 3

Figure 1. Cabac synchronization in WPP

In JCTVC-F275 [2], Flush mode was proposed and adopted. With a flush and re-initialization of the internal state variables of CABAC at the end of each line of LCUs, each line of LCUs is compressed into a “chunk” of bits that is independent from the desired level of parallelism. On the other hand, Non-flush mode has disadvantages in single core encoding and decoding. In single core design, it needs to store the offset and the range values of each buffer in each thread at the end of each LCU line because it is not flushed but needs to be switched to code another thread (See Figure 2). Furthermore, since each line’s information cannot be delimitated, to decode the information, the amount of bitstream to be loaded is not neat and overloaded with the redundant information of another line’s information within a thread.

Thread 0

Thread 1

Need to store
offset and range
of each buffer

Figure 2. Sequential parsing and decoding of bitstream encoded by two threads in single core design

# Experimental result

The performance tables shown here have been produced for 2 substreams based on HM5.0-dev-misc(r1763) patched by solution of #286. The anchor is generated by WaveFrontSynchro=1 (i.e. Cabac synchronization by the LCU of the above to the right by 1 LCU) and WaveFrontFlush=0 (i.e. non-flush mode).

Table 1 is given by changing the option WaveFrontSynchro from 1 to 2. In this configuration, Cabac synchronizes probabilities of the first LCU in each LCU line by the LCU of the above to the right by 2 LCU.

Table 1. WaveFrontSynchro from 1 to 2

Table 2 is given by changing the option WaveFrontFlush from 0 to 1. In this configuration, Cabac is flushed at the end of LCU in each LCU line.

Table 2. WaveFrontFlush from 0 to 1

Summarization of Table 1 and Table 2 is given as follows.

|  | Change: Synchro 1  2 | Change: NonFlush  Flush |
| --- | --- | --- |
| AI-HE | 0.0 | 0.0 |
| RA-HE | 0.0 | 0.2 |
| LB-HE | 0.0 | 0.4 |
| AI-LC | 0.0 | 0.0 |
| RA-LC | 0.0 | 0.2 |
| LB-LC | -0.1 | 0.4 |
| RA-HE10 | -0.1 | 0.2 |
| Average | 0.0 | 0.2 |

As shown on the above table, the average performance difference by changing synchronization is 0.0% and the average performance difference by flushing is 0.2%.

# Conclusion

Though all the synchronization values have not been tested, adaptive synchronization effect also has not been proven. By changing synchronization value from 1 to 2, performance loss is negligible. In this proposal, non-flushing mode was also tested compared to flushing mode and its performance seems not effective under consideration of its complexity to keep offset and range values in single core design and to load and switch bitstream for each LCU line.

It is proposed to remove non-flush mode and use the fixed synchronization value as 1 for simplification.

# References

- G. Clare, F. Henry, S. Pateux, “Wavefront Parallel Processing for HEVC Encoding and Decoding”, JCTVC-F274, 6th Meeting: Torino, IT, 14-22 July, 2011

- G. Clare, F. Henry, S. Pateux, “Wavefront and Cabac Flush: Different Degrees of Parallelism Without Transcoding”, JCTVC-F274, 6th Meeting: Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-G1103_d6 is shown with strikethrough.

- Newly added text is shown in red.

### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| … |  |

### 7.4.2.2 Picture parameter set RBSP semantics

entropy_coding_synchro specifies whether a specific synchronization process for context variables is invoked before decoding the first LCU of a line of LCUs in a tile. This syntax element also specifies whether a specific memorization process for context variables is invoked after decoding entropy_coding_synchro+1 LCUs of a line of LCUs in a tile. If entropy_coding_synchro is equal to 0, the specific memorization process and synchronization process for context variables are not invoked. If entropy_coding_synchro is different from 0, the specific memorization process is invoked after decoding entropy_coding_synchro+1 LCUs of a line of LCUs in a tile and the specific synchronization process is invoked before decoding the first LCU of a line of LCUs in a tile. The value of entropy_coding_synchro shall be in the range of 0 to tileWidthInLCUs−1, inclusive.

cabac_istate_reset_flag specifies that a specific CABAC state re-initialization process of internal variables is invoked when a vertical independent tile boundary is reached.

### 9.2 CABAC parsing process for slice data

…

The address of the LCU containing the spatial neighbor block T (Error: Reference source not found), tbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current LCU as follows.

tbAddrT = cuAddress( x0 + entropy_coding_synchro * ( 1 << Log2MaxCUSize ) − 1, y0 − 1 ) 	(9-1)

The variable availableFlagT is obtained by invoking the treeblock availability derivation process as specified in subclause Error: Reference source not found with tbAddrT as input.

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and entropy_coding_synchro is not equal to 0, the following applies.

–	If ( tile_boundary_independence_flag is equal to 1 and ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to 0 ) or ( tile_boundary_independence_flag is equal to 0 and tbAddr % picWidthInLCUs is equal to 0, the following applies.

–	When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

–	When cabac_istate_reset_flag is equal to 1, the decoding process for binary decisions before termination specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine specified in subclause Error: Reference source not found.

–	If tile_boundary_independence_flag is equal to 1, the current bitstream pointer is set to indicate BitStreamTable[i] with the index i derived as follows.

num_substreams_per_tile = ( num_substreams_minus1 +1 ) / 
			( ( num_tile_rows_minus1 + 1 ) * ( num_tile_columns_minus1 + 1 ) ) 	(9-2) 
i = ( ( tbAddr − firstTbInTileAddr ) / tileWidthInLCUs ) % ( num_substreams_per_tile ) + 
	num_substreams_per_tile * tileIdx

NOTE: the first part of the above formula (before % sign)  is used to determine the number of LCU lines in the current tile since the current slice has started. This number, modulo the number of substreams per tile, indicates which substream pointer to use for that tile. To this is added the number of substreams present in tiles already processed.

–	Otherwise (tile_boundary_independence_flag is equal to 0), the current bitstream pointer is set to indicate BitStreamTable[i] with the index i derived as follows.

i = ( tbAddr / picWidthInLCUs ) % ( num_substreams_minus1 + 1 )	(9-3)

–	Otherwise, if (tile_boundary_independence_flag is equal to 1 and ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to entropy_coding_synchro + 1) or (tile_boundary_independence_flag is equal to 0 and tbAddr % picWidthInLCUs is equal to entropy_coding_synchro + 1), the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

…