| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0350 |
| --- | --- |

| Title: | CE12 Subtest 5: Cross-verification of Panasonic’s proposal JCTVC-H0148 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Masaru Ikeda Teruhiko Suzuki Sony City Osaki, 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2820 Shuo.Lu@jp.sony.com Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross verification results of Panasonic’s proposal on deblocking filter using adaptive weighting factors (JCTVC-H0148). Source code was provided by the proponent, and the performance has been evaluated under common test conditions as defined in JCTVC-G1200. No mismatch was found comparing to the results reported by the proponent. Code study was also executed and the software reflects exactly what is proposed. Detailed results can be found in the attached excel file.

# Introduction

Proposal JCTVC-H0148[1] is part of CE10[3], in which weighting factors are introduced into the deblocking filter of current HM5.0. Totally five weighting factors are used, three for the strong filter while two for the weak filter. ALL weighting factors are estimated at encoder side, and are transmitted to decoder side.

For the cross-checking results, BD-rate, the encoding/decoding time, and subjective evaluation are shown as below.

# Results

## Test condition

The experiments are executed based on HM5.0. The configurations and common test conditions specified in JCTVC-G1200[2] are used. Both encoding and decoding time is measured on the same machine as anchor, and decoding time is gathered without outputting decoded YUV data.

## BD-rate and encoding/decoding time

Summary of cross-check results are shown in the following tables in terms of BD-rate. Complete data can be found in the attached excel file.

### Mandatory test cases

According to common test conditions JCTVC-G1200[2], cross-check results of mandatory test cases, including Intra/Random access/Low delay B in HE and LC respectively, as well as Random access, HE, 10 bit, are shown as below.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A(8bit) | -0.3% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random access HE |  |  | Random access LC |  |  | Random access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2% | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.1% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.2% |  |  |  |
| Class D | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% | 0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% | -0.2% | 0.0% | 0.0% |
| Class F | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A(8bit) |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | 0.2% | -0.1% | 0.1% | 0.5% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class D | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class E | 0.2% | 0.2% | 0.4% | 0.3% | 0.1% | 0.4% |
| Overall | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.3% |
| Class F | 0.0% | 0.0% | -0.4% | -0.4% | -1.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

### Optional test cases

Additionally, cross-check results of optional test cases, Low delay P in HE and LC, are shown as below.

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A(8bit) |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | 0.0% | 0.1% | 0.6% | -0.3% |
| Class C | 0.1% | 0.1% | -0.1% | 0.0% | 0.3% | 0.3% |
| Class D | 0.2% | 0.6% | 0.0% | 0.3% | 0.1% | 0.7% |
| Class E | 0.3% | 0.0% | 0.0% | 0.2% | 0.2% | -0.3% |
| Overall | 0.1% | 0.2% | 0.0% | 0.2% | 0.3% | 0.1% |
| Class F | 0.0% | 0.3% | 0.5% | -0.2% | -1.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Subjective quality

TBD

# Conclusion

In this contribution, the cross verified results of deblocking filter proposed by Panasonic are reported. It is confirmed that the results match perfectly with those reported by Panasonic.

# References

[1] A-K. Seifert, T. Wedi, M. Narroschke, Deblocking filter using adaptive weighting factors, JCTVC-H0148, Geneva, CH, November 2011.

[2] F. Bossen, Common HM test conditions and software reference configurations, JCTVC-G1200, Geneva, CH, November 2011.

[3] A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 10 (CE10): Deblocking Filtering, JCTVC-G1210, Geneva, CH, November 2011.

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.