| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0351 |
| --- | --- |

| Title: | Cross-check of Technicolor’s resolution adaption using single resolution storage |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This is a cross-check of JCTVC-H0321, Technicolor’s proposal resolution adaption using single-resolution storage. The results have been confirmed to agree with those of the proponent, up to rounding errors due to the method of calculation. Further study is suggested to analyse the results in more detail and understand the effects of different anchors.

# Method

This document is a cross-check of JCTVC-H0321 [1], which proposes a form of resolution adaption which requires only one resolution to be stored for prediction purposes. Combined filtering operations are defined for motion compensation and up- and down-sampling. These remove rounding operations and allow for motion estimation and compensation to be performed without creating and storing a reference at a different resolution.

Software was provided by the proponent, which was a modified form of the AHG18 software incorporating the filtering method. The software was studied and found to comply with the proponent’s description. During the cross-check, the software was modified in order to obtain the results by an internal calculation rather than an external tool.

# Results

The simulation conditions were based on common conditions [2] with additional resolution adaption flags. 16 frames were simulated for random access configurations and 9 frames for low delay configurations. In both cases, the resolution change was at frame 4 (numbered from 0). Both up- and down-sampling resolution switching (i.e. low->high and high->low were simulated).

Comparison is made against an anchor using long half-phase filtering for down-conversion, and matching quarter-phase filtering for up-conversion.

The software produced PSNR results for each frame at the native resolution i.e. with respect to a down-sampled reference for low-resolution pictures and with respect to the original picture for high-resolution picture. The results provided by the proponent combined these values together and used a total bit rate for the sequence (both low and high resolution parts). As provided, the software did not produce this combined PSNR figure and combined bit rate, and so was modified to do so as part of the cross-check. The proponent also subsequently provided a breakdown into low and high resolution parts.

A further slight complication is that when simulating low->high resolution switching for the anchor and for the proposal, it was noted that the low-resolution pictures are different. The proposal modifies the down-sampling for the current picture also to remove rounding effects. This means that the PSNR values are calculated against different (although very similar) anchors. It is believed that this has very little bearing on the results, but more investigation may be required.

All simulations were performed on a homogeneous Intel cluster running Debian Linux. Results matched those of the proponent, up to rounding errors arising from the different methods (internal versus external) of calculation of rate and PSNR. Summary results are included below.

The measured encoder and decoder times are significantly higher for the proposal compared to the anchor.

## Low to high

The summary results are as follows:

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.7% | -0.2% | -0.9% | -0.7% | 0.1% | 0.1% | 0.1% |
| Class B | -0.3% | -1.2% | -0.9% | -0.4% | -0.8% | -0.6% | 0.0% | -0.1% | 0.4% |
| Class C | -0.2% | -0.6% | -0.2% | -0.1% | -1.1% | -1.2% | 0.0% | 0.4% | 0.9% |
| Class D | -0.1% | 0.3% | 1.4% | -0.1% | -0.1% | -0.9% | 0.0% | 1.0% | -0.2% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | -0.9% | 0.2% | 0.6% | -0.9% | 0.7% | 3.0% | -0.8% |
| Overall | -0.2% | -0.5% | -0.1% | -0.2% | -0.7% | -0.9% | 0.0% | 0.3% | 0.3% |
|  | -0.2% | -0.9% | 0.2% | -0.2% | -0.7% | -0.7% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 122% |  |  | 129% |  |  | 123% |  |  |
| Dec Time[%] | 139% |  |  | 148% |  |  | 135% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -0.9% | -0.4% | -0.9% | -0.8% | 0.1% | -0.1% | 0.0% |
| Class C | -0.2% | -0.5% | 0.7% | 0.0% | 0.2% | -0.2% | -0.2% | -0.7% | -0.7% |
| Class D | -0.2% | -0.6% | 0.4% | 0.2% | -2.1% | -1.1% | 0.1% | -0.1% | -0.7% |
| Class E | -0.7% | -2.0% | -1.7% | -0.7% | -0.8% | -2.0% | 0.1% | -1.3% | 0.3% |
| Class F | 0.2% | -0.1% | 0.0% | 0.3% | -0.2% | 0.3% | -0.4% | -0.1% | 0.3% |
| Overall | -0.2% | -0.7% | -0.2% | -0.1% | -0.8% | -0.7% | -0.1% | -0.4% | -0.2% |
|  | -0.2% | -0.8% | -0.1% | -0.1% | -0.8% | -0.6% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 118% |  |  | 114% |  |  | 115% |  |  |
| Dec Time[%] | 129% |  |  | 132% |  |  | 127% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -1.0% | -0.4% | -0.7% | -0.7% | -0.1% | -0.4% | -0.3% |
| Class C | -0.3% | -0.5% | 0.5% | -0.2% | 0.1% | -0.4% | 0.0% | -0.4% | -0.1% |
| Class D | -0.2% | -1.0% | 0.0% | -0.2% | -1.4% | -1.1% | -0.3% | -0.3% | -2.1% |
| Class E | -0.5% | -2.1% | -2.0% | -0.5% | -1.3% | -2.1% | 0.0% | -0.6% | 0.5% |
| Class F | -0.1% | 0.6% | -0.4% | 0.2% | 0.1% | -14.9% | -0.1% | 0.2% | -1.5% |
| Overall | -0.3% | -0.7% | -0.5% | -0.2% | -0.6% | -3.8% | -0.1% | -0.3% | -0.7% |
|  | -0.3% | -0.8% | -0.3% | -0.2% | -0.8% | -0.7% | -0.1% | -0.1% | -0.8% |
| Enc Time[%] | 114% |  |  | 116% |  |  | 114% |  |  |
| Dec Time[%] | 130% |  |  | 131% |  |  | 125% |  |  |

## High to low

The summary results are as follows:

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -2.6% | -3.1% | -0.5% | -2.3% | -2.7% | -0.1% | -0.4% | -0.3% |
| Class B | -0.8% | -3.7% | -3.3% | -0.8% | -3.2% | -2.9% | -0.1% | -0.3% | -0.4% |
| Class C | -0.5% | -1.4% | -1.7% | -0.5% | -1.2% | -1.5% | 0.0% | 0.0% | -0.4% |
| Class D | -1.0% | -2.1% | -2.0% | -0.5% | -2.2% | -1.8% | -0.6% | -0.2% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class F | -0.8% | -1.2% | -0.9% | -0.6% | -1.0% | -1.4% | -0.4% | -0.3% | -0.3% |
| Overall | -0.7% | -2.5% | -2.5% | -0.6% | -2.3% | -2.2% | -0.2% | -0.2% | -0.4% |
|  | -0.7% | -2.5% | -2.5% | -0.6% | -2.3% | -2.2% | -0.2% | -0.3% | -0.4% |
| Enc Time[%] | #REF! |  |  | 134% |  |  | 102% |  |  |
| Dec Time[%] | 165% |  |  | 171% |  |  | 160% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.5% | -1.9% | -1.9% | -0.6% | -2.5% | -2.0% | 0.1% | -0.3% | 0.0% |
| Class C | -0.4% | -1.3% | -0.9% | -0.4% | -0.5% | -0.7% | -0.1% | 0.2% | 0.3% |
| Class D | -0.5% | -1.1% | -0.8% | -0.3% | -1.3% | -0.7% | -0.4% | -0.4% | -0.2% |
| Class E | -1.1% | -4.2% | -2.6% | -1.5% | -4.2% | -3.5% | -0.2% | -0.5% | -0.1% |
| Class F | -0.3% | -0.9% | -0.5% | -0.2% | -4.9% | -0.4% | -0.1% | -0.1% | 0.1% |
| Overall | -0.5% | -1.8% | -1.3% | -0.6% | -2.6% | -1.4% | -0.1% | -0.2% | 0.0% |
|  | -0.5% | -1.5% | -1.3% | -0.6% | -1.8% | -1.4% | -0.1% | -0.3% | 0.0% |
| Enc Time[%] | 117% |  |  | 119% |  |  | 102% |  |  |
| Dec Time[%] | 138% |  |  | 141% |  |  | 133% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -2.6% | -2.4% | -0.7% | -2.9% | -2.4% | 0.0% | -0.4% | -0.2% |
| Class C | -0.3% | -1.1% | -0.7% | -0.3% | -0.5% | -0.6% | 0.0% | -0.1% | 0.2% |
| Class D | -0.4% | -1.1% | -0.9% | -0.5% | -1.1% | -1.3% | -0.1% | -0.1% | -0.4% |
| Class E | -1.3% | -4.3% | -2.8% | -1.8% | -4.1% | -3.3% | -0.3% | -0.6% | -0.5% |
| Class F | -0.2% | -0.7% | -0.1% | -0.3% | 0.2% | -0.6% | -0.1% | -0.1% | -0.4% |
| Overall | -0.5% | -1.9% | -1.4% | -0.7% | -1.6% | -1.6% | -0.1% | -0.2% | -0.2% |
|  | -0.5% | -1.9% | -1.4% | -0.7% | -1.9% | -1.6% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 112% |  |  | 117% |  |  | 103% |  |  |
| Dec Time[%] | 137% |  |  | 141% |  |  | 135% |  |  |

# References

[1]	JCTVC-H0321, “AHG18: Resolution Adaptation Coding (ARC) using single resolution in DPB”, P. Bordes and P. Andrivon (Technicolor), JCT-VC 8th Meeting: San José, CA, USA, 1–10 February, 2012

[2] JCTVC-G1200, “Common HM test conditions and software reference configurations”, F. Bossen, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011