| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0353 |
| --- | --- |

| Title: | AHG8: Comments on HEVC version 1 Profile and Level definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Vivienne Sze Hideo Tamama | Email: | zhou@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

For HEVC version 1 Profile and Level definition, it is proposed to define two profiles, namely Baseline Profile (BP) and High Profile (HP).  BP should support the low-latency applications, while HP supports high-efficiency applications. No 4x4 inter prediction is allowed in any profile.Bi-directional B-slices shall not be allowed in BP, while other tools can be the same for BP and HP. Furthermore, it is proposed to impose constraints on parallel processing tools (Tiles, Wavefront Parallel Processing, Entropy Slices and Slices) for BP and HP. More specifically, tiles shall exist in bitstreams when pixel rate exceeds 1080p@60 (125 Mpixels/sec), and shall not be used when pixel rate is below 1080p@60. Tiles, if used, shall be independent and evenly divided in terms of pixel rate. Slices, entropy slices and wavefont parallel processing (WPP) shall not cross tile boundaries. Constraints shall also be imposed on entropy slices to contain the CABAC initialization overhead.

# Introduction

We propose to define two 8-bit video profiles for HEVC version 1, namely, Baseline Profile (BP) and High Profile (HP).  BP should support the low-latency applications, while HP supports high-efficiency applications. BP does not support Bi-directional B-slices due to low-latency requirements, while HP does. While it is understood that BP may need more tool restrictions, it is acceptable if other coding tools are exactly the same for BP and HP. The following Table shows the difference between proposed BP and HP.

| Coding tools | Baseline Profile | High Profile |
| --- | --- | --- |
| Bi-directional B-slices | NO | Yes |
| No 4x4 inter prediction | NO | NO |
| Other coding tools | Yes | Yes |

Several parallel processing tools have been adopted into HEVC in addition to regular slices used in AVC to support parallel processing for multi-core implementations.  These tools include Tiles, Wavefront Parallel Processing and Entropy Slices.

Tiles enable a picture to be vertically partitioned into groups if LCU as shown in Fig. 1.  Currently in HEVC, tiles can be dependent or independent.

13

14

15

16

LCU #1

Tile #1

Column boundary

Tile #3

Tile #2

1

2

3

4

5

6

7

8

9

10

11

12

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

Row boundary

Fig. 1 – Picture divided into Tiles

Wavefront parallel processing (WPP) shown in Fig. 2 enables each row of LCUs to be processed in parallel.  All dependencies are kept across LCUs. The top dependencies are satisfied as long as the LCU in the row above is processed ahead of the current row (i.e the LCU to the top left of the current LCU is completed.

X

X

X

X

Fig. 2 – Picture divided into LCU rows for WPP

Entropy slices allow for parallel entropy coding, meaning that there are no dependencies across entropy slices during entropy coding.  However, dependencies are allowed across LCUs for source coding (e.g. intra prediction).   LCUs are assigned to entropy slices in raster scan order (similar to the assignment used for slices in AVC).

Arbitrary combinations of parallel processing tools significantly increase codec implementation costs, especially for decoders. Some of combinations might not have actual use cases. Therefore, for BP and HP specification we propose the following constraints on the combinations of parallel processing tools.

# Proposed constraints on parallel processing tools for BP and HP

It is proposed to define configurations and restrictions such that certain tools are only enabled under actual use cases, and in a manner that eliminates unnecessary combinations which will be highly complex to implement.

## Tiles shall be independent

Tiles should remain independent for entropy coding and pixel decoding/prediction.  This enables multiple cores to run in parallel with minimal data communication across cores.

## Tiles shall be divided evenly

Tiles shall be evenly divided in terms of pixels (with an LCU granularity); this will help balance out the workload across cores to enable higher throughput.

## Limit number of tiles based on pixels rate

- Maximum pixel rate per tile is 125 Mpixels/sec

  - Multiple tiles shall not exist for bitstreams with pixels rates less than [125 Mpixels/sec, equivalent to 1920x1080 at 60 fps].

  - Multiple tiles shall exist for bitstreams with pixels rate greater than [125 Mpixels/sec, equivalent to 1920x1080at 60 fps].

- Number  is fixed based on pixel rate

  - Number of tiles = ceil (pixel rate / 125 Mpixels/sec )

## Entropy Slices, Wavefront Parallel Processing and Slices exist within a tile

- Slices, entropy slices and wavefont parallel processing (WPP) shall not cross tile boundaries.  (i.e. they shall be self contained within a tile.)

- Limit number of entropy slices to a maximum of 2 per tile to contain CABAC initialization overhead.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification Working Draft 5” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).