| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0360 (rev. 1) |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li † Gary J. Sullivan * Jizheng Xu * | Tel: Email: | +1 425 882 8080 garysull@microsoft.com jzxu@microsoft.com |
| Source: | † University of Science and Technology of China * Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC and AVC High Profile. It builds upon the prior work reported in JCTVC-G399, updating the results by using the latest available reference software (JM-18.2 and HM-5.0) and configuration common conditions. Experimental results reportedly show that 1) for the high-efficiency all-intra configuration, HM-5.0 can save about 23% in bit rate; 2) for the high-efficiency random-access configuration, HM-5.0 can save about 33% in bit rate; and 3) for the high-efficiency low-delay configuration, HM-5.0 can save about 41% in bit rate when compared with JM-18.2. It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

# Introduction

This contribution presents the results of objective (PSNR-based) compression comparison tests between the current state of the HEVC draft standard and the AVC High Profile as an anchor reference. Experimental results show that, when compared with JM-18.2, the following results are obtained:

- for the high-efficiency all-intra configuration, HM-5.0 can save about 23% in bit rate;

- for the high-efficiency random-access configuration, HM-5.0 can save about 35% in bit rate; and

- for the high-efficiency low-delay configuration, HM-5.0 can save about 43% in bit rate.

Class F was excluded in the average results.

# Experimental results

The JM software was configured with the same coding structure and referencing structure as HM-5.0. The JM and HM used the same QP settings. For the all-intra case, there was nothing special to be configured. For the random access case, the JM uses B pictures with two identical lists at level 0. For low delay cases, the JM uses P pictures with 1+3 referencing structure. The default HM-5.0 high efficiency anchor is used as the HEVC encoding (two 10-bit class A sequences were excluded). Both the JM anchor and HM test encoding used 8-bit encoding and decoding.

The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

The main modifications of JM-18.2 are described below.

It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

## All Intra case

Nothing special was configured.

## Random access case

- Using 4 reference pictures

- Using the same referencing structure as HM-5.0 (by MMCO).

- Using B pictures with two identical lists at level 0.

## Low delay case

- Using 4 reference pictures

- Using a 1+3 referencing structure (by MMCO)

- Using P slices

The summary test results are shown in the tables below.

Table 1 All intra case

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | -24.5% | -21.9% | -16.8% | -23.8% |
| Class B | -24.0% | -27.6% | -24.5% | -24.5% |
| Class C | -20.9% | -25.9% | -26.9% | -22.2% |
| Class D | -17.0% | -20.9% | -22.0% | -18.1% |
| Class E | -28.4% | -19.7% | -24.4% | -27.4% |
| Average Without F | -22.6% | -23.8% | -23.6% | -23.0% |
| Class F | -24.3% | -23.2% | -24.7% | -25.0% |

Table 2 Random access case

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | -33.3% | -31.4% | -29.6% | -33.0% |
| Class B | -41.9% | -43.9% | -39.6% | -42.0% |
| Class C | -32.1% | -36.5% | -37.1% | -33.1% |
| Class D | -30.1% | -34.6% | -36.2% | -31.3% |
| Class E |  |  |  |  |
| Average Without F | -35.0% | -37.8% | -36.7% | -35.6% |
| Class F | -27.5% | -30.2% | -32.0% | -28.6% |

Table 3 Low delay case

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | -46.3% | -46.9% | -47.0% | -46.5% |
| Class C | -38.5% | -44.4% | -45.1% | -39.7% |
| Class D | -35.3% | -43.3% | -43.4% | -36.7% |
| Class E | -49.9% | -46.1% | -49.7% | -49.7% |
| Average Without F | -42.3% | -45.2% | -46.2% | -43.0% |
| Class F | -35.1% | -40.7% | -42.2% | -37.0% |

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-G1200, “Common test conditions and software reference configurations,” Geneva, CH, Nov. 2011.