| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0362 |
| --- | --- |

| Title: | Motion prediction at entropy slice boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li † Jizheng Xu * Houqiang Li † | Email: | jzxu@microsoft.com |
| Source: | † University of Science and Technology of China * Microsoft Corp. |  |  |

_____________________________

# Abstract

Entropy slice is a light weighted parallel mechanism which only breaks the entropy decoding status. The intra prediction can across the entropy the entropy slice boundary. This contribution discusses the problem whether the motion prediction can across the entropy slice boundary or not.

# Introduction

Entropy slice is a light weighted parallel mechanism which only breaks the entropy decoding status. The intra prediction can across the entropy the entropy slice boundary. This contribution discusses the problem whether the motion prediction can across the entropy slice boundary or not.

# Solution

Before the MVP index and merge index parsing dependency issue solved, the motion prediction cannot across the entropy slice boundary.

But from HM-4.0, the parsing dependency problem has been fully solved. Thus, the entropy decoding process is fully independent with the motion reconstruction process.

To keep consistency with the behavior of intra prediction at entropy slice boundary, we propose that the motion prediction can across the entropy slice boundary.

In the current WD (JCTVC-G1103_d3.doc), it does not say that the motion information at the entropy boundary could not be used as the motion prediction for other entropy slices, as described in 6.4.3.

6.4.3 Derivation process for the availability of treeblock addresses

Input to this process is a treeblock address tbAddr.

Output of this process is the availability of the treeblock tbAddr.

NOTE – The meaning of availability is determined when this process is invoked.

The treeblock is marked as available, unless one of the following conditions is true in which case the treeblock shall be marked as not available:

–	tbAddr < 0

–	tbAddr > CurrTbAddr

–	the treeblock with address tbAddr belongs to a different slice than the treeblock with address CurrTbAddr and the entropy_slice_flag of the slice containing the treeblock with address CurrTbAddr is equal to 0.

–	tile_boundary_independence_flag is equal to 1 and the treeblock with address tbAddr is contained in a different tile than the treeblock with address CurrTbAddr.

The software modification is also simple, when getting MVPs and merge candidates, the parameter bEnforceEntropySliceRestriction should be set to false instead of default true.

This modification should have no impact on the common test conditions.

This is a mismatching between WD and software and we think what WD does is correct and propose to fix this bug in the software.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).