### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_APS_enabled_flag ¦ ¦ sample_adaptive_offset_enabled_flag ¦ ¦ adaptive_loop_filter_enabled_flag ) { |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| slice_sample_adaptive_offset_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag) |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if (slice_adaptive_loop_filter_flag) |  |
| if (!loop_filter_across_slice_flag){ |  |
| num_sgu_alf_padding | u(v) |
| } |  |
| } |  |
| aps_id | ue(v) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| … … |  |

### Slice header semantics

slice_address specifies the address in slice granularity resolution in which the slice starts and shall be represented by ( Ceil( Log2( NumLCUsInPicture ) ) + SliceGranularity ) bits in the bitstream where NumLCUsInPicture is the number of LCUs in a picture.

The variable LCUAddress is set to ( slice_address >> SliceGranularity ) and represents the LCU part of the slice address in raster scan order. The variable GranularityAddress is set to ( slice_address - ( LCUAddress << SliceGranularity ) ) and represents the sub-LCU part of the slice address expressed in z-scan order.

[Ed. (BB): “z-scan order” is not defined anywhere and should be defined, or the phrase herein is changed not to use the term.]

The variable SliceAddress is then set to ( LCUAddress << ( log2_diff_max_min_coding_block_size << 1 ) ) + ( GranularityAddress << ( ( log2_diff_max_min_coding_block_size << 1 ) – SliceGranularity ) and the slice decoding starts with the largest coding unit possible at the slice starting coordinate.

num_sgu_alf_padding specifies the total number of slice granularity units in coding order that lie on the bottom of an LCU but not on the bottom of a picture for which adaptive loop filter is applied with padding inside a coding unit. It shall be represented by ( Ceil( Log2( NumLCUsInPicture – slice_address –  NumSGUonPicBd ) ) + SliceGranularity) bits in the bitstream where NumLCUsInPicture is the number of LCUs in a picture and NumSGUonPicBd is the number of slice granularity units on the picture bottom boundary.

#### Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

- a filter index array of (nS)x(nS), fIdx.

- A variable noSGUinSlice specifying the total number of slice granularity units in a slice that lie on the bottom of an LCU but not on the bottom of the picture up to and including the current slice granularity unit in the coding order.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_flag.

A variable lcuHeight is set equal to ( 1 << Log2MaxCUSize ) and a variable vbLine is set equal to lcuHeight – 4.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as following ordered steps:

- A variable dist2VB is derived as follows.

dist2VB = ( ( yC + y ) % lcuHeight – vbLine )		(8-1)

- A variable dist2VB is modified as follows.

- If dist2VB is less than –vbLine+2 and yC is larger than 2, dist2VB is set equal to dist2VB+lcuHeight,

- Otherwise, if yC+lcuHeight is equal or larger than pic_height_in_luma_samples, dist2VB is set equal to 5.

- A variable dist2VB is modified as follows.

- If the vertical coordinate of the bottommost luma sample in the current slice granularity unit is equal to the vertical coordinate of the bottom most luma sample in the current tile, dist2VB is set equal to 5.

- If noSGUinSlice > num_sgu_alf_padding, dist2VB is set equal to 5.

- If alfFilterShape is equal to 0 and dist2VB is equal to 0 or -1, the following applies.

recFiltPicture[xC+x][yC+y]=s''[xC+x,yC+y]		(8-2)

- Otherwise, if alfFilterShape is equal to 0 and dist2VB is equal to 1 or -2, the following applies.

recFiltPicture[xC+x][yC+y]s''[xC+x,yC+y]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y][i]]>>1	(8-3)

where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Table 8-1 and Table 8-2, respectively.

- Otherwise, the following applies.

recFiltPicture[xC+x][yC+y]=i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y][i]]	(8-4)

Table 8-1 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | 0 | 2 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | -2 | 0 | 2 |
| alfFilterShape = 1 | 0 | 0 | 0 | 0 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 0 | 0 | 0 | 0 |

Table 8-2 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process

| i |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dist2VB < -2 or dist2VB > 1 | alfShape == 0 | -2 | -2 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| dist2VB == -2 | alfShape == 0 | -2 | -2 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| dist2VB == 1 | alfShape == 0 | -1 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| dist2VB < -4 or dist2VB > 3 | alfShape == 1 | -4 | -3 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 |
| dist2VB == -4 or dist2VB == 3 | alfShape == 1 | 0 | -3 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 0 |
| dist2VB == -3 or dist2VB == 2 | alfShape == 1 | 0 | 0 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 0 | 0 |
| dist2VB == -2 or dist2VB == 1 | alfShape == 1 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| dist2VB == -1 or dist2VB == 0 | alfShape == 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

(a) alfFilterShape == 0                          (b) alfFilterShape == 1

Figure 8-1 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)

#### Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

- a variable cIdx specifying the chroma component index.

- A variable noSGUinSlice specifying the total number of slice granularity units in a slice that lie on the bottom of an LCU but not on the bottom of the picture up to and including the current slice granularity unit in the coding order.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_chroma_flag.

A variable lcuHeight is set equal to ( 1 << ( Log2MaxCUSize – 1 ) ) and a variable vbLine is set equal to lcuHeight – 2.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as the following ordered steps:

- A variable dist2VB is derived as follows.

dist2VB = ( ( yC + y ) % lcuHeight – vbLine )		(8-5)

- A variable dist2VB is modified as follows.

- If dist2VB is less than –vbLine+2 and yC is larger than 2, dist2VB is set equal to dist2VB+lcuHeight,

- Otherwise, if yC+lcuHeight is equal or larger than pic_height_in_luma_samples >> 1, dist2VB is set equal to 5.

- A variable dist2VB is modified as follows.

- If the vertical coordinate of the bottommost luma sample in the current slice granularity unit is equal to the vertical coordinate of the bottom most luma sample in the current tile, dist2VB is set equal to 5.

- If noSGUinSlice > num_sgu_alf_padding, dist2VB is set equal to 5.

- If alfFilterShape is equal to 0 and dist2VB is equal to 0 or -1, the following applies.

recFiltPicture[xC+x][yC+y]=s''[xC+x,yC+y]		(8-6)

- Otherwise, if alfFilterShape is equal to 0 and dist2VB is equal to 1 or -2, the following applies.

recFiltPicture[xC+x][yC+y]s''[xC+x,yC+y]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]]>>1	(8-7)

where N is set equal to AlfNumCoeffChroma-1 and horPos[i] and verPos[i] are specified in Table 8-1 and Table 8-2, respectively.

- Otherwise, the following applies.

recFiltPicture[xC+x][yC+y]=i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]]	(8-8)