| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0373 |
| --- | --- |

| Title: | CE5.f: Cross-check of Microsoft's JCTVC-H0361 Residual Scalar Quantization for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xing Wen Oscar C. Au Dept. of ECE, Hong Kong University of Science and Technology Clearwater Bay, Hong Kong | Tel: Email: | +852 6626 1845 wxxab@ust.hk eeau@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution presents a summary of a cross-check performed by the Hong Kong University of Science and Technology on Microsoft’s proposal for Residual Scalar Quantization (RSQ) JCTVC-H0361. The cross-check performance results reportedly match perfectly to those provided by Microsoft. The implemented algorithm is also reportedly in line with Microsoft’s proposal.

# Introduction

In JCTVC-H0361, RSQ is introduced which is a transform skipping mode for each intra TU. Basically for a RSQ TU, the transform is skipped.

# Simulation results

Tests were conducted on configurations as specified in the common test conditions [1]. Low-Delay P configurations were not tested (and included in the attached Excel spreadsheets). The anchor is HM 5.0.

And the time factors are not reported since we are running simulation on machines with different hardware configurations.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | -0.2% | 0.1% | 0.1% |  |  |  |
| Class B | -0.1% | 0.2% | 0.2% | -0.2% | 0.2% | 0.2% |  |  |  |
| Class C | -0.5% | 0.0% | 0.0% | -0.9% | 0.2% | 0.4% |  |  |  |
| Class D | -0.4% | 0.0% | 0.0% | -0.9% | 0.2% | 0.3% |  |  |  |
| Class E | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Overall | -0.2% | 0.1% | 0.1% | -0.5% | 0.2% | 0.2% |  |  |  |
|  | -0.2% | 0.1% | 0.1% | -0.5% | 0.2% | 0.2% |  |  |  |
| Class F | -10.0% | -8.3% | -8.1% | -12.0% | -8.2% | -8.3% |  |  |  |
| Enc Time[%] | 129% |  |  | 148% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.3% | -0.1% | 0.1% | 0.2% | 0.3% |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.3% | 0.4% | 0.1% | 0.0% | 0.0% |
| Class C | -0.3% | 0.1% | 0.0% | -0.7% | 0.0% | 0.1% |  |  |  |
| Class D | -0.2% | -0.1% | 0.1% | -0.5% | 0.4% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.1% | -0.3% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | -0.1% | 0.0% | 0.1% | -0.3% | 0.2% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class F | -7.3% | -6.2% | -6.2% | -10.8% | -5.9% | -5.8% |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class C | -0.1% | -0.2% | 0.1% | -0.1% | 0.0% | -0.2% |  |  |  |
| Class D | 0.0% | -0.3% | -0.5% | -0.1% | 0.5% | -0.9% |  |  |  |
| Class E | 0.0% | -0.5% | -0.4% | 0.0% | 0.2% | -1.0% |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | -0.1% | 0.2% | -0.4% |  |  |  |
|  | 0.0% | -0.1% | -0.1% | -0.1% | 0.1% | -0.5% |  |  |  |
| Class F | -4.3% | -4.0% | -3.8% | -7.7% | -4.0% | -4.1% |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |

These results match those provided by the proponents.

# References

[1] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Nov. 2011.