| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0374 |
| --- | --- |

| Title: | Cross-check of Hisilicon's JCTVC-H0465 for simplification of mode dependent intra smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xing Wen Oscar C. Au Dept. of ECE, Hong Kong University of Science and Technology Clearwater Bay, Hong Kong | Tel: Email: | +852 6626 1845 wxxab@ust.hk eeau@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution presents a summary of a cross-check performed by the Hong Kong University of Science and Technology on Hisilicon’s proposal simplification of mode dependent intra smoothing JCTVC-H0465. The cross-check performance results reportedly match perfectly to those provided by Microsoft. The implemented algorithm is also reportedly in line with Microsoft’s proposal.

# Simulation results

Tests were conducted on configurations as specified in the common test conditions [1]. The anchor is HM 5.0 and only AI-HE and AI-LC were tested.

And the time factors are not reported since we are running simulation on machines with different hardware configurations.

Table1.  Removing intra smoothing filtering for 8x8 intra predictions except planar mode

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.06% | -0.02% | -0.01% | 0.11% | -0.04% | -0.05% |
| Class B | 0.06% | 0.00% | -0.02% | 0.10% | -0.02% | -0.04% |
| Class C | 0.00% | 0.01% | 0.04% | 0.03% | -0.01% | -0.02% |
| Class D | 0.03% | 0.04% | 0.02% | 0.05% | 0.00% | -0.01% |
| Class E | 0.03% | 0.00% | -0.02% | 0.08% | -0.06% | -0.02% |
| Overall | 0.04% | 0.01% | 0.00% | 0.07% | -0.02% | -0.03% |
|  | 0.03% | 0.00% | 0.01% | 0.07% | -0.02% | -0.03% |
| Class F | -0.03% | -0.03% | -0.01% | -0.01% | -0.02% | -0.04% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table2. Removing intra smoothing filtering for 8x8 intra predictions except planar mode

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.05% | 0.00% | 0.03% | 0.09% | 0.01% | -0.01% |
| Class B | 0.04% | 0.00% | 0.00% | 0.07% | 0.00% | -0.01% |
| Class C | -0.01% | 0.00% | 0.03% | 0.00% | -0.01% | -0.01% |
| Class D | 0.02% | 0.08% | 0.04% | 0.04% | 0.01% | -0.03% |
| Class E | 0.05% | 0.03% | 0.07% | 0.09% | -0.02% | 0.00% |
| Overall | 0.03% | 0.02% | 0.03% | 0.05% | 0.00% | -0.01% |
|  | 0.03% | 0.02% | 0.04% | 0.05% | 0.00% | -0.01% |
| Class F | -0.05% | 0.07% | -0.01% | -0.03% | -0.02% | -0.02% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

These results match those provided by the proponents.

# References

[1] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Nov. 2011.