| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0375 |
| --- | --- |

| Title: | CE8.d.2: SAO Line Buffer Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | Semih Esenlik +49 6103 766 144 {Semih.Esenlik, Matthias.Narroschke, Thomas.Wedi}@panasonic.de |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of CE8 on in-loop filtering. Proposed is a method to reduce the line memory which is required by Sample Adaptive Offset (SAO) operation in the encoder and decoder. In HM-5.0 partial pixel classification results of SAO can be stored in the line memory instead of actual pixel values. Therefore the line memory requirement for SAO is 0.25 lines for luma and each of the chroma components in the decoder. In the encoder the line memory required for SAO estimation process is 0.75 lines for luma and each of the chroma components. In the contribution a simple line buffer compression method is proposed that exploits the high amount of correlation between partial pixel classification results of neighboring pixels. As a result the line memory requirements of the encoder and decoder for SAO are reduced by 50% for the luma and chroma components (0.125 lines of reduction for decoder and 0.375 lines of reduction for encoder). The proposed method does not have any impact on the coding efficiency or runtimes as well as on the visual quality.

# Introduction

To reduce external frame buffer access, hardware implementations of encoders and decoders usually employ on-chip memory that is used to store data that is used multiple times. A specific type of on-chip memory is called “line memory”. In Figure 1 the line memory that is required by the deblocking filter is depicted. During the encoding/decoding process the coded blocks are processed one by one in a raster scan order. Since the bottom and right neighbors of the currently processed block are not yet available, the three loop filtering operations Deblocking Filter (DF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) have to be delayed at the coding block borders as they require unavailable samples from not-yet-processed neighboring blocks. For quick memory access, decoders and decoders typically store those samples that are required for delayed filtering in the line memory.

During the 7th JCT-VC meeting in Geneva, option 1 of the proposal JCTVC-G212 [1] has been adopted that eliminated the line memory requirement of ALF completely. However for the LCU based processing of DF and SAO, still 4.25 lines of line memory is required to be implemented in total for the luma component in the decoder and 4.75 lines in the encoder. The breakdown of the line memory requirement for each filtering stage can be summarized as:

- Deblocking filter: 4 lines for luma and 2 lines for each chroma component both in the encoder and decoder.

- SAO: 0.25 lines for luma and 0.25 lines for each chroma component in the decoder. 0.75 lines for luma and 0.75 lines for each chroma component in the encoder.

The size of the line memory that needs to be implemented is proportional to the frame width. As an example if the resolution of the frame is given as 2k by 4k, then 1 line of line memory corresponds to 31.25k bits of on-chip storage for luma component.

Block that is currently decoded

Horizontal Line Memory

Frame Width

Vertical Line Memory

4 Lines

Figure 1: Horizontal and vertical line memory for Deblocking Filter.

# Line buffer analysis for SAO

## Partial pixel classification results of SAO

Figure 2 shows the edge offset classification patterns of SAO in HM5.0. The pixel categories are computed according to rules that are defined in Table 1.

Figure 2: Edge offset classification patterns

Table 1: Edge offset classification rules

For edge offset, the pixel category can be computed according to the following equation:

Category=sign(CenterPixel−NeigborPixel1)+sign(CenterPixel−NeigborPixel2)

Where sign(X)=1X>00X==0−1X<0

In the equation the terms sign(CenterPixel−NeigborPixel1) and sign(CenterPixel−NeigborPixel2) are called partial pixel classification results.

## Line Memory for SAO in HM5.0

The Figure 3 below shows the SAO application/estimation process in LCU-based processing. The SAO has to be applied after the horizontal and vertical deblocking operations are complete. In the figure last 3 pixel rows of the LCU (represented by light green and grey pixels) can not be deblocked by vertical deblocking since the LCU below is not yet available. Therefore SAO operation cannot use the last three lines of pixels in an LCU and needs to be delayed as well. As a result last 4 pixel rows in an LCU cannot be processed by SAO, since samples that are not yet deblocked are required by the SAO operation.

Figure 3: SAO application in LCU-based processing, light green pixels represent partially deblocked, dark green pixels represent fully deblocked samples.

Although the 4th line at the LCU boundary cannot be processed by SAO in the above figure, the partial pixel classification result can be computed. The following is the equation for the pixel category calculation:

Category=sign(CenterPixel−NeigborPixelUP)+sign(CenterPixel−NeigborPixelDOWN)

Please note that the term sign(CenterPixel−NeigborPixelUP)in the equation can already be computed during the current processing instant (since both center pixel and the upper pixel are deblocked fully).

Finally in the line memory the result of thesign(CenterPixel−NeigborPixelUP), which is called the partial pixel classification function, can be stored instead of the actual pixel values of the upper pixels. Since the partial pixel classification results can take on 3 distinct values (-1, 0 and 1), the required line memory per pixel is two bits for SAO.

As an example if the resolution of the frame is given as 2k by 4k, then the line memory requirement of SAO corresponds to 7.8k bits for luma and 3.9k bits for both chroma components in the decoder.

During the estimation process of SAO, the encoder usually collects statistics for each one of the pixel classification methods. Three of the pixel classification methods require line memory which are 45-Degree, 90-Degree and 135-Degree Edge Offset pixels classification methods. Therefore the line memory that is required to be implemented in the encoder is 3 times higher than that of the decoder. As a result in the encoder the line memory requirement for SAO is 23.5k bits for luma and 11.7k bits for each chroma component.

# Proposed SAO line memory reduction method

The proposed line memory reduction method utilizes the high correlation between the partial pixel classification results of neighboring pixels. If for instance a center pixel is smaller in intensity value than its upper neighbor, then it is highly probable that the center pixel to the right will be smaller in intensity value than its upper neighbor as well. In other words in the figure below sign(P1,CENTER−P1,UP) will very likely have the same value assign(P2,CENTER−P2,UP).

Figure 4: Four neighboring pixels

According to the proposed method the partial pixel classification results of two neighboring pixels are coded together in the line memory. The pseudo-algorithm for the luma component is given as follows (for the complete algorithm please refer to Section 8):

Part_class_result_P1 =sign(P1,CENTER−P1,UP);

Part_class_result_P2 =sign(P2,CENTER−P2,UP);

If ((vertical coordinate of the pixel % height of LCU) == 4th line above LCU boundary)

{

If (Part_class_result_P1 != Part_class_result_P2 && (horizontal coordinate of P1, CENTER)%2 == 0)

{

Part_class_result_for_P1 = 0;

Part_class_result_for_P2 = 0;

}

}

According to the pseudo-algorithm, a representative partial pixel classification result is stored in the line memory for every two pixels. Therefore the line memory for SAO is reduced by 50%. If a picture size of 2k by 4k is assumed for the worst case decoding scenario, then the achieved reduction in the line memory corresponds to 3.9k bits (from 7.8k bits in HM5.0 to 3.9k bits) for luma and 1.95k bits (from 3.9k bits in HM5.0 to 1.95k bits) for chroma components in the decoder.

The line memory reduction method is applied only for 45-Degree, 90-Degree and 135-Degree Edge Offset pixels classification methods. 0-Degree Edge Offset and Band Offset pixel classification methods do not require line memory hence the proposed line memory reduction method is not applied.

## Line memory reduction in the encoder

The proposed line memory reduction method reduces the line buffer requirements in the encoder by 50% as well. As a result the achieved reduction in the line memory corresponds to 0.375 lines for luma and each of the chroma components (from 0.75 lines to 0.375 lines).

# Experimental results

As the reference, the official tagged version of the HM5.0 software [2] is used. The simulations are performed according to the common conditions [2]. The results are presented in Table 2.

The proposed method has almost no impact on the coding gain. Moreover the encoding and decoding times are same as the anchor.

Table 2: Simulation results according to [2].

Moreover visual tests have been performed in order to asses the performance of the proposed tool. It has been verified with informal tests that the proposed tool does not create any visual artifacts.

# Conclusions

In the contribution a method for reducing the line memory requirement for the LCU-based SAO filtering procedure is proposed. The proposed method provides 50% reduction in the horizontal line memory for the luminance and for each chrominance components both in the encoder and decoder. The amount of line memory reduction in the decoder corresponds 0.125 lines both for the luma and chroma components. In the encoder the reduction in the line memory requirement corresponds to 0.375 lines for both luma and chroma components. Results of experiments following the common test conditions show no change in coding gain and runtimes. Moreover it has been verified by informal tests that no visual artifacts are created.

Since the amount of line memory that is required to be implemented for SAO in encoder and decoder chips is reduced by 50%, the proposed method is thought to be very beneficial for LCU-based processing approach. We propose to include this technique in the HEVC standard.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” Document of Joint Collaborative Team on Video Coding, JCTVC-G212, November 2011

- HM 5.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva, November, 2011.

# WD Modifications

##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Variable LcuHeight is set equal to 1<<Log2MaxCUSize if cIdx is equal to 0, otherwise, set equal to 1<< (Log2MaxCUSize-1).

Variable VBLine is set equal to LcuHeight-4 if cIdx is equal to 0, otherwise, set equal to LcuHeight-2.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-3.

- A variable edgeIdx is specified as

if ( (yC+j) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_Height  &&  saoTypeIdx != 1)

{

if ( (xC+i) % 2 == 0)

{

upSign0 = Sign( recPicture[ xC + i, yC + j ] – recPicture[ xC + i + hPos[0], yC + j + vPos[0] ] );

upSign1 = Sign( recPicture[ xC + i + 1, yC + j ] – recPicture[ xC + i + 1 + hPos[0], yC + j + vPos[0] ] );

upSign   = (upSign0 == upSign1)  ?  upSign1 : 0 ;

}

edgeIdx = 2 + upSign +  Sign( recPicture[ xC + i, yC + j ] – recPicture[ xC + i + hPos[1], yC + j + vPos[1] ] );	(8-1)

}

else

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-2)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-3)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]	(8-4)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-4.

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-5)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table 8-3 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | -1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 8-4 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |