#### 7.4.2.5   Adaptation parameter set RBSP semantics

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, inclusive.

##### aps_inner_id and aps_id pair identifies the adaptation parameter set that is referred to in the slice header if the value of aps_fragmentation_flag is 1. The value of aps_inner_id shall be in the range of 0 to TBD, inclusive.

##### 7.4.1.2.1   Order of sequence, picture and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of picture, sequence and adaptation parameter sets for coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found.

- NOTE 1 – The sequence, picture and adaptation parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence, picture and adaptation parameter sets may, in some applications, be conveyed "out-of-band".

- An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one adaptation parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular adaptation parameter set RBSP results in the deactivation of the previously-active adaptation parameter set RBSP (if any).

- When an adaptation parameter set RBSP (with a particular value of aps_id) is not active and it is referred to by a coded slice NAL unit (using that value of aps_id), it is activated. This adaptation parameter set RBSP is called the active adaptation parameter set RBSP until it is deactivated by the activation of another adaptation parameter set RBSP. An adaptation parameter set RBSP, with that particular value of aps_id, shall be available to the decoding process prior to its activation.

- If the value of aps_fragmentation_flag is equal to 1, aps_id and aps_inner_id pair is used in the activation of the adaptation parameter set RBSP. When an adaptation parameter set RBSP with a particular value of aps_id and aps_inner_id pair is not active and it is referred to by a coded slice NAL unit using those values of aps_id and aps_inner_id, all adaptation parameter set RBSP syntax structures having that aps_id and up to and including that aps_inner_id are activated. Those adaptation parameter set RBSPs are called the active adaptation parameter set RBSPs until they are deactivated by the activation of another adaptation parameter set RBSP having a different aps_id. Adaptation parameter set RBSPs, with those particular values of aps_id and aps_inner_id, shall be available to the decoding process prior to their activation.