| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0376 |
| --- | --- |

| Title: | Non-CE8: Low-delay support for APS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Virginie Drugeon, Thomas Wedi Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 – 1306 semih.esenlik@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

For the 8th JCT-VC Meeting at San Jose, several proposals related to LCU-based Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) are submitted. LCU-based syntax for ALF and SAO provides the possibility of low encoding delay for the two in-loop filters. This contribution proposes a mechanism to allow usage of APS in low delay scenarios in connection with LCU-based ALF and SAO approaches. The proposal uses the merits of the APS in order to carry the filtering information that is shared by multiple slices in a frame. Moreover a fragmentation scheme is proposed that meets the low-delay application requirements for ALF and SAO. As opposed to signalling all of the filtering parameters inside the slice, the proposed low-delay APS fragmentation scheme does not suffer from reduction in the ALF and SAO coding gain in the case where a frame is partitioned into multiple slices.

# Introduction

ALF and SAO are tools that provide high coding gains and that increase the subjective quality. However in HM5.0 the parameter estimation process is frame-based, which causes the following problems:

- Increased external memory access in the encoder.

- Frame-level delay in the encoding.

In order to address the problems LCU-based ALF and SAO proposals have been submitted to the JCT-VC group. The fundamentals of the LCU-based filter estimation approach can be summarized in two bullet items:

- The filtering coefficients are estimated and applied on the fly: After an LCU is encoded filter parameters are generated and applied to the LCU without any additional delay.

- Spatial prediction of filtering parameters: In order to reduce the overhead previously designed filter parameters are stored in tables and reused on the following LCUs.

The fundamentals of the LCU-based ALF and SAO are depicted in Figure 1 by an example.

Figure 1: LCU-based SAO and ALF

In the example the frame is partitioned into 12 LCUs. In the first LCU a filter (Filter 1) is generated and used to filter the first LCU. Then the filter is stored to the table of filters. The following 3 LCUs reuse Filter 1, which is a form of spatial prediction which helps reducing the signaling overhead.

In the 5th LCU a new filter is generated which is Filter 2. Filter 2 is appended in the table of filters. Starting from the 6th LCU, the LCUs in the frame are now allowed to reuse one of the two filters in the table of filters.

Finally in the 11th LCU a new filter is generated (Filter 3) and appended to the table of filters. The last LCU in the frame is allowed to reuse 3 filters from the table of filters.

In LCU-based SAO and ALF approach the spatial prediction is used heavily in order to reduce the signaling overhead.

# Problem

The current Adaptation Parameter Set (APS) design in HM5.0 does not allow slice-level delay for the ALF and SAO, since all of the slices in a frame have to refer to the same APS. Therefore it was observed that all of the contributions related to LCU-based ALF and SAO choose to signal the filter parameters in the slice header or within the LCU data. Moreover some contributions advocate that the filter parameters should be signalled inside the slice in the low-delay mode and in the APS if there are no low-delay requirements.

If the filter parameters are signalled in the slice in order to meet low-delay requirements coding loss is expected since the filter parameters that are signalled in one slice cannot be reused by the neighboring slice (self-contained slices concept). The problem is depicted in Figure 2 for the previous example in the case of partitioning of the frame in multiple slices.

Figure 2: LCU-based ALF and SAO in the case of partitioning of a frame into multiple slices.

In the figure above the frame is partitioned into two slices. In the low delay mode the filter parameters are going to be signaled inside the slice. As a result filters 1 and 2 that are generated in slice 1 cannot be reused in slice 2.

In low-delay applications if the filter parameters are signaled in the slice, the table of filters needs to be reset at the beginning of each slice. If a frame is partitioned into a lot of slices then the coding gains of SAO and ALF are expected to drop significantly.

# Proposal

## Moving shared filtering information to APS

The motivation for the introduction of APS is to carry adaptive information that is shared among multiple slices. Therefore first of the filter coefficients that are shared among the LCUs is moved to the APS.

Please note that in LCU-based ALF and SAO approaches the filter parameter set contains two types of information:

- Filter coefficients that are reused by different multiple LCUs. These include actual filter coefficients and offset values.

- LCU-specific auxiliary information. These include information that is not shared among the LCUs such as filtering on/off flags, id of the filter that is used for the LCU, flag that signals whether a new filter is generated for the LCU or a previous filter is reused etc.

According to the proposal only the information that is shared among LCUs is moved to the APS. The LCU-specific information is kept inside the slice. As a result the size of the APS is not increased unnecessarily.

## Fragmentation of the APS according to low-delay requirements

The LCU-based ALF and SAO approaches that are advocated by the proposals [1], , [3] and [4] can achieve low-delay encoding. Please note that frame-based ALF and SAO approaches create a frame level delay since SAO and ALF parameters are estimated over the whole frame.

In LCU-based approaches each LCU has the option to generate a new ALF or SAO parameter where the parameters are estimated over the currently encoded LCU. If a new SAO/ALF parameter is not generated, then the LCU has an option to reuse one of the parameters that are generated before according to the encoding order of the LCUs. Therefore LCU-based ALF and SAO approaches introduce a parameter prediction mechanism which allows re-use of filter parameters that are generated in a previous LCU.

In the fragmentation of the APS this prediction mechanism of LCU-based ALF and SAO, as well as the low-delay requirements are taken into account. For the purpose of fragmentation a new syntax element is introduced in the APS which is called APS_inner_id that is used to label individual fragments. The following figure presents the proposed APS structure.

Figure 3: Proposed APS structure

According to the proposed APS structure that is presented in Figure 3, the parameter sets corresponding to SAO and ALF are fragmented into different APS fragmentation units. The Q-Matrix parameters and Deblocking Filter parameters are not fragmented and are signalled in the first APS fragment. The first APS fragment is specified by the value of the APS_inner_id syntax element being equal to 0.

The Figure 5 below describes the referencing structure between slices and APS fragments. The referencing structure between the APS fragments follows the same prediction mechanism as in the case of ALF and SAO parameter set generation. As an example the second slice of the frame (Slice 1) in the figure refers to the second APS fragment which is represented by the (APS_id = 1, APS_inner_id = 1) pair. The second APS fragment then inherits the parameter sets that are signalled in the first APS fragment (which is referred by the first slice of the frame). As a result second slice in the frame is allowed to re-use the ALF and SAO parameters that were generated during the encoding of the first slice of the frame.

Figure 4: Referencing structure

## Merging of the fragmentation units in the decoder

The following figure describes the merging of the APS fragments from the perspective of the decoder. In the figure parameter tables that are used by slices are generated according to the referencing structure explained in the previous section. The parameters that are signalled in the APS fragments are appended at the bottom of the “Parameter table of the slice” according to the order which is given by the APS_inner_id.

Figure 5: Merging of the fragments and generation of parameter tables for slices

Please note that the ALF and SAO parameters are populated in the “Parameter table of the slice” according to the order they are generated in the encoder. Moreover, now the filter parameters that are generated for SAO and ALF during the encoding of a slice can be reused by the following slice (in the decoding order).  Therefore the coding loss that would stem from sending filter parameters inside the slice (and prohibiting re-use of filter coefficients between slices) totally recovered.

# Summary

The LCU-based ALF and SAO approaches that are advocated by many contributions provide low-delay support for the two filtering tools and would allow widespread adoption of the tools. However signalling of the filter parameters inside the slice would deteriorate the performance of such tools, since the reuse of filter parameters would not be possible between the slices. This means that overhead associated with signalling of filter parameters would render ALF and SAO practically useless, especially if the number of LCUs within a slice becomes small.

In the current proposal the filter parameters that are shared by multiple slices are moved to the APS, which is in line with the general purpose of the APS. The LCU-specific auxiliary SAO and ALF information are left in the slice data in order not to increase the size of the APS. Then the APS is fragmented according to match the transmission schedule (one APS fragment per frame as an example). Finally referencing mechanism is introduced for population of APS fragments.

As a result the coding loss of ALF and SAO in the case of partitioning of a frame into multiple slices is totally eliminated. The following three simple examples of low-delay application scenarios show the benefit of the APS fragmentation method more concretely.

- If the packet loss probability is low in a particular low-delay application scenario, fragmented APS allows coding gain for ALF and SAO as if a frame is not partitioned into slices. In other words partitioning the frame into slices does not reduce the gain of ALF and SAO as opposed to the case where filtering coefficients are signalled inside the slice.

- If the packet loss probability is high, unequal protection can be assigned to different APS fragments.

- If the packet loss probability is very high, then the APS fragments and slice data can be combined in a single packet. Moreover the ALF and SAO filter re-use can be avoided in the encoder which would switch off the prediction between APS fragments. Please note that this solution is almost the same as sending all filter parameters inside the slice.

Moreover in non-low-delay scenarios the APS does not need to be fragmented, all filter parameters pertaining to the slices of a frame can be transmitted inside a single APS fragment where multiple slices referring to the same APS fragment.

The tables Table 1, Table 2 and Table 3 present the proposed syntax for the SPS, APS and slice header. Please note that the independent parsing of each APS fragment should be guaranteed by mechanisms such as signalling the number of ALF filter coefficients that is present inside an APS fragment. Moreover since the filtering information that is shared between multiple slices is separated from LCU-specific auxiliary information it might be necessary (depending on the particular LCU-based SAO or ALF proposal) to map the filter parameters that are signalled inside the APS to the individual slice. For this purpose in Table 3 a syntax element is introduced in the slice header which is called pointer_to_parameter_in_APS. Such a parameter can be used to signal for instance the id of the first ALF filter that is generated inside the slice.

# Conclusion

The contribution proposes a mechanism to allow usage of APS in low delay scenarios in connection with LCU-based ALF and SAO approach. The proposal uses the merits of the APS in order to carry the filtering information that is shared by multiple slices in a frame. Moreover, a fragmentation scheme is proposed that meets the low-delay application requirements for ALF and SAO. As opposed to signalling all of the filtering parameters inside the slice, the proposed low-delay APS fragmentation scheme does not suffer from reduction in the ALF and SAO coding gain in the case where a frame is partitioned into multiple slices. It is proposed to include the proposed low-delay APS fragmentation scheme in the HEVC standard.

# References

- C.-Y. Chen et al., “CE8.a.1: One-stage SAO and ALF with LCU-based syntax”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0272, San Jose, February 2012.

- C.-Y. Chen et al., “CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntax”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0274, San Jose, February 2012.

- A. Fuldseth et al., “CE8: ALF with low latency and reduced complexity”, Document of Joint Collaborative Team on Video Coding, JCTVC-G498, Geneva, November 2011.

- C.-M. Fu et al., “Sample Adaptive Offset with LCU-based Syntax”, Document of Joint Collaborative Team on Video Coding, JCTVC-F056, Torino, July, 2011

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Syntax Changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if (adaptive_loop_filter_enabled_flag ¦¦ sample_adaptive_offset_enabled_flag){ |  |
| aps_fragmentation_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 1: Proposed SPS syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_inner_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if ( aps_inner_id == 0){ |  |
| aps_deblocking_filter_flag | u(1) |
| } |  |
| if( aps_sample_adapt ive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count | u(8) |
| byte_align( ) |  |
| sao_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count | u(8) |
| byte_align( ) |  |
| alf_param( ) |  |
| } |  |
| if ( aps_inner_id == 0){ |  |
| if (aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2: Proposed APS syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag) |  |
| slice_sample_adaptive_offset_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag) |  |
| slice_adaptive_loop_filter _flag | u(1) |
| aps_id | ue(v) |
| if (slice_adaptive_loop_filter _flag ¦¦ slice_sample_adaptive_offset_flag){ |  |
| if (aps_fragmentation_flag) |  |
| aps_inner_id | ue(v) |
| if (LCU specific ALF or SAO information present in the slice) |  |
| pointer_to_parameter_in_APS //mapping of filter parameters from APS to slice |  |
| } |  |
| … |  |
| … |  |

Table 3: Proposed slice header syntax