| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0379 |
| --- | --- |

| Title: | Context reduction of 8x8 significance map coding by hybrid scan with mask |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu, Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In HM5.0, 22 contexts were assigned to encode the significant_coeff_flag of 8x8 transform coefficients. This contribution removed the 22 contexts of 8x8 transform coefficients in HM5.0 by reusing the mask based context index selection method for 16x16 and 32x32 transform coefficients as originally proposed in JCTVC-D185. To facilitate parallel computation of significant_coeff_flag, this contribution replaced the 8x8 horizontal scan by horizontal-diagonal scan and the 8x8 vertical scan by vertical-diagonal scan. The BD-rate for AI_HE, RA_HE, LB_HE are -0.04%, -0.10%, -0.10%. The BD-rate for AI_LC, RA_LC, LB_LC are -0.21%, -0.16%,  -0.22%. The BD-rate for RA_HE10 is 0.03% .

# Introduction

This contribution removed the 22 contexts of the coding of 8x8 significant_coeff_flag in HM5.0 by using mask based coding method as originally proposed in JCTVC-D185[1]. In particular, for the encoding of significant_coeff_flag, HM5 adopted JCTVC-G1015 [5] which combined the followings:

- 4x4 position based coding method from JCTVC-G657 [2].

- 8x8 position based coding method from JCTVC-G366 [3].

- 16x16/32x32 mask based coding method from JCTVC-G768 [4].

This contribution replaced the 8x8 position based coding method in HM5.0 [2] and expanded the coverage of the mask based coding method in HM5.0 [4] from 16x16 and 32x32 to include the coding of the 8x8 signficant_coeff_flag.

As shown in Figure 1, the coding of 16x16 and 32x32 significant_coeff_flag in HM5.0 is based on 4x4 sub-block diagonal scan. In general, it supports the parallel determination of two significant_coeff_flag by using the green and the orange mask shape in Figure 1 for context index calculation. In particular, the orange mask shape is the green mask shape with a hole in it to support parallel processing at the orange positions. There are no horizontal and vertical scan for the coding of 16x16 and 32x32 significant_coeff_flag.

When the coverage of the mask based coding method is expanded to include 8x8, the coding method should maintain the property that at least two contexts can be determined in parallel as in HM5. As show in Figure 2, HM5.0 uses the 8x8 diagonal, horizontal, and vertical scans to encode the 8x8 transform coefficients. For the 8x8 diagonal scan, only the green mask is needed. For the vertical scan, the orange mask could be used for all positions to support parallel processing. For the horizontal scan, a new mask with hole for all position is needed to support parallel processing. However, this approach resulted in coding losses. Harmonization of scans and mask shape is needed to maintain and to improve coding efficiency.

4x4 sub-block scan

mask for green positions

mask for orange positions

Figure 1: To support parallel processing, the 4x4 sub-block diagonal scan in HM5 has the green and orange mask shapes. The orange mask shape is the green mask shape but with a hole in it.

diagonal scan

horizontal scan

vertical scan

Figure 2: The diagonal, horizontal, and vertical scan for coding of 8x8 significance map in HM5.0.

# Parallel friendly scans and mask shapes for 8x8

To support processing and to improve coding efficiency, the 8x8 horizontal scan and the 8x8 vertical scan in Figure 2 are placed by the horizontal-diagonal and the vertical-diagonal scan and masks respectively in Figure 3.

diagonal scan

horizontal-diagonal scan

vertical-diagonal scan

mask for blue positions

mask for orange positions

mask for green positions

Figure 3: Proposed parallel friendly scans and masks in 8x8.

# Simulation results

The context reductions were integrated into HM5.0. The simulations were performed in three Microsoft HPC clusters, the common test conditions and reference configurations are followed:

- All intra simulations are performed on AMD Opteron Processor 6136 cluster @ 2.4GHz.

- All RA simulations are performed on Intel Xeon X5690 cluster @ 3.47GHz.

- All LD simulations are performed on Intel Xeon X5680 cluster @ 3.33GHz.

Error: Reference source not found shows the BD-rage of the proposed 22 context reduction for the coding of significance map.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.04% | 0.14% | 0.09% | -0.14% | -0.04% | -0.02% |  |  |  |
| Class B | 0.01% | 0.23% | 0.15% | -0.18% | -0.07% | -0.04% |  |  |  |
| Class C | -0.08% | 0.09% | 0.03% | -0.22% | -0.15% | -0.16% |  |  |  |
| Class D | -0.17% | 0.08% | -0.05% | -0.28% | -0.21% | -0.24% |  |  |  |
| Class E | 0.02% | 0.10% | 0.14% | -0.20% | 0.08% | 0.05% |  |  |  |
| Overall | -0.04% | 0.13% | 0.07% | -0.21% | -0.09% | -0.09% |  |  |  |
|  | -0.04% | 0.13% | 0.09% | -0.22% | -0.09% | -0.09% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.06% | 0.10% | -0.05% | -0.17% | 0.18% | -0.15% | 0.04% | 0.63% | 1.06% |
| Class B | -0.02% | 0.29% | 0.28% | -0.08% | 0.10% | 0.20% | 0.01% | 0.31% | 0.22% |
| Class C | -0.16% | 0.07% | 0.16% | -0.21% | 0.04% | -0.17% |  |  |  |
| Class D | -0.14% | -0.09% | 0.54% | -0.21% | 0.48% | 0.11% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | 0.11% | 0.27% | -0.16% | 0.19% | 0.03% | 0.03% | 0.45% | 0.59% |
|  | -0.10% | 0.10% | 0.28% | -0.16% | 0.21% | -0.01% | 0.03% | 0.47% | 0.64% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.38% | 0.38% | -0.13% | -0.36% | 0.09% |  |  |  |
| Class C | -0.15% | 0.28% | 0.29% | -0.30% | -0.17% | -0.19% |  |  |  |
| Class D | -0.14% | 0.25% | 0.70% | -0.33% | 0.65% | -0.25% |  |  |  |
| Class E | -0.07% | 0.39% | -0.24% | -0.09% | -1.43% | -0.48% |  |  |  |
| Overall | -0.10% | 0.33% | 0.32% | -0.22% | -0.26% | -0.17% |  |  |  |
|  | -0.10% | 0.36% | 0.31% | -0.21% | -0.24% | -0.22% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 1: BD-rate of 22 context reductions.

# Summary

As shown in Table 2, this contribution reduced 22 contexts for the coding of significance map and resulted in average luminance BD-rate of -0.04% to -0.21% for all test cases except RA_HE-10 with BD-rate of 0.03%.

|  | HE | LC | HE-10 |
| --- | --- | --- | --- |
| I | -0.04% | -0.21% |  |
| RA | -0.10% | -0.16% | 0.03% |
| LB | -0.10% | -0.22% |  |

Table 2: Average BDR of proposed 22 significance map context reduction.

# References

- Hisao Sasai and Takahiro Nishi, “Simplified Context modeling for Transform Coefficient Coding”, JCTVC-D185, 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Gergely Korodi, Jinwen Zan, Da-ke He, Encoding and decoding significant coefficient flags for small Transform Units using partition sets”, JCTVC-G657, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Cheung Auyeung and Jun Xu, “Description Context reduction of significance map coding with CABAC”, JCTVC-G366, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Yinji Piao, Junghye Min, and JeongHoon Park, “Reduced contexts for significance map coding of large transform in CABAC”, JCTVC-G768, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Cheung Auyeung, Jun Xu, Gergely Korodi, Jinwen Zan, Da-ke He, Yinji Piao, Elena Alshina, Junghye Min, JeongHoon Park, “A combined proposal from JCTVC-G366, JCTVC-G657, and JCTVC-G768 on context reduction of significance map coding with CABAC”, JCTVC-G1015, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix:   Proposed WD text based on JCTVC-G1103_d6

6.6 Scanning order array initialisation process

Input to this process is a block widthsize blkWidth and a block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies the up-right diagonal scan as specified in subclause Error: Reference source not found with blkWidth and blkHeight as inputs. If blkWidth not equal to 8 or blkHeight not equal 8,  then scanIdx equal to 1 specifies a horizontal scan, and scanIdx equal to 2 specifies a vertical scan. Otherwise the blkWidth is equal to 8 and blkHeight  is equal to 8,  then scanIdx equal to 1 specifies a horizontal-diagonal scan, and scanIdx equal to 2 specifies a vertical-diagonal scan.

The array index pos specifies the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = DiagScan

if ( (blkHeight !=  8) || (blkWidth != 8)) {

i  =  0
y  =  0
while( y  < blkHeight ) {
	x  =  0
	while( x  <  blkWidth ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
}

i  =  0
x  =  0
while( x  < blkWidth ) {
	y  =  0
	while( y  <  blkHeight ) {
		ScanOrder[ 2 ][ i ][ 0 ]  = x
		ScanOrder[ 2 ][ i ][ 1 ]  = y
		y++
		i++
	}
	x++
}

}

else {
              y_off  =  0
              i  =  0
             while( y_off < 8) {

ScanOrder[1][i][0]  =  0

ScanOrder[1][i][1]  =  y_off

ScanOrder[2][i][0]  =  y_off

ScanOrder[2][i][1]  =  0

i++

x  =  1
	while( x<8) {

ScanOrder[1][i][0]  =  x

ScanOrder[1][i][1]  =  y_off

ScanOrder[2][i][0]  =  y_off

ScanOrder[2][i][1]  =  x

i++

ScanOrder[1][i][0]  =  x − 1

ScanOrder[1][i][1]  =  y_off  + 1

ScanOrder[2][i][0]  =  y_off  + 1

ScanOrder[2][i][1]  =  x − 1

i++

x++

}

ScanOrder[1][i][0]  =  7

ScanOrder[1][i][1]  =  y_off  + 1

ScanOrder[2][i][0]  =  y_off  + 1

ScanOrder[2][i][1]  =  7

y_off += 2

i++

}

}

9.2.3.1.1.5		Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in  as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Table 9-4 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, and either

  - scanIdx != 1

or when all of the following conditions are true,

  - xC is not equal to 0 or yC % 2 is equal to 1

  - xC is not equal to 6 or yC % 2 is equal to 0

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, and either

  - scanIdx is equal to 1

or either all of the following conditions are true,

  - scanIdx is equal to 0

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2

or either all of the following conditions are true,

  - scanIdx is equal to 2

  - yC is not equal to 0 or xC % 2 is equal to 1

  - yC is not equal to 6 or xC % 2 is equal to 0

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-12)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-14)

Table 9-3 – Specifcation of ctxIdxMap4x4[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap4x4[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |
| i | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| ctxIdxMap4x4[ i ] | 0 | 1 | 2 | 4 | 1 | 1 | 2 | 4 | 3 | 3 | 5 | 5 | 4 | 4 | 5 |

Table 9-4 – Specifcation of ctxIdxMap8x8[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap8x8[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 8 | 6 | 6 | 7 | 9 | 9 | 7 | 7 |

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

9.2.1.1		Initialisation process for context variables

Table 9-5 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialisation variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 |  |  |  |  |  |  |  |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue |  |  |  |  | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 149 | 70 | 52 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 118 |  |  |  |  |  |  |  |  |  |  |  | 74 | 115 | 87 | 89 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 118 |  |  |  |  |  |  |  |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue |  |  |  |  | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 70 | 53 | 67 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 117 |  |  |  |  |  |  |  |  |  |  |  | 168 | 131 | 150 | 120 |
|  | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 |  |  |  |  |  |  |  |
|  | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initValue |  |  |  |  | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 86 | 84 | 67 |
|  | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| initValue | 117 |  |  |  |  |  |  |  |  |  |  |  | 152 | 131 | 150 | 120 |