| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0380 |
| --- | --- |

| Title: | AHG15: SPS and PPS syntax modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Thomas Wedi Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen Germany Viktor Wahadaniah Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +49 6103 766 240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes several modifications of the SPS and PPS syntax. It is claimed that the proposed modifications result in a harmonised and cleaner syntax for the mentioned parameter sets. In particular, modifications are proposed for:

- block size signaling

- PCM related syntax

- slice granularity

# Block size signaling

In HEVC working draft 5 [1], the current SPS syntax signals the following block size information:

- Minimum (or smallest) coding unit size (MinCU or SCU)

- Maximum (or largest) coding unit size as a difference to the minimum coding unit size (MaxCU or LCU)

- Minimum transform block size (MinTr)

- Maximum transform block size as a difference to the minimum transform block size (MaxTr)

- Minimum PCM coding unit size (MinPCMCU)

During the 7th JCTVC meeting in Geneva, it was decided to introduce an additional parameter in the syntax (see section 5.13.1 of the meeting report [2]): maximum PCM coding unit size (MaxPCMCU).

The block sizes signaled in the SPS are dependent from each other. However, this dependency is currently not fully exploited. Thus, some block size combinations which can be signalled, are allowed, others would lead to an invalid bitstream. For example, the minimum transform size has to be smaller than the minimum coding unit size.

An analysis of the block size combinations in HEVC leads to the following relationships between the above mentioned block sizes:

MinTr≤MinCU≤MaxCUMinTr≤MaxTr≤MaxCU

If the PCM mode is enabled:     MinCU≤MinPCMCU≤MaxPCMCU≤MaxCU

It is noticed that the maximum coding unit size is always the biggest block size and the minimum transform size always the smallest.

Furthermore, each of the above mentioned block sizes have a limited range of allowed values (the red values are the ones used in the current common test conditions):

| Block size | Currently allowed values (WD5) |
| --- | --- |
| MinCU | 8x8 / 16x16 / 32x32 / 64x64 / 128x128? |
| MaxCU | 8x8? / 16x16 / 32x32 / 64x64 /128x128? |
| MinTr | 4x4 / 8x8 / 16x16 / 32x32 |
| MaxTr | 4x4 / 8x8 / 16x16 / 32x32 |
| MinPCMCU | 8x8? / 16x16 / 32x32 / 64x64 /128x128? |
| MaxPCMCU | 8x8? / 16x16 / 32x32 / 64x64 /128x128? |

However, the current HM5.0 encoder does not accept a maximum coding unit size of 128x128 or 8x8. Therefore, in order to harmonise the software and the working draft, it is proposed to restrict the size of the maximum coding unit: 16×16≤MaxCU≤64×64

In order to exploit the dependencies, it is proposed to signal the maximum coding unit size first in the SPS and signal all other above mentioned block sizes with respect to it. The following block size signaling is therefore proposed:

- Maximum coding unit size (MaxCU or LCU)

- Mimimum coding unit size as a difference to the maximum coding unit size (MinCU or SCU)

- Maximum transform size as a difference to the maximum coding unit size (MaxTr)

- Minimum transform size as a difference to the maximum transform size (MinTr)

- Maximum PCM coding unit size as a difference to the maximum coding unit size (maxPCMCU)

- Minimum PCM coding unit size as a difference to the maximum PCM coding unit size (MinPCMCU)

# PCM related syntax

The following syntax elements in SPS control the use of the PCM coding mode for a sequence:

- PCM bit depth for luma samples

- PCM bit depth for chroma samples

- Minimum PCM coding block size

- Maximum PCM coding block size

The PCM coding mode is currently disabled for a whole sequence by defining a size of the minimum PCM coding unit that is bigger than the maximum coding unit. However, since the maximum PCM coding unit size was introduced, two sizes bigger than the maximum coding unit have to be signaled now. This signalisation is rather confusing and redundant. Therefore, it is proposed to introduce a flag to enable/disable PCM and make the presence of the above mentioned PCM syntax elements dependent on the value of this flag.

# Slice granularity

The syntax element slice_granularity in PPS is used to signal the granularity of the slices. When slice_granularity is equal to 0, a slice can only end at the LCU borders. It is noticed that the possible range of values of this syntax element is strongly dependent on the selected minimum and maximum coding unit sizes:

| MinCU | MaxCU | Possible values of slice_granularity |  |  |
| --- | --- | --- | --- | --- |
| 8x8 | 16x16 | 0 (16x16) |  |  |
| 8x8 | 32x32 | 0 (32x32) | 1 (16x16) |  |
| 8x8 | 64x64 | 0 (64x64) | 1 (32x32) | 2 (16x16) |
| 16x16 | 16x16 | 0 (16x16) |  |  |
| 16x16 | 32x32 | 0 (32x32) | 1 (16x16) |  |
| 16x16 | 64x64 | 0 (64x64) | 1 (32x32) | 2 (16x16) |
| 32x32 | 32x32 | 0 (32x32) |  |  |
| 32x32 | 64x64 | 0 (64x64) | 1 (32x32) |  |
| 64x64 | 64x64 | 0 (64x64) |  |  |

It is noticed that the slice granularity always has to be smaller than or equal to the maximum coding unit size. However, the slice granularity was restricted to be always bigger than 16x16. Therefore, there is a design problem in the current working draft if the maximum coding unit size is not restricted to be bigger than 16x16 (see proposal in section 1 of this document).

Furthermore, it is noticed that the value 0 is the only possible value for several configurations. The value 0 which indicates that the slice granularity is equal to the maximum coding unit size is likely to be used more often than other values.

The syntax element slice_granularity is currently coded with a fixed length code using 2 bits. As the value 0 is likely to be used more often, it is proposed to exploit this in the coding of this syntax element by the use of a variable length code. There are two possibilities:

- Move this syntax element from the PPS to the SPS and use truncated exp-Golomb codes te(v) to signal it depending on the values of the selected maximum and minimum coding unit sizes.

- Keep this syntax element in the PPS, but use exp-Golomb codes ue(v) to signal it.

# New proposed SPS syntax

The following table is based on the table from section 7.3.2.1 of JCTVC-G1103_d6.doc [1]. The modifications are marked in red.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| max_num_ref_frames | ue(v) |
| log2_max_coding_block_size_minus4 | ue(v) |
| log2_diff_min_max_coding_block_size | ue(v) |
| log2_diff_max_transform_block_coding_block_size | ue(v) |
| log2_diff_min_max_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| pcm_enabled_flag | u(1) |
| if ( pcm_enabled_flag ) { |  |
| log2_diff_max_pcm_coding_block_size | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| if ( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| if ( log2_max_coding_block_size_minus4 != 0 && log2_diff_min_max_coding_block_size != 0 ) |  |
| slice_granularity | te(v) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for ( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# New proposed SPS semantics

log2_max_coding_block_size_minus4 specifies the maximum size of a coding block.

The variable Log2MaxCUSize is set equal to log2_max_coding_block_size_minus4 + 4.

The bitstream shall not contain data that result in Log2MaxCUSize greater than 6.

log2_diff_min_max_coding_block_size specifies the difference between the minimum and maximum coding block size.

The variable Log2MinCUSize is set equal to log2_max_coding_block_size_minus4 + 4 − log2_diff_min_max_coding_block_size.

The bitstream shall not contain data that result in Log2MinCUSize smaller than 3.

log2_diff_max_transform_block_coding_block_size specifies the difference between the maximum size of a transform block and the maximum size of a coding block.

The variable Log2MaxTrafoSize is set equal to log2_max_coding_block_size_minus4 + 4 − log2_diff_max_transform_block_coding_block_size.

The bitstream shall not contain data that result in Log2MaxTrafoSize greater than 5 or smaller than 2.

log2_diff_min_max_transform_block_size specifies the difference between the minimum and maximum transform size.

The variable Log2MinTrafoSize is set equal to log2_max_coding_block_size_minus4 + 4 − log2_diff_max_transform_block_coding_block_size − log2_diff_min_max_transform_block_size.

The bitstream shall not contain data that result in Log2MinTrafoSize greater than Log2MinCUSize.

The bitstream shall not contain data that result in Log2MinTrafoSize greater than 5 or smaller than 2.

pcm_enabled_flag equal to 1 specifies that the I_PCM coding method may be used for coding.

log2_diff_max_pcm_coding_block_size specifies the difference between the maximum size of I_PCM coding blocks and the maximum size of a coding block.

The variable Log2MaxIPCMCUSize is set equal to log2_max_coding_block_size_minus4 + 4 − log2_diff_max_pcm_coding_block_size.

The bitstream shall not contain data that result in Log2MaxIPCMCUSize greater than 5.

log2_diff_max_min_pcm_coding_block_size specifies the difference between the minimum and maximum size of I_PCM coding blocks.

The variable Log2MinIPCMCUSize is set equal to log2_max_coding_block_size_minus4 + 4 − log2_diff_max_pcm_coding_block_size − log2_diff_max_min_pcm_coding_block_size.

# Conclusion

Several modifications of the current SPS and PPS syntax are proposed in this contribution. The modifications are proposed to harmonise, improve and clean up the current syntax.

It is proposed to discuss the allowed range of the different syntax elements addressed in this contribution, in particular the range of the block sizes, and modify the coding of these syntax elements as proposed.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC document JCTVC-G1103_d6, 7th meeting, Geneva, CH, November 2011.

- G. J. Sullivan, J.-R. Ohm, “Meeting report of the seventh meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 21–30 Nov. 2011”, JCT-VC document JCTVC-G_Notes_dA, 7th meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).