| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0381 |
| --- | --- |

| Title: | AHG15: Partial APS update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Semih Esenlik Thomas Wedi Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen Germany Viktor Wahadaniah Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +49 6103 766 240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

During the 7th JCTVC meeting in Geneva, the issue of “partial update” of APS was addressed. There were two approaches discussed for “partial update”: send more than one APS index in the slice header or use a copying process from already transmitted APS. Both of the proposed approaches have drawbacks. This contribution proposes to improve the already discussed techniques. It is proposed to introduce “copy” flags in the APS and explicit references to other already transmitted APS.

# Introduction

During the 7th JCTVC meeting in Geneva, “partial update” was defined as the possibility to change only parts of the information carried in APS without transmission of a full APS carrying both the changed and unchanged information [1]. There were two approaches discussed for “partial update”: send more than one APS index in the slice header [2], [3], and [4] or use a copying process from already transmitted APS [5].

The first approach proposed to reference several APS in the slice header. However, it was mentioned that having several APS indexes in the slice header may not be desirable. Furthermore, rules must be defined to handle the case when several APS contains different parameters for the same tool.

The second approach used a partial update of the APS by conditional replacement. In this proposal, some information contained in already transmitted APS is copied to other APS indexes using parameter set maintenance NAL units (Figure 1). However, this approach using maintenance NAL units is not robust against packet losses. Figure 2 shows an example where a maintenance NAL unit is lost.

Figure 1: partial update of APS using maintenance NAL units

Figure 2: Problem if a maintenance NAL unit is lost

As can be seen in Figure 2, the loss of a maintenance NAL unit leads to an error in the parameter sets that is propagated.

# Proposed solution for partial APS update

The proposed solution is similar to the second approach. However, the idea is to have “self-contained” APS where all the information necessary to fill up the memory location is contained in the APS NAL unit itself. This saves the bits necessary to transmit a maintenance NAL unit.

In the proposed approach, “copy” flags are added to the APS for each tool. A “copy” flag indicates if the parameters for one given tool are explicitly transmitted or copied from another already transmitted APS. If the parameters are copied, an APS index is signaled that refers to an APS where the parameters are explicitly transmitted (Figure 3).

For each tool, a different syntax and semantic is used:

- For the quantization matrices, only one “present” flag is signaled. If the “present” flag is equal to 1, the parameters for quantization matrices are explicitly signaled. If the “present” flag is zero, the parameters are copied from another APS that is signaled.

- For SAO and ALF, a “present” flag and a “copy” flag are signaled. If the “present” flag is equal to 0, the tools are not used and the “copy” flag is not present. If the “present” flag is equal to 1, the “copy” flag is present. If the “copy” flag is equal to 0, the parameters for SAO or ALF are explicitly signaled. If the “copy” flag is equal to 1, the parameters are copied from another APS that is signaled.

- For the deblocking filter, a “present” flag and a “copy” flag are signaled. If the “present” flag is equal to 0, the default parameters for the deblocking filter are used and the “copy” flag is not present. If the “present” flag is equal to 1, the “copy” flag is present. If the “copy” flag is equal to 0, the parameters are explicitly signaled. If the “copy” flag is equal to 1, the parameters are copied from another APS that is signaled.

Figure 3: Proposed partial update of APS

# Proposed APS syntax

The modifications introduced are marked in red.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| aps_deblocking_filter_present_flag | u(1) |
| if ( aps_deblocking_filter_present_flag ) |  |
| aps_deblocking_filter_copy_flag | u(1) |
| aps_sample_adaptive_offset_present_flag | u(1) |
| if ( aps_sample_adaptive_offset_present_flag ) |  |
| aps_sample_adaptive_offset_copy_flag | u(1) |
| aps_adaptive_loop_filter_present_flag | u(1) |
| if ( aps_adaptive_loop_filter_present_flag ) |  |
| aps_adaptive_loop_filter_copy_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| else |  |
| aps_id_QM | ue(v) |
| if( aps_deblocking_filter_present_flag && !aps_deblocking_filter_copy_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if (!disable_deblocking_filter_flag) { |  |
| beta_ offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| else if(aps_deblocking_filter_present_flag && aps_deblocking_filter_copy_flag ) |  |
| aps_id_deb | ue(v) |
| if( aps_sample_adaptive_offset_present_flag && !aps_sample_adaptive_offset_copy_flag ) |  |
| sao_param( ) |  |
| else if(aps_sample_adaptive_offset_present_flag && aps_sample_adaptive_offset_copy_flag) |  |
| aps_id_sao | ue(v) |
| if( aps_adaptive_loop_filter_present_flag && !aps_adaptive_loop_filter_copy_flag ) |  |
| alf_param( ) |  |
| else if(aps_adaptive_loop_filter_present_flag && aps_adaptive_loop_filter_copy_flag) |  |
| aps_id_alf | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

In order to reduce error propagations in case of an APS packet loss, it is further proposed to include the following restrictions on the signaled APS indexes:

- The APS referred to shall not copy the parameters from another APS. This restriction is introduced to avoid long chains of references that may reduce the error robustness of the proposed scheme. For example, aps_id_sao shall not refer to an APS where aps_sample_adaptive_offset_copy_flag is equal to 1.

- An APS shall not refer to itself.

# Conclusion

This contribution proposes a new alternative for partial update of APS, which further reduces the bitrate and is more robust against packet losses compared to the solution using an additional NAL unit type, while keeping the advantages of the APS partial update.

It is proposed to adopt this scheme for APS partial update.

# References

- S. Wenger, “JCT-VC break-out report: Adaptation Parameter Set issues”, JCT-VC document JCTVC-G1016, 7th meeting, Geneva, CH, November 2011.

- J. Tanaka, Y. Morigami, T. Suzuki, “Non-CE4 Subtest3 : Extension of Adaptation Parameter Sets syntax for Quantization matrix”, JCT-VC document JCTVC-G295, 7th meeting, Geneva, CH, November 2011.

- M. Li, P. Wu, “Multiple Adaptation Parameter Sets Referring”, JCT-VC document JCTVC-G332, 7th meeting, Geneva, CH, November 2011.

- J. Tanaka, Y. Morigami, T. Suzuki, M. Li, P. Wu, “Using Multiple APSs for Quantization Matrix Parameters Signaling”, JCT-VC document JCTVC-G1026, 7th meeting, Geneva, CH, November 2011.

- S. Wenger, “Parameter set updates using conditional replacement”, JCT-VC document JCTVC-E309, 5th meeting, Geneva, CH, March 2011.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).