| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0382_r2 WG11 Number: m23257 |
| --- | --- |

| Title: | Inclusion of adaptive loop filtering in the HEVC standard |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 In Suk Chong2 Yu-Wen Huang3 |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com yuwen.huang@mediatek.com |
| Source: | CE coordinators (Toshiba1, Qualcomm2, MediaTek3) |  |  |

_____________________________

# Introduction

Adaptive loop filtering (ALF) was adopted in TMuC (Test Model under Consideration) and it has been part of WD (Working Draft) and HM (HEVC Test Model) from the beginning. ALF has been studied by the committee and making progress in terms of coding efficiency improvement and complexity reduction.

This contribution proposes to include ALF in the HEVC standard, and provide information how the HM 5 decoder is implemented.

# Benefit to support ALF

## Maturity of ALF

As described in the introduction section, ALF has been studied from the beginning of the “collaborative phase” of HEVC standardization.

Complexity reduction – number of filter shapes, pixel classification, number of filter coefficients, line buffers, etc.

HM 1:

Number of filter shapes is three (5x5/7x7/9x9 diamond).

Maximum number of coefficients is 21 including DC offset.

Pixels are classified up to 16 classes based on pixel base sum of Laplacian.

8 lines of line buffer are required at least.

HM3:

Number of filter shapes is three (5x5/7x7/9x7 diamond).

Maximum number of coefficients is 20 including DC offset.

Pixels are classified up to 15 classes based on 4x4 block base sum of Laplacian using 8 pixels with directional features in block adaptive (BA) mode.

6 lines of line buffer are required at least.

HM4:

Number of filter shapes is two (5x5 star and 11x5 cross).

Maximum number of coefficients is 10 including DC offset.

4 lines of line buffer are required at least.

HM5:

Number of filter shapes is two (5x5 star and 9x9 cross).

Maximum number of coefficients is 9 (no DC offset).

No line buffer is required at least.

Restriction of bit accuracy of ALF coefficients.

Use CAVLC to code ALF coefficients in APS.

Code cleanup.

Coding efficiency improvement

HM1 (TMuC):

CU synchronized ALF control.

HM3:

Addition of region adaptive (RA) mode.

Pixels are classified up to 15 classes based on 4x4 block base sum of Laplacian with directional features in block adaptive (BA) mode.

HM5:

Spatial prediction of ALF coefficients.

Misc.

HM1 (TMuC):

Signal ALF coefficients and ALF mode in slice header.

HM2:

Addition of a flag to indicate disabling filtering across slice boundary.

HM3:

Signal ALF coefficients and ALF mode in PPS.

Disable ALF when PCM mode is used.

HM4:

Signal ALF coefficients and ALF mode in APS.

HM5:

Addition of a flag to indicate disabling filtering across tile boundary.

Encoding algorithm simplification

HM1 (TMuC):

16-pass design scheme.

HM2:

1/2-pass design scheme as optional configuration.

HM4:

4x4 block base sum of Laplacian calculation using 4 pixels.

HM5:

2-pass design scheme as common test conditions (not yet included in the common test conditions).

Code cleanup.

Therefore, ALF is mature enough to be included in the HEVC standard.

## Discussion of simulation results

The coding efficiency improvement by ALF in HM 5.0 on the common test conditions is 1.6% (AI-HE), 3.3% (RA-HE), 2.8% (LDB-HE), 4.2% (LDP-HE) and 4.7% (RA-HE10) on average and up to 12.1% with the decoding time increase of 14% on average and no additional line buffer. It was reported in JCTVC-G923 that there is affinity for ALF with SIMD implementation. ALF provides higher BD-rate gain in high resolution sequences which will be widely deployed in the near future. Since the lifetime of the HEVC standard is expected to be longer than a decade, the benefit to support ALF in the HEVC standard becomes bigger in the future. For example, since television monitor that adopts 4K panel will launch this year, entertainment applications will support 4K contents through multiple channels (i.e., broadcasting, internet, and etc) in couple of years. Furthermore, Digital Cinema may also utilize the HEVC standard in such entertainment applications.

In applications that store compressed video in limited storage capacity such as DVD, Blu-Ray, SD card, etc., variable bit rate encoding provides better (or robust) picture quality throughout a sequence, which is realized by saving bits from relatively simple scenes and assigning the saved bits to complex scenes within the maximum bitrate allowed. In typical sequences, the duration of complex scenes is not so long. Therefore, saving 3 to 5% of bits from the other scenes would help improving the picture quality of the complex scenes. If ALF is used, it is not necessary to sacrifice the picture quality of the other scenes by increasing Qp to save bits.

In JCTVC-H0277, it is reported that the coding efficiency improvement for KTA 720p and 1080p sequences is 4.8% (RA-HE), 4.4% (LDB-HE) and 9.5% (LDP-HE) on average. Therefore, ALF is a promising tool.

When comparing ALF with APF that applies filter but not store the filtered image as reference pictures, the BD-rate gain of ALF is 1.2% (RA-HE), 2.3% (LDB-HE), 2.7% (LDP-HE) and 1.2% (RA-HE10) on average and up to 6.6% with 14% less decoding time on average. Furthermore, ALF processing in WD5 can be performed on the fly during LCU decoding without additional line buffer. On the other hand, a straight forward implementation of APF would require line buffers to reduce the external memory access.

Considering the above situation, ALF is one of the essential tools in the HEVC standard.

## Subjective evaluation of ALF

Subjective evaluation of ALF was done for HE with ALF on/off to see if the trend found in the objective measure is the same visually. A set of the decoded sequences obtained from the common test conditions simulation was viewed. 11 test sequences chosen which are from either common test sets or non-common test sets (i.e., sequences from VCEG activity) with QP=32,37 for both random access and low delay B structure were used. Those sequences are as follows.

BasketballDrive (Class B)

Cactus (Class B)

Kimono (Class B)

RaceHorses (Class C)

BasketballDrill (Class C)

Vidyo1 (Class E)

Vidyo3 (Class E)

SlideEditting (Class F)

Slideshow(Class F)

RiverBed (1080P)

SpineCalendar (720P)

ALF on cases mostly have lower bitrate and higher PSNR than ALF off cases. The monitors used in this test was Apple 32 inch LCD with resolution of 2560x1600.

Out of 11 sequences, 4 sequences (i.e., RiverBed, SpineCalendar, SlideEditting, and Slideshow) show clear visual gain of ALF for moving pictures and 4 sequences (i.e., Vidyo3, RaceHorses, BasketballDrive, and BasketballDrill) show clear visual gain for still images but slight gain for moving pictures. One interesting observation is that for some test sequences, ALF shows visual gain even though BD rate gain is less than other sequences (i.e., < 2.0% BD rate gain for RiverBed). Also some proposal shows the possibility that can improve visual quality by exploiting bit savings due to ALF [1].

# Conclusion

This contribution presents the benefit to support ALF in the HEVC standard and proposes to include ALF in the HEVC standard.

# Reference

[1]	“Subjective quality assessment of ALF on still images”, JCTVC-H0097, San Jose, CA, February 2012.

# Patent rights declarations

MediaTek Inc., Qualcomm and Toshiba Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).