| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0383 |
| --- | --- |

| Title: | Complexity reduction of significance map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposed to reduce the complexity of the coding of significant_coeff_flag in HM5.0 by using the same mapping to select luma and chroma context. As a result, a 15 elements lookup table and multiple branches can be removed to select the context index, and WD text is also simplified. A reduction of decoder runtime of 0% to 3% was observed. The BD-rate for AI_HE, RA_HE, LB_HE are 0.00%, -0.01%, 0.01%. The BD-rate for AI_LC, RA_LC, LB_LC are 0.00%, 0.01%,  -0.01%. The BD-rate for RA_HE10 is 0.03% .

# Introduction

For the encoding of significant_coeff_flag, HM5 adopted JCTVC-G1015 [4] which combined the followings:

- 4x4 positional based coding method from JCTVC-G657 [1] which has 9 contexts for luma and 6 contexts for chroma.

- 8x8 position based coding method from JCTVC-G366 [2] which has 11 contexts for luma and 11 contexts for chroma.

- 16x16/32x32 mask based coding method from JCTVC-G768 [3] which has 7 contexts for luma and 4 contexts for chroma.

As shown in Error: Reference source not found, the 4x4 positional based coding method in HM5 [1] has different grouping of significance map for luma and chroma. Therefore, different mapping for chroma and luma are needed to map the position in the 4x4 significance map to the corresponding context index increment. As a result, the following complexities are in HM5:

- Two 15 elements mapping tables are used in the HM5.0 software to determine the context increment.

- Branches based on the luma/chroma decisions are needed to determine the context increment in the coding of 4x4, 8x8, 16x16/32x32 significance map.

0

1

4

5

2

3

4

5

6

6

8

8

7

7

8

0

1

2

4

1

1

2

4

3

3

5

5

4

4

5

grouping of 4x4 luma context

grouping of 4x4 chroma context

Figure 1: In HM5.0 the 4x4 significance map has different grouping for luma and chroma contexts.

0

1

4

5

2

3

4

5

6

6

8

8

7

7

8

0

1

2

4

1b

2

4

3

3

5

5

4a

4a

5

grouping of 4x4 luma context

grouping of 4x4 chroma context

1a

Figure 1: In this contribution, the context 1a, 1b, and 4a are added to the chroma context so that the luma and chroma have the same number of contexts and the same mapping as presented by the color can be used to determine the context increments.

# A complexity reduction proposal

As shown by the grouping colors in Figure 1, this contribution proposes to reuse the same grouping of the 4x4 luma contexts for the grouping of the 4x4 chroma contexts. As a result, complexity is reduced in the following aspects:

- The chroma 15 elements mapping tables in the HM5.0 software can be replaced by the three additional contexts 1a, 1b, and 4a, where the context 1a, 1b are initialized with the same initial value as context 1, and the context 4a is initialized with the same initial value as context 4.

- Branches based on the luma/chroma decisions to determine the context increment in the coding of 4x4, 8x8 are removed. Branches based on the luma/chroma decision to determine the initial context offset in 16x16/32x32 significance map is also removed.

# Simulation results

The context reductions were integrated into HM5.0. The simulations were performed in three Microsoft HPC clusters, the common test conditions and reference configurations are followed:

- All intra simulations are performed on AMD Opteron Processor 6136 cluster @ 2.4GHz.

- All RA simulations are performed on Intel Xeon X5690 cluster @ 3.47GHz.

- All LD simulations are performed on Intel Xeon X5680 cluster @ 3.33GHz.

Error: Reference source not found shows the BD-rate and timing of the proposed complexity reduction for the coding of significance map.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.00% | -0.02% | -0.02% | 0.00% | 0.01% | -0.01% |  |  |  |
| Class B | 0.00% | -0.03% | 0.01% | 0.00% | 0.00% | 0.03% |  |  |  |
| Class C | 0.01% | -0.06% | -0.02% | 0.00% | -0.01% | 0.00% |  |  |  |
| Class D | 0.00% | 0.01% | -0.04% | 0.00% | 0.02% | -0.02% |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | -0.02% |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 0.00% | 0.01% | 0.00% |  |  |  |
|  | 0.00% | -0.02% | -0.01% | 0.00% | 0.01% | 0.00% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.03% | -0.11% | -0.21% | 0.00% | 0.07% | -0.17% | 0.03% | 0.15% | 0.13% |
| Class B | 0.00% | -0.06% | -0.01% | 0.02% | 0.15% | 0.20% | 0.03% | -0.05% | -0.09% |
| Class C | -0.04% | -0.06% | 0.07% | 0.02% | -0.03% | -0.06% |  |  |  |
| Class D | 0.03% | -0.19% | 0.08% | -0.01% | 0.31% | -0.24% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.10% | 0.01% | 0.01% | 0.14% | -0.03% | 0.03% | 0.04% | 0.00% |
|  | -0.01% | -0.13% | 0.01% | 0.01% | 0.11% | -0.07% | 0.04% | 0.07% | 0.05% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.10% | -0.04% | -0.12% | 0.17% |  |  |  |
| Class C | 0.01% | -0.01% | 0.09% | 0.03% | 0.08% | 0.06% |  |  |  |
| Class D | -0.02% | -0.05% | 0.10% | -0.02% | 0.63% | -0.09% |  |  |  |
| Class E | 0.07% | -0.37% | -0.43% | 0.03% | 0.09% | -0.10% |  |  |  |
| Overall | 0.01% | -0.08% | 0.00% | -0.01% | 0.16% | 0.03% |  |  |  |
|  | 0.01% | -0.05% | -0.03% | -0.01% | 0.19% | 0.05% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |  |  |  |

Table 1: BD-rate of proposed complexity reductions.

# Summary

As shown in Table 3: Average BDR of proposed significance map coding complexity reduction. and Table 3, this contribution reduced the decoder execution time from 0% to 3% and resulted in average luminance BD-rate of -0.01% to 0.03%.

|  | HE | LC | HE-10 |
| --- | --- | --- | --- |
| I | 99% | 100% |  |
| RA | 99% | 99% | 100% |
| LB | 97% | 99% |  |

Table 2: Average decoder time of proposed significance map coding complexity reduction.

|  | HE | LC | HE-10 |
| --- | --- | --- | --- |
| I | 0.00% | 0.00% |  |
| RA | -0.01% | 0.01% | 0.03% |
| LB | 0.01% | -0.01% |  |

Table 3: Average BDR of proposed significance map coding complexity reduction.

# References

- Gergely Korodi, Jinwen Zan, Da-ke He, Encoding and decoding significant coefficient flags for small Transform Units using partition sets”, JCTVC-G657, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Cheung Auyeung and Jun Xu, “Description Context reduction of significance map coding with CABAC”, JCTVC-G366, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Yinji Piao, Junghye Min, and JeongHoon Park, “Reduced contexts for significance map coding of large transform in CABAC”, JCTVC-G768, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Cheung Auyeung, Jun Xu, Gergely Korodi, Jinwen Zan, Da-ke He, Yinji Piao, Elena Alshina, Junghye Min, JeongHoon Park, “A combined proposal from JCTVC-G366, JCTVC-G657, and JCTVC-G768 on context reduction of significance map coding with CABAC”, JCTVC-G1015, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix:   Proposed WD text based on JCTVC-G1103_d6

9.2.1.1		Initialisation process for context variables

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-28 | 0..51 | 52..101 | 101..152 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |

9.2.3.1.1.5		Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Table 9-5 as follows..

sigCtx  =  ctxIdxMap4x4[ (yC << 2) + xC ]		(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Table 9-6 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  20		(9-4)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 21		(9-12)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-14)

Table 9-5 – Specifcation of ctxIdxMap4x4[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap4x4[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |

Table 9-6 – Specifcation of ctxIdxMap8x8[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap8x8[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 8 | 6 | 6 | 7 | 9 | 9 | 7 | 7 |

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

Table 9-7 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialisation variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 | 88 | 103 | 71 | 53 | 87 | 134 | 86 | 84 |
|  | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| initValue | 70 | 68 | 89 | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 87 | 87 | 149 | 52 | 70 |
|  | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| initValue | 52 | 118 | 133 | 116 | 114 | 129 | 132 | 162 | 115 | 51 | 115 | 66 | 120 | 74 | 115 | 87 | 89 |
|  | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 118 | 134 | 118 | 101 | 68 | 85 | 101 | 116 | 100 |
|  | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 |
| initValue | 68 | 67 | 136 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 102 | 102 | 70 | 67 | 53 |
|  | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 |
| initValue | 67 | 117 | 102 | 117 | 1158 | 114 | 84 | 115 | 99 | 100 | 83 | 114 | 152 | 168 | 131 | 150 | 120 |
|  | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 | 103 | 118 | 101 | 68 | 85 | 101 | 116 | 116 |
|  | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 |
| initValue | 68 | 67 | 152 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 102 | 102 | 86 | 67 | 84 |
|  | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 |
| initValue | 67 | 117 | 102 | 117 | 115 | 99 | 100 | 115 | 99 | 100 | 83 | 114 | 152 | 152 | 131 | 150 | 120 |

| Table 9-8 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| skip_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| cu_qp_delta | I | as specified in subclause Error: Reference source not found | 2 | Error: Reference source not found | 0 |
|  | P |  | 2 | Error: Reference source not found | 3 |
|  | B |  | 2 | Error: Reference source not found | 6 |
| pred_mode_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| part_mode | I | as specified in subclause Error: Reference source not found | 0 | Error: Reference source not found | 2 |
|  | P |  | 3 | Error: Reference source not found | 3 |
|  | B |  | 3 | Error: Reference source not found | 6 |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mpm_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | as specified in subclause Error: Reference source not found | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| merge_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| merge_idx | P | TU, cMax = MaxNumMergeCand − 1 | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| ref_idx_lc | B | TU, cMax = num_ref_idx_lC_active_minus1 | 2 | Error: Reference source not found | 3 |
| ref_idx_l0 | P | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Error: Reference source not found | 0 |
|  | B |  | 2 | Error: Reference source not found | 3 |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Error: Reference source not found | 3 |
| abs_mvd_greater0_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| abs_mvd_greater1_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | P/B | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_lc_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| mvp_l0_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| mvp_l1_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| split_transform_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| cbf_luma | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 4 |
| cbf_cb, cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| last_significant_coeff_x_prefix | I | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Error: Reference source not found | 0 |
|  | P |  | 8 | Error: Reference source not found | 30 |
|  | B |  | 8 | Error: Reference source not found | 60 |
| last_significant_coeff_y_prefix | I | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 | Error: Reference source not found | 0 |
|  | P |  | 8 | Error: Reference source not found | 30 |
|  | B |  | 8 | Error: Reference source not found | 60 |
| last_significant_coeff_x_suffix | all | FL, cMax = (last_significant_coeff_x_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_y_suffix | all | FL, cMax = (last_significant_coeff_y_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| significant_coeff_group_flag | I | FL, cMax = 1 | 0 | Table 9-26 | 0 |
|  | P |  | 0 | Table 9-26 | 4 |
|  | B |  | 0 | Table 9-26 | 8 |
| significant_coeff_flag | I | FL, cMax = 1 | 0 | Table 9-28 | 0 |
|  | P |  | 0 | Table 9-28 | 51 |
|  | B |  | 0 | Table 9-28 | 102 |
| coeff_abs_level_greater1_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 32 |
|  | B |  | 0 | Error: Reference source not found | 64 |
| coeff_abs_level_greater2_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 24 |
|  | B |  | 0 | Error: Reference source not found | 48 |
| coeff_abs_level_minus3 | all | prefix and suffix as specified in subclause Error: Reference source not found | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |