| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H384 WG11 Number: 23259 |
| --- | --- |

| Title: | CE4 Subset2.2: Results for up-sampling large size quantization matrices based on JCTVC-G152 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu Ximin Zhang Peter T.-D. Chuang Shawmin Lei 2860 Junction Ave San Jose, CA 95134, U.S.A. | Tel: Email: | +1 408 526 1899 shan.liu@mediatek.com, ximin.zhang@mediatek.com, peter.chuang@mediatek.com, shawmin.lei@mediatek.com. |
| Source: | MediaTek USA Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results for the method previously proposed in JCTVC-G152 [1] on HM5. In JCTVC-G152, it was proposed to skip coding the large size (i.e. 16x16 and 32x32) quantization matrices; instead generate them from 8x8 quantization matrices by up-sampling. Experiments were conducted based on CE4 subset2 anchor software [2]; 4x4 and 8x8 quantization matrices are coded using the AVC method. Taking the quantization matrices defined in “symmetry1.txt” as input, results report total 2026 bits are required for representing all quantization matrices per picture, with “MaxError” equals to 19 and “AvgError” equals to 2.00. In terms of BD-rate, in the worst case (i.e. quantization matrices are updated each picture) the proposed method reports average 4.6% savings for AI-HE, average 22.1% savings for RA-HE and RA-LC, average 28.8% savings for LB-HE, compared with the anchor AVC method.

# Introduction

It was decided in previous meetings that quantization matrices are supported in HEVC. Unlike in the H.264/AVC standard, where only 4x4 and 8x8 transforms are involved, in HEVC, transforms can be as large as 16x16 and 32x32 (under current common test conditions); besides, non-square transforms such as  4x16, 16x4, 8x32 and 32x8 transforms have also been adopted. Therefore, it is desired to support quantization matrices with all the sizes that have been motioned. As a consequence, the bit rate overhead for coding all these quantization matrices, as well as the memory size for storing all these quantization matrices could be tremendously high. In the last Geneva meeting, the method for generating non-square quantization matrices from square quantization matrices by one-directional sub-sampling [1] was adopted to HM and WD, which reduces the burden of coding and storing quantization matrices in HEVC to some extent. However, the bit rate overhead for coding 16x16 and 32x32 quantization matrices is still a concern. Thus there is the need for a more effective solution for handling large size (i.e. 16x16 and 32x32) quantization matrices in HEVC.

# Proposed Method

In JCTVC-G152 [1] it was proposed to skip coding 16x16 and 32x32 quantization matrices. Instead, they are generated by up-sampling 8x8 quantization matrices. The up-sampling is achieved by interpolation, which is illustrated in Equations (1)-(6).

C2Nx2N(2*i, 2*j) 	   	= CNxN(i, j), 					i=0,…N-1; j=0,…N-1. 	(1)

C2Nx2N(2*i, 2*j+1)   	= (CNxN(i, j) + CNxN(i, j+1)) >> 1, 		i=0,…N-1; j=0,…N-2.	(2)

C2Nx2N(2*i+1, 2*j) 	= (CNxN(i, j) + CNxN(i+1, j)) >> 1, 		i=0,…N-2; j=0,…N-1.	(3)

C2Nx2N(2*i+1, 2*j+1) 	= (CNxN(i,j)+CNxN(i,j+1)+CNxN(i+1,j)+CNxN(i+1,j+1)) >> 2,

i=0,…N-2; j=0,…N-2.	(4)

C2Nx2N(i, 2N-1)		= C2Nx2N(i, 2N-2)				i=0,…2N-2.		(5)

C2Nx2N(2N-1, j)		= C2Nx2N(2N-2, j)				j=0,…2N-1.		(6)

Fig. 1 presents a sample 8x8 quantization matrix. A sample 2Nx2N (16x16) quantization matrix that is generated from the sample NxN (8x8) quantization matrix shown in Fig. 1using the proposed method is shown in Fig. 2.

| 6 | 10 | 13 | 16 | 18 | 23 | 25 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 16 | 18 | 23 | 25 | 27 | 29 |
| 13 | 16 | 18 | 23 | 25 | 27 | 29 | 31 |
| 16 | 18 | 23 | 25 | 27 | 29 | 31 | 33 |
| 18 | 23 | 25 | 27 | 29 | 31 | 33 | 36 |
| 23 | 25 | 27 | 29 | 31 | 33 | 36 | 38 |
| 25 | 27 | 29 | 31 | 33 | 36 | 38 | 40 |
| 27 | 29 | 31 | 33 | 36 | 38 | 40 | 42 |

Fig. 1 A sample 8x8 quantization matrix

| 6 | 8 | 10 | 11 | 13 | 14 | 16 | 17 | 18 | 21 | 23 | 24 | 25 | 26 | 27 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 9 | 10 | 12 | 14 | 15 | 17 | 18 | 20 | 22 | 24 | 25 | 26 | 27 | 28 | 28 |
| 10 | 10 | 11 | 13 | 16 | 17 | 18 | 20 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 29 |
| 11 | 12 | 13 | 15 | 17 | 18 | 20 | 22 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 30 |
| 13 | 14 | 16 | 17 | 18 | 20 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 31 |
| 14 | 15 | 17 | 18 | 20 | 22 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 32 |
| 16 | 17 | 18 | 20 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 33 |
| 17 | 18 | 20 | 22 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 34 |
| 18 | 20 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 36 |
| 21 | 22 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 35 | 36 | 37 | 37 |
| 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 37 | 38 | 38 |
| 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 37 | 38 | 39 | 39 |
| 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 35 | 36 | 37 | 38 | 39 | 40 | 40 |
| 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 37 | 38 | 39 | 40 | 41 | 41 |
| 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 42 |
| 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 42 |

Fig. 2 Sample 16x16 quantization matrix generated from the sample 8x8 quantization matrix (shown in Fig. 1) using the proposed method.

Besides of the original JCTVC-G152 proposed method, an additional test is included for harmonization with adopted HVS 16x16 and 32x32 quantization matrices. In this test, a set of 8x8 basis quantization matrices to represent 16x16/32x32 default quantization matrices is also proposed in CE4 subtest2.2. In the proposed implementation for this method, two 8x8 matrices are generated from the current default 16x16 and 32x32 Inter/Intra matrices by down-sampling. These two 8x8 matrices are stored and used as the basis default matrices for 16x16 and 32x32 TU. During the coding process, these two 8x8 matrices will be up-sampled to generate 16x16 and 32x32 quantization matrices if the default quantization matrices are selected by the 16x16/32x32 TU in the current slice. In doing so, the memory usage to store the 16x16 and 32x32 default quantization matrices are saved.

# Experimental Results

Simulations were conducted following common test conditions defined in JCTVC-G1200 Error: Reference source not found and CE4 description [4]. Two sets of results are provided below based on CE4 subset2 recommendations.

Table 1 report the total number of bits used for representing quantization matrices (per picture), the maximum error and average error for each set of input quantization matrices in CE4 subset2 test cases.

Table 1 Total bits, MaxError and AvgError generated by the proposed method for CE4 subset2 test cases

|  | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][1] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][2] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][3] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][4] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][5] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [1][0] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][3] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][4] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][5] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Total Bit | 2026 | 1260 | 2576 | 2552 | 1428 | 2110 |
| MaxError | 19 | 14 | 66 | 37 | 62 | 103 |
| AvgError | 2.00 | 3.45 | 19.98 | 4.87 | 18.83 | 33.99 |

|  | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][1] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][2] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][3] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][4] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][5] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [1][0] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][3] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][4] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][5] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Total Bit | 2400 | 1456 | 2860 | 3714 | 1684 | 2430 |
| MaxError | 10 | 11 | 86 | 129 | 55 | 134 |
| AvgError | 2.19 | 2.78 | 17.22 | 15.43 | 15.57 | 28.32 |

Table 2 reports the BD-rate for the proposed method compared with CE4 subset2 anchor. For input quantization matrices, “symmetry1” was used as released and recommended in CE4. The results show average 4.4% BD-rate savings compared with anchor for AI-HE, average 20-30% BD-rate savings compared with anchor for RA and LB configurations. The BD-rate savings are mostly from the bits saved from coding 16x16 and 32x32 quantization matrices. PSNR difference between the proposed and the anchor is not significant.

Table 2 BD-rate for the proposed method compared with CE4 subset2 anchor, using “symmetry1” input.

# Conclusion

This contribution reports the results for the method that was previously proposed in JCTVC-G152 [1] on HM5. Taking one of the CE4 subset2 test cases “symmetry1” as input, the experimental results report total 2026 bits are required for representing all quantization matrices per picture, with “MaxError” equals to 19 and “AvgError” equals to 2.00. In terms of BD-rate, in the worst case (i.e. quantization matrices are updated each picture) the proposed method reports average 4.6% savings for AI-HE, average 22.1% savings for RA-HE and RA-LC, average 28.8% savings for LB-HE, compared with the anchor AVC method.

# References

- X. Zhang, S. Liu and S. Lei, “Method and syntax for generating large quantization matrices and non-square quantization matrices”, JCTVC-G152, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, November 2011.

- CE4 subset2.2 anchor software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0-qmatrix.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, November 2011.

- K. Sato, M. Budagavi, M. Coban, H. Aoki and X. Li, “Description of Core Experiment 4 (CE4): Quantization”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).