| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0385 WG11 Number: m23260 |
| --- | --- |

| Title: | CE8.b.1: Cross check of Modification of ALF BA classification (JCTVC-H0413) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC-H0413 for Samsung and SONY’s CE8 proposal.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-H0413, which is a contribution to CE8 Subset b.1. Verification is performed under the CE8 test conditions.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.4% | 0.1% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.0% | 0.3% |
| Class C | 0.1% | -0.1% | 0.2% |
| Class D | 0.1% | -0.2% | -0.1% |
| Class E | 0.1% | 0.1% | -0.3% |
| Class F | 0.0% | 0.2% | -0.3% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.3% | -0.1% |
| Class C | 0.2% | 0.1% | -0.1% |
| Class D | 0.1% | 0.3% | -0.1% |
| Class E | 0.1% | -0.1% | 0.3% |
| Class F | 0.0% | 0.0% | 0.2% |
| Overall | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.3% | -0.2% | 0.0% |
| Class B | 0.1% | 0.0% | 0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.2% | -0.1% | 0.0% |
|  | 0.2% | -0.1% | 0.0% |
| Class F |  |  |  |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

Table 1. Samsung and SONY’s proposal on CE8 Tool b.1

# Source code review

The code follows the description of CE8 proposal