| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0386 |
| --- | --- |

| Title: | Information for scalable extension high layer syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jill Boyce, Wonkap Jang, Danny Hong, Stephan Wenger 433 Hackensack Ave Hackensack, NJ 07601 USA Ye-Kui Wang, Ying Chen Qualcomm USA | Tel: Email: | +1 201 478 6245 {jill,wonkap,danny,stephan}@ vidyo.com {yekuiw,cheny}@qualcomm.com |
| Source: | Vidyo, Qualcomm |  |  |

____________________________

# Abstract

This document provides an example design of NAL unit header and parameter sets for future HEVC scalable and 3DV extensions, based on the proposed extension hooks in JCTVC-H0388.

The extension hooks in JCTVC-H0388 are proposed to enable future HEVC scalable and 3DV extensions in a backwards-compatible manner, maintaining the existing temporal scalability capability.  In the example design, a mapping of each layer_id value to dependency_id, quality_id, view_id, and depth_flag values is provided in the VPS.  The VPS also indicates if a layer is dependent upon another layer, and if so, the directly dependent reference layer.  The access unit delimiter is changed to provide per layer presence information with normative impact, and is made mandatory.

# The example design

## NAL unit header

The NAL unit header is the same as in JCTVC-H0388.  There is no need to change the NAL unit header, the same NAL unit header design applies to both single-layer coding in the HEVC base specification and the future scalable and 3DV extensions.

## Video parameter set

The VPS provides information about the dependency relationships of the layers in a bitstream, as well as many other information that are applicable to all slices across all (scalability or view) layers in the entire coded video sequence.  It contains some of the information that was present in the SVC Scalability Information SEI message, but unlike the SEI message, the VPS is normative and required.  The presence of the VPS is required for all bitstreams, even those that do not make use of the HEVC extension features.  This allows for backwards compatibility of future extensions.  The burden on encoders and decoders is minimal, as a VPS which indicates that no extension layers are present will contain very few bits that are not needed for the base layer.  Any profile that restricts the number of layers in the VPS to one will not require any special decoding process beyond parsing the VPS and the GPS.

In the VPS, for each possible layer_id value, values are specified of dependency_id, quality_id, view_id, depth_map_flag, and extension_id for the layer.

For each layer_id value, the VPS also contains a dependent_flag.  If the value is equal to 1, the directly dependent reference layer is signaled using delta_reference_layer_id_plus1.  A variable ReferenceLayer_id is calculated to indicate the directly dependent reference layer.

At most one VPS may be active at a time.  A VPS is activated when it is not already active and it is referred by a coded slice NAL unit in a particular layer in an IDR access unit being decoded, though through the active GPS for that particular layer.  The VPS must be available before the NAL unit that activates the VPS.  Once activated, the VPS applies to the entire coded video sequence.  In other words, the active VPS can only change at an IDR access unit.

Changing the contents of the VPS frequently is not recommended.  The access unit delimiter is used to change which layers are active without requiring a change in the contents of the VPS or the use of IDR access units.  The VPS itself contains a maximum number of layers and layer dependencies for the sequence.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | ue(v) |
| num_layers_minus1 | ue(v) |
| num_temporal_layers_minus1 | u(3) |
| for ( i = 0; i <= num_temporal_layers_minus1; i++ ) { |  |
| profile_idc[ i ] | u(8) |
| reserved_zero_8bits[ i ] /* equal to 0 */ | u(8) |
| level_idc[ i ] | u(8) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| chroma_format_idc | u(2) |
| temporal_id_nesting_flag | u(1) |
| bit_equal_to_one /* equal to 1 */ | u(1) |
| extension_type | ue(v) |
| num_rep_formats_minus1 | ue(v) |
| for( i = 0; i < num_rep_formats_minus1; i++ ) { |  |
| bit_depth_luma_minus8[ i ] | ue(v) |
| bit_depth_chroma_minus8[ i ] | ue(v) |
| chroma_format_idc[ i ] | u(2) |
| } |  |
| for( i = 1; i <= num_layers_minus1; i++ ) { |  |
| rep_format_idx[ i ] | ue(v) |
| if( extension_type = = 0 ¦ ¦ extension_type = = 2 ) { |  |
| dependency_id[ i ] | ue(v) |
| quality_id[ i ] | ue(v) |
| } |  |
| if( extension_type = = 1 ¦ ¦ extension_type = = 2 ) { |  |
| view_id[ i ] | ue(v) |
| depth_flag[ i ] | u(1) |
| } |  |
| dependent_flag | u(1) |
| } |  |
| num_additional_profiles_levels_minus1 | ue(v) |
| for( i = 0; i <= num_additional_profiles_levels_minus1; i++ ) { |  |
| additional_profile_idc[ i ] | u(8) |
| additional_reserved_zero_8bits[ i ] /* equal to 0 */ | u(8) |
| additional_level_idc[ i ] | u(8) |
| num_applicable_operation_points_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_applicable_operation_points[ i ]; j++ ) { |  |
| temporal_id[ i ][ j ] | ue(v) |
| layer_id[ i ][ j ] | ue(v) |
| if( extension_type = = 1 ){ |  |
| num_target_output_views_minus1[ i ][ j ] | ue(v) |
| for( k = 0; k < num_target_output_views_minus1[ i ][ j ]; k++ ) |  |
| layer_id[ i ][ j ][ k ] | ue(v) |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

bit_equal_to_one shall be equal to 1.

extention_type equal to 0 specifies that multiple dependency and/or quality layers may be present in the bitstream.  extension_type equal to 1 specifies that multiple view layers may be present in the bitstream. extension_type equal to 2 specifies that multiple dependency and/or quality layers as well as multiple view layers may be present in the bitstream. The value of extension_type shall be in the range of 0 to 2, inclusive.

num_rep_formats_minus1 plus 1 specifies the maximum number of different sets of bit depth and chroma format (i.e., the sets of bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc values) in the coded video sequence. The value of num_rep_formats_minus1 shall be in the range of 0 to X, inclusive. The set of bit depth and chroma format for the base layer is signalled by bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc, and additional sets of bit depth and chroma format are signalled for enhancement layers by the following set of syntax elements bit_depth_luma_minus8[ i ], bit_depth_chroma_minus8[ i ], and chroma_format_idc[ i ].

bit_depth_luma_minus8[ i ], bit_depth_chroma_minus8[ i ], and chroma_format_idc[ i ] specify the i-th additional set of bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc values in the coded video sequence.

rep_format_idx[ i ] specifies the values index to the set of additional bit depth and chorma format that applies to the layer with layer_id equal to i. The values of bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc for the layer with layer_id equal to i are equal to bit_depth_luma_minus8[ rep_format_idx[ i ] ], bit_depth_chroma_minus8[ rep_format_idx[ i ] ], and chroma_format_idc[ rep_format_idx[ i ] ], respectively. The value of rep_format_idx[ i ] shall be in the range of 0 to X, inclusive.

dependency_id[ i ] specifies a dependency identifier for the layer with layer_id equal to i.  dependency_id[ i ] shall be in the range of 0 to X inclusive.  When not present, dependency_id[ i ] shall be inferred to be 0.  When num_directly_dependent_layers[ i ] is greater than 0, dependency_id[ i ] shall be equal to or greater than the dependency identifier of any layer the layer with layer_id equal to i depends on.

quality_id[ i ] specifies a equality identifier for the layer with layer_id equal to i. quality_id[ i ] shall be in the range of 0 to X inclusive. When not present, quality_id[ i ] shall be inferred to be 0. When num_directly_dependent_layers[ i ] is greater than 0, quality_id[ i ] shall be equal to or greater than the quality identifier of any layer that the layer with layer_id equal to i depends on and that has dependency identifier equal to dependency_id[ i ].

view_id[ i ] specifies a view identifier for the layer with layer_id equal to i. view_id[ i ] shall be in the range of 0 to X inclusive. When not present, view_id[ i ] shall be inferred to be 0.

depth_flag[ i ] equal to 1 specifies that the layer with layer_id equal to i represents a depth map.  depth_flag[ i ] equal to 0 specifies that the layer with layer_id equal to i represents texture. When not present, depth_flag[ i ] shall be inferred to be 0.

num_directly_dependent_layers[ i ] specifies the number layer the layer with layer_id equal to i directly depends on. When num_directly_dependent_layers[ i ] is equal to 0, the layer with layer_id equal to i is an independent layer, which can be decoded independently of other layers. When not present, the value of num_directly_dependent_layers[ i ] shall be inferred to be 0.

delta_reference_layer_id_minus1[ i ][ j ] specifies the j-th layer the layer with layer_id equal to i is directly dependent upon. The value of delta_reference_layer_id_minus1[ i ] shall be in the range of 0 to num_layers_minus1 – 1, inclusive.  The variable, RefLayerId[ i ][ j ], which is equal to the layer_id of the j-th layer the layer with layer_id equal to i is directly dependent upon, is derived as

if( j  = =  0 )
			RefLayer_id[ i ][ j ] = i − delta_reference_layer_id_minsu1[ i ] – 1
		else
			RefLayer_id[ i ][ j ] = RefLayer_id[ i ][ j – 1 ] – delta_reference_layer_id_minsu1[ i ] – 1

num_additional_profiles_levels_minus1 plus 1specifies the number of additional set of profile and level information signalled in the video parameter set. The value of num_additional_profiles_levels_minus1 shall be in the range of 0 to X, inclusive.

additional_profile_idc[ i ], additional_reserved_zero_8bits[ i ], and additional_level_idc[ i ] specify the i-th additional set of profile and level information signalled in the video parameter set.

num_applicable_operation_points_minus1[ i ] plus 1 specifies the number of operation points to which the i-th set of additional profile and level information applies.

temporal_id[ i ][ j ] specifies the temporal_id of the j-th operation point to which the i-th additional set of profile and level information applies.

num_target_output_views_minus1[ i ][ j ] plus 1 specifies the number of target output views in the j-th operation point to which the i-th additional set of profile and level information applies.

layer_id[ i ][ j ][ k ] specifies the layer_id of the k-th target output view in the j-th operation point to which the i-th set of additional profile and level information applies.

Other syntax elements have the same semantics as the syntax elements with the same names in the VPS in JCTVC-H0388.

## Sequence/picture/adaptation/group parameter sets

- The SPS, PPS, APS, and GPS will be extended to append more scalable and/or 3DV specific syntax elements, if any, using the backwards-compatible extension mechanism through the last two syntax elements in the RBSP syntaxes, similarly as the above extension of VPS.

- However, it is expected that most likely only an extension to the SPS is needed.

## Access unit delimiter

Mandatory use of a modified access unit delimiter is included in the example design, for each access unit.   The access unit contains information that applies to all layers in the sequence, for that particular access unit.

The encoder uses the access unit delimiter to indicate which layers are active in each access unit, which allows dynamic changes to layer presence and dependencies, within the bounds specified in the VPS, which limits the maximum number and types of layers.    Unlike SVC’s layers not present SEI message, the changed access unit delimiter has normative decoding impact.  It incorporates some of the capabilities of the ref_layer_dq_id syntax element of the SVC slice header scalable extension, and the no_inter_pred_pref_flag in the NAL unit header SVC extension.

Layers can be removed by the encoder without requiring the use of IDR frames.  When an encoder removes a layer in an access unit, the dependencies of other layers upon the removed reference layer for that access unit are modified.

The MANE can modify the contents of the access unit delimiter during bitstream extraction, when it removes layers.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| primary_pic_type | u(3) |
| if (num_layers_minus1 > 0) |  |
| for( i = 0; i <= num_layers_minus1; i++ ) { |  |
| layer_not_present_flag[ i ] | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

layer_not_present_flag[ i ] indicates the layer identifier of a layer representation for which no VCL NAL units are present in the target access unit set, impacting the decoding process of subclause X.  If not present, layer_not_present_flag[ i ] shall be inferred to be 0.

When ReferenceLayer_id[ j ] is equal to i and layer_not_present_flag[ i ] is equal to 0,

- if dependent_flag[ i ] equal to 0

  - dependent_flag[ j ] is inferred to be equal to 0.

- else

  - ReferenceLayer_id[ j ] = ReferenceLayer_id[ i ]

## General decoding semantics

A bitstream contains one or more layers.  Each layer is either a dependent or independent layer, as indicated by dependent_flag.   A dependent layer is decoded using a directly dependent layer, represented by the variable ReferenceLayer_id, which itself may have further dependencies.  An independent layer may be coded independently of other layers, e.g. a simulcast layer.

A sub-bitstream is derived as a subset from a bitstream by discarding zero or more NAL units.  A sub-bitstream contains one or more independent layer and zero or more dependent layers.

A layer set contains exactly one independent layer and zero or more dependent layers.  A layer set associated with a layer contains the layer and any reference layers which that layer is dependent upon.

## VPS and layer set example

An example VPS is illustrated below, for a sequence with 5 layers.

| layer_id | dependent_flag | ReferenceLayer_id (derived from delta_reference_layer_id_minus1) | dependency_id | quality_id | view_id |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | - | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |
| 2 | 1 | 1 | 2 | 0 | 0 |
| 3 | 1 | 0 | 3 | 0 | 0 |
| 4 | 1 | 3 | 3 | 1 | 0 |
| 5 | 0 | - | 4 | 0 | 0 |

Layer 0 is an independent layer, not dependent on other layers. The layer set associated with layer 0 includes layer 0.

Layer 1 is a spatial scalable dependent layer, and its directly dependent reference layer is layer 0.  The layer set associated with layer 1 includes layers 0 and 1.

Layer 2 is a spatial scalable dependent layer, and its directly dependent reference layer is layer 1.  The layer set associated with layer 1 includes layers 0, 1, and 2.

Layer 3 is a spatial scalable dependent layer, and its directly dependent reference layer is layer 0.  The layer set associated with layer 3 includes layers 0 and 3.

Layer 4 is a quality dependent layer, and its directly dependent reference layer is layer 3.  The layer set associated with layer 4 includes layers 0, 3, and 4.

Layer 5 is an independent layer, not dependent on other layers.  The layer set associated with layer 5 includes layer 5.

# Discussion

It is anticipated that most scalable or 3DV extensions will not make full use of the maximum number of layers in each of the possible dimensions.  Combining the dependency_id, quality_id, view_id, and depth_flag into a single layer_id lookup table is bitrate efficient, keeping the NAL unit header short.  An industry forum aimed at a particular application may choose to restrict the layer_id values to a direct mapping of a subset of those parameters.  For example, a scalable video conferencing system may choose to allocate the 5 layer_id bits as 2 bits for dependency_id and 3 bits for quality_id.

The VPS provides information on sequence-layer dependencies, and is expected to not change frequently.  The encoder can set the VPS parameters to indicate the maximum scalability capabilities that will be used in the coded sequence, in terms of number of layers, types of layers, and number of layer dependencies.  The proposed access unit delimiter allows the encoder to indicate which layers are active in each access unit, with normative impact.

When an encoder removes a layer in an access unit, the dependencies of other layers upon the removed reference layer for that access unit are modified.   For example, consider a coded sequence using 2 layers of spatial scalability, with layer 0 as the independent layer, and layer 1 dependent on layer 0.  An encoder may choose to not use two spatial layers for a number of frames, but only a single layer at the higher resolution.  By setting the layer_not_present_flag[0] equal to 1 for an access unit, the decoder knows when decoding the layer 1 frame that it will be decoded independently from the layer 0 frame.

Similarly, consider a coded sequence using 3 layers of spatial scalability, with layer 0 as the independent layer, layer 1 dependent on layer 0, and layer 2 dependent on layer 1.  An encoder may choose to not use the intermediate layer 1 for one or more frames.  By setting the layer_not_present_flag[1] equal to 1 for an access unit, the decoder may infer when decoding that frame that the layer 2 frame will be decoded dependently on the layer 0 frame, rather than the layer 1 frame.

Additionally, a MANE may perform bitstream extraction and remove layer 2 from the bitstream, and update the access unit delimiter to change the value of layer_not_present_flag[2] to 1.  A MANE has restrictions when removing layers.  It may only remove a reference layer when it also removes any layers that are dependent upon it.

When a decoder receives an access unit delimiter, it will know that the previous access unit is complete, and that it may decode all present layers.  The layers_not_preset_flag values allow the decoder to determine whether a packet has been lost, or if the access unit can be safely decoded with no errors.

The dependent_flag in the VPS is useful to a decoder or MANE, to identify independent layers.  If a decoder receives two independent layers, it can choose to decode and render only one of the layers.   The layer with the higher layer_id is expected to be of higher quality and greater decoding complexity.

# Patent rights declaration(s)

Vidyo Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).