| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H387 WG11 Number: m23262 |
| --- | --- |

| Title: | Non-CE4: Handling large size quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu Ximin Zhang Shawmin Lei 2860 Junction Ave San Jose, CA 95134, U.S.A. | Tel: Email: | +1 408 526 1899 shan.liu@mediatek.com, ximin.zhang@mediatek.com, shawmin.lei@mediatek.com. |
| Source: | MediaTek USA Inc. |  |  |

_____________________________

# Abstract

This contribution reports the studies and results about methods for handling large size quantization matrices. First, the relationship between BD-rates (bit rates) and the number of quantization matrices to code is discussed. Then, a couple of mechanisms for deriving large size (16x16 and 32x32) quantization matrices are proposed. In the first method, it is propose to derive 16x16 quantization matrices by up-sampling 8x8 quantization matrices using interpolation, and derive 32x32 quantization matrices by up-sampling 16x16 quantization matrices by repetition. In the second method, it is propose to derive 16x16 and 32x32 quantization matrices by up-sampling 8x8 quantization matrices, using interpolation for low frequencies and repetition for higher frequencies. In both methods, large size quantization matrices may be skipped from coding. Experiments were conducted based on CE4 subset2 anchor software [1]. Taking “Symmetry1” as an example, results reportedly show that in the worst case (i.e. quantization matrices are updated each picture) by skipping coding 16x16 and 32x32 quantization matrices, the average BD-rate savings are 4.6% AI-HE, 22.1% RA-HE and RA-LC, 28.8% for LB-HE, compared with current anchor. Maximum error and average error of derived quantization matrices compared with original quantization matrices are 19 and 2.0, respectively. By skipping coding only 32x32 quantization matrices, the average BD-rate savings are 3.2% AI-HE, 14.7% RA-HE and RA-LC, 20.6% for LB-HE, compared with current anchor. Maximum error and average error of derived quantization matrices compared with original quantization matrices are 9 and 0.5, respectively.

# Introduction

It was decided in previous meetings that quantization matrices are supported in HEVC. Unlike in the H.264/AVC standard, where only 4x4 and 8x8 transforms are involved, in HEVC, transforms can be as large as 16x16 and 32x32 (under current common test conditions); besides, non-square transforms such as  4x16, 16x4, 8x32 and 32x8 transforms have also been adopted. Therefore, it is desired to support quantization matrices with all the sizes that have been motioned. As a consequence, the bit rate overhead for coding all these quantization matrices, as well as the memory size for storing all these quantization matrices could be tremendously high. In the last Geneva meeting, the method for generating non-square quantization matrices from square quantization matrices by one-directional sub-sampling [1] was adopted to HM and WD, which reduces the burden of coding and storing quantization matrices in HEVC to some extent. However, the bit rate overhead for coding 16x16 and 32x32 quantization matrices is still a concern. Thus there is the need for a more effective solution for handling large size (i.e. 16x16 and 32x32) quantization matrices in HEVC.

# Proposed Method

## Handling 16x16 quantization matrices

Besides non-square transforms, the current HM supports 4x4, 8x8, 16x16 and 32x32 transforms. It has been largely agreed that 4x4 and 8x8 quantization matrices are coded using AVC (or AVC like) method. For large size (32x32 and 16x16) quantization matrices, the current options are (a) lossless coding; (b) lossy coding; and (c) up-sampling without coding. In fact, (c) can be seen as a special case of (b). Taking one test CE4 subset2.2 test case “symmetry1” as input quantization matrices, we did two tests.

- Test 1: code 4x4, 8x8 quantization matrices (anchor/AVC method), up-sample (interpolate) 16x16 and 32x32 quantization matrices from 8x8 quantization matrices without coding.

- Test 2: code 4x4, 8x8 and 16x16 quantization matrices (anchor/AVC method), up-sample (interpolate) 32x32 quantization matrices from 8x8 quantization matrices without coding.

The maximum and average errors (difference between original and reconstructed quantization matrices) for the two tests are shown in Table 1 in Section 3. The BD-rates for the Test 1 and Test 2 are shown in Table 2 and Table 3 in Section 3, respectively. Clearly there is a trade off between the bit rate overhead spent in coding quantization matrices and the accuracy of presenting quantization matrices. Test 2 compromised roughly 30% BD-rate (bit rate) savings compared with Test 1, in return, all 16x16 quantization matrices are lossless, 32x32 quantization matrices are interpolated from 16x16 quantization matrices (instead of from 8x8 quantization matrices as in Test 1) and thus, the errors between original and reconstructed 32x32 quantization matrices may be reduced. It is recommended to consider the option of coding 4x4, 8x8 and 16x16 quantization matrices and generating 32x32 quantization matrices by up-sampling 16x16 quantization matrices.

## Up-sampling Method 1

In the last meeting, there was some concern about the computational complexity for up-sampling quantization matrices using linear interpolation. In this contribution, we propose to generate 16x16 quantization matrices from 8x8 quantization matrices using simple linear interpolation, shown in Equations (1)-(6). 32x32 quantization matrices are derived from 16x16 quantization matrices by repeating existing samples, as illustrated in Equation (7).

C2Nx2N(2*i, 2*j) 	   	= CNxN(i, j), 					i=0,…N-1; j=0,…N-1. 	(1)

C2Nx2N(2*i, 2*j+1)   	= (CNxN(i, j) + CNxN(i, j+1)) >> 1, 		i=0,…N-1; j=0,…N-2.	(2)

C2Nx2N(2*i+1, 2*j) 	= (CNxN(i, j) + CNxN(i+1, j)) >> 1, 		i=0,…N-2; j=0,…N-1.	(3)

C2Nx2N(2*i+1, 2*j+1) 	= (CNxN(i,j) + CNxN(i,j+1) + CNxN(i+1,j) + CNxN(i+1,j+1)) >> 2,

i=0,…N-2; j=0,…N-2.	(4)

C2Nx2N(i, 2N-1)		= C2Nx2N(i, 2N-2)				i=0,…2N-2.		(5)

C2Nx2N(2N-1, j)		= C2Nx2N(2N-2, j)				j=0,…2N-1.		(6)

C2Nx2N(2*i, 2*j)	= C2Nx2N(2*i+1, 2*j)

= C2Nx2N(2*i, 2*j+1)

= C2Nx2N(2*i+1, 2*j+1)

= CNxN(i, j) ,						i=0,…N-1; j=0,…N-1. 	(7)

## Up-sampling Method 2

Another possible way to reduce the computational complexity in quantization matrix up-sampling is to use linear interpolation for deriving partial large quantization matrices and use simple repetition for generating the rest coefficients. An example is shown in Fig. 1. In this example, a 16x16 quantization matrix is derived from an 8x8 quantization matrix, where the top-left 8x8 (low frequency) coefficients are calculated using the interpolation method illustrated in Equation (1)-(6), and the rest (higher frequency) coefficients are generated by repeating existing samples as illustrated in Equation (7). Furthermore, a syntax parameter could be added to SPS and/or PPS to indicate the region that is up-sampled by interpolation or repetition.

| 6 | 10 | 13 | 16 | 18 | 23 | 25 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 16 | 18 | 23 | 25 | 27 | 29 |
| 13 | 16 | 18 | 23 | 25 | 27 | 29 | 31 |
| 16 | 18 | 23 | 25 | 27 | 29 | 31 | 33 |
| 18 | 23 | 25 | 27 | 29 | 31 | 33 | 36 |
| 23 | 25 | 27 | 29 | 31 | 33 | 36 | 38 |
| 25 | 27 | 29 | 31 | 33 | 36 | 38 | 40 |
| 27 | 29 | 31 | 33 | 36 | 38 | 40 | 42 |

| 6 | 8 | 10 | 11 | 13 | 14 | 16 | 17 | 18 | 18 | 23 | 23 | 25 | 25 | 27 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 9 | 10 | 12 | 14 | 15 | 17 | 18 | 18 | 18 | 23 | 23 | 25 | 25 | 27 | 27 |
| 10 | 10 | 11 | 13 | 16 | 17 | 18 | 20 | 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 |
| 11 | 12 | 13 | 15 | 17 | 18 | 20 | 22 | 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 |
| 13 | 14 | 16 | 17 | 18 | 20 | 23 | 24 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 |
| 14 | 15 | 17 | 18 | 20 | 22 | 24 | 25 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 |
| 16 | 17 | 18 | 20 | 23 | 24 | 25 | 26 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 |
| 17 | 18 | 20 | 22 | 24 | 25 | 26 | 27 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 |
| 18 | 18 | 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 |
| 18 | 18 | 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 |
| 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 |
| 23 | 23 | 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 |
| 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 | 40 | 40 |
| 25 | 25 | 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 | 40 | 40 |
| 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 | 40 | 40 | 42 | 42 |
| 27 | 27 | 29 | 29 | 31 | 31 | 33 | 33 | 36 | 36 | 38 | 38 | 40 | 40 | 42 | 42 |

Fig. 1 Sample 16x16 quantization matrix generated from sample 8x8 quantization matrix using proposed method 2.

# Experimental Results

Simulations were conducted following common test conditions defined in JCTVC-G1200 Error: Reference source not found and CE4 description [4].

Table 1 reports the number of bits spent on coding quantization matrices, maximum and average error for Test 1 (code 4x4, 8x8 quantizatin matrices) and Test 2 (code 4x4, 8x8 and 16x16 quantization matrices) that have been described in Section 2.1. The test case is “Symmetry1” as recommended in CE4. Test 2a uses linear interpolation for up-sampling from 16x16 to 32x32; Test 2b uses repetition for up-sampling from 16x16 to 32x32.

Table 1 Total bit, MaxError and AvgError for Test 1 and Test 2.

| Symmetry1 | Test 1 | Test 2a | Test 2b |
| --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 |
| Matrix [0][0] Bit | 79 | 79 | 79 |
| Matrix [0][1] Bit | 79 | 79 | 79 |
| Matrix [0][2] Bit | 79 | 79 | 79 |
| Matrix [0][3] Bit | 81 | 81 | 81 |
| Matrix [0][4] Bit | 81 | 81 | 81 |
| Matrix [0][5] Bit | 81 | 81 | 81 |
| Matrix [1][0] Bit | 335 | 335 | 335 |
| Matrix [1][1] Bit | 2 | 2 | 2 |
| Matrix [1][2] Bit | 335 | 335 | 335 |
| Matrix [1][3] Bit | 291 | 291 | 291 |
| Matrix [1][4] Bit | 291 | 291 | 291 |
| Matrix [1][5] Bit | 291 | 291 | 291 |
| Matrix [2][0] Bit | 0 | 1143 | 1143 |
| Matrix [2][1] Bit | 0 | 2 | 2 |
| Matrix [2][2] Bit | 0 | 2 | 2 |
| Matrix [2][3] Bit | 0 | 825 | 825 |
| Matrix [2][4] Bit | 0 | 2 | 2 |
| Matrix [2][5] Bit | 0 | 825 | 825 |
| Matrix [3][0] Bit | 0 | 0 | 0 |
| Matrix [3][1] Bit | 0 | 0 | 0 |
| Total Bit | 2026 | 4825 | 4825 |
| MaxError | 19 | 9 | 9 |
| AvgError | 1.998031 | 0.493110 | 0.452264 |

Table 2 and Table 3 reports the BD-rates for above Test 1 and Test 2 compared with CE4 subset2 anchor, respectively. For input quantization matrices, “symmetry1” was used as released and recommended in CE4. Linear interpolation is used for up-sampling.

Table 2 BD-rate for the proposed method compared with CE4 subset2 anchor, using “symmetry1” input.

Table 3 BD-rate for the proposed method compared with CE4 subset2 anchor, using “symmetry1” input.

Table 5 and Table 6 report the number of bits spent on coding quantization matrices, maximum and average error for the method 1 that has been described in Section 2.2, and the method 2 that has been described in Section 2.2, respectively. The test cases are recommended in CE4.

Table 5 Total bit, MaxError and AvgError for up-sampling method 1 in 2.2.

Table 6 Total bit, MaxError and AvgError for up-sampling method 2 in 2.3.

# Conclusion

This contribution reports the studies and results about methods for handling large size quantization matrices. Experimental results reportedly show that in the worst case (i.e. quantization matrices are updated each picture) by skipping coding 16x16 and 32x32 quantization matrices, the average BD-rate savings are 4.6% AI-HE, 22.1% RA-HE and RA-LC, 28.8% for LB-HE, compared with current anchor. By skipping coding only 32x32 quantization matrices, the average BD-rate savings are 3.2% AI-HE, 14.7% RA-HE and RA-LC, 20.6% for LB-HE, compared with current anchor. It is also proposed to use different methods for deriving 16x16 and 32x32 quantization matrices, and/or treating low and high frequency coefficients.

# References

- CE4 subset2.2 anchor software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0-dev-qmatrix.

- X. Zhang, S. Liu and S. Lei, “Method and syntax for generating large quantization matrices and non-square quantization matrices”, JCTVC-G152, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).