| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0388 |
| --- | --- |

| Title: | High level syntax hooks for future extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Wonkap Jang, Danny Hong, Stephan Wenger 433 Hackensack Ave Hackensack, NJ 07601 USA Ye-Kui Wang, Ying Chen Qualcomm USA | Tel: Email: | +1 201 478 6245 {jill,wonkap,danny,stephan}@ vidyo.com {yekuiw,cheny}@qualcomm.com |
| Source: | Vidyo, Qualcomm |  |  |

_____________________________

# Abstract

Hooks to the high-level syntax of the HEVC design are proposed to enable future scalable and 3DV extensions in a backwards-compatible manner, maintaining the existing temporal scalability capability.

The NAL unit header design is revised and a Video Parameter Set (VPS) is proposed. A 5-bit layer_id field is added to the NAL unit header, while maintaining a 2-byte NAL unit header length, fixed for both VCL and non-VCL NAL units, allowing Media Aware Network Elements (MANEs) to perform bitstream adaptation based on either or both of layer_id and temporal_id.

An example design that support both scalability and 3DV is provided for information in JCTVC-H0386, to illustrate how the proposed high-level syntax hooks enable future scalable and 3DV extensions in a way that is backwards-compatible with the base HEVC specification.

# Motivation

If we follow the design principle for AVC parameter sets design, wherein SPS includes the parameters that are common for all slices in the entire coded video sequence (CVS), and PPS includes the parameters that are common for all slices in one picture and are very likely to be shared by all slices in multiple pictures, it is natural to have a new type of parameter set (let’s call it video parameter set, VPS) that is one level above SPS in the context of scalability and/or 3DV. Then,

- VPS  includes the parameters that are common for all slices across all (scalability or view) layers in the entire CVS

- SPS includes the parameters that are common for all slices in a particular (scalability or view) layer in the entire CVS, and may be shared by multiple (scalability or view) layers

- PPS includes the parameters that are common for all slices in a particular layer representation (the representation of one scalability or view layer in one access unit) and are very likely to be shared by all slices in multiple layer representations

In our understanding, the reason why VPS was not included in the SVC extension of AVC was mainly because such a design would break backward compatibility with the base AVC specification.  The SVC “hack” results in an unclear/unclean architecture, a lot of SPS parameters have to be redundantly sent, and a very complicated SPS activation process (refer to the many and long paragraphs for SPS activation process in Annex G and Annex H of the AVC specification).  At the present moment in the HEVC standardization process, there is a great opportunity to avoid the above for HEVC and its future extensions.

There are also some shortcomings associated with the existing SVC and MVC extensions related to the NAL unit header design.  The NAL unit header for plain AVC VCL NAL units is one byte, while the length is four bytes for SVC and MVC NAL units, in different syntax structures.  As a result of the non-uniform NAL unit header design, the sub-bitstream extraction processes in both SVC and MVC are complicated and there is no guarantee to get clean sub-bitstreams without NAL units that are not useful at all.  Again, at the present moment in the HEVC standardization process, there is a great opportunity to make a clean NAL unit header design with a fixed length NAL and uniform syntax structure for HEVC and its future extensions.

# Proposal

## General terminology

A bitstream contains one or more layers.  A layer is a spatial, quality, view, or depth layer.  Each layer may refer separately to a sequence parameter set.  Temporal layers are not considered to be layers, instead there are temporal sub-layers contained within a layer.  Each layer is uniquely identified by its layer_id.  Bitstreams compliant with the current non-scalable HEVC design have only one layer.

Each layer is either a dependent or independent layer.  A dependent layer is decoded depending upon at least one reference layer, which itself may have further dependencies.  An independent layer is decoded independently of other layers, e.g. an independent layer is a base layer or a simulcast layer.

Each layer has an associated layer set, which includes that layer plus any reference layers which that layer is dependent upon, directly or indirectly, if any.  A layer set contains exactly one independent layer and zero or more dependent layers.

A sub-bitstream is defined similarly as in SVC.  A sub-bitstream is derived as a subset from a bitstream by discarding a number of NAL units.  A sub-bitstream contains one or more independent layers and zero or more dependent layers.

## NAL unit header

A reserved_zero_5bits syntax element is added to the second byte of the NAL unit header, using the existing reserved bits and the bit formerly used for output_flag.  It is intended that in a future extension this syntax element be renamed layer_id.  A MANE may use the value of the layer_id field for bitstream adaptation, in addition to the temporal_id value already present in the NAL unit header.

In the VPS, as showing in the example design, a mapping can be provided of the possible layer_id values to scalability and 3DV parameters, including dependency_id, quality_id, view_id, and depth_flag, which a more complicated MANE could use for bitstream adaptation.

5 bits are provided for the layer_id syntax element.  The value of layer_id shall be equal to 0 for bitstreams conforming to profiles to be defined in the HEVC base specification.  In the example design for future HEVC scalable and 3DV extensions, the value of 31 for layer_id is reserved for further future extensions.  Layers with higher values of layer_id are expected to be of higher quality or provide greater functionality, e.g. additional views.

The output_flag syntax element is removed from the NAL unit header, and is proposed to be moved to an SEI message, slice header, or the access unit delimiter.

Both the temporal_id and layer_id are proposed to be present in both VCL NAL units and non-VCL NAL units, to allow for a simple bitstream extraction process.

The proposed NAL unit header syntax and semantics are as follows.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| reserved_one_4bits | u(4) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| temporal_id | u(3) |
| reserved_zero_5bits | u(5) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 2 |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

temporal_id specifies a temporal identifier for the NAL unit. The value of temporal_id shall be the same for all NAL units of an access unit.  When an access unit contains any NAL unit with nal_unit_type equal to 4 or 5, temporal_id shall be equal to 0. When nal_unit_type equal to XX (VPS), or 7 (SPS), temporal_id shall be equal to 0.

reserved_zero_5bits shall be equal to 0.  Decoders shall ignore the value of reserved_zero_5bits.

## Video Parameter Set

In future HEVC scalable and 3DV extensions, the VPS is supposed to provide information about the dependency relationships of the layers in a bitstream, as well as many other information that are applicable to all slices across all (scalability or view) layers in the entire coded video sequence.  It would contain some of the information that was present in the SVC Scalability Information SEI message, but unlike the SEI message, the VPS is normative and required.  The presence of the VPS would be required for all bitstreams, even those that do not make use of the HEVC extension features.  This allows for backwards compatibility of future extensions.  The burden on encoders and decoders is minimal, as a VPS which indicates that no extension layers are present will contain very few bits that are not needed for the base layer.  Any profile that restricts the number of layers in the VPS to one will not require any special decoding process beyond parsing the VPS and the GPS.

In the VPS in the example design for future HEVC scalable and 3DV extensions in JCTVC-H0386, for each possible layer_id value, values are specified of dependency_id, quality_id, view_id, and depth_flag for the layer.  Profile and level information for each layer are also included in the VPS, as well as the profile and level for each temporal sub-layer of each layer.

At most one VPS may be active at a time.  A VPS is activated when it is not already active and it is referred by a coded slice NAL unit in a particular layer in an IDR access unit being decoded.  The VPS must be available before the NAL unit that activates the VPS.  Once activated, the VPS applies to the entire coded video sequence.  In other words, the active VPS can only change at an IDR access unit.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| max_temporal_layers_minus1 | u(3) |
| max ccess unit delimeter. uture extension this syntax element be renamed layer_id. hat refers to aps_id. ref_aps_flag, and ref__layers_minus1 | u(5) |
| for ( i = 0; i <= num_temporal_layers_minus1; i++ ) { |  |
| profile_idc[ i ] | u(8) |
| reserved_zero_8bits[ i ] /* equal to 0 */ | u(8) |
| level_idc[ i ] | u(8) |
| } |  |
| video_parameter_set_id | ue(v) |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream.

max_layers_minus1 plus 1 specifies the maximum number of layers that may be present in the coded video sequence.  num_layers_minus1 shall be equal to 0. [Ed. This could be renamed reserved_zero_5bits.]

profile_idc[ i ], reserved_zero_8bits[ i ], and level_idc[ i ] specify the set of profile and level information that applies to the temporal sub-layer set with the greatest temporal_id equal to i.

video_parameter_set_id identifies a video parameter set. The value of view_parameter_set_id shall be in the range of 0 to 7, inclusive.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

+

## Sequence parameter set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies the video parameter set that is referred to by the sequence parameter set. The value of video_parameter_set_id shall be in the range of 0 to 7, inclusive.

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for sps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for sps_extension_flag in a sequence parameter set NAL unit.

sps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

Other syntax elements have the same semantics as the syntax elements with the same names in the SPS of HEVC WD5.

## Picture parameter set

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_extension_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. pps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for pps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for pps_extension_flag in a sequence parameter set NAL unit.

pps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

# Discussion

The existing WD design includes temporal scalability, including the presence of the temporal_id syntax element in the NAL unit header.  This proposal treats temporal layers differently from the other types of scalable layers, allowing for less change in the current WD design.  Any single layer profiles defined as part of the initial version of the standard will include support for temporal scalability, and enable backwards compliancy for extensions with only a negligible impact on the design complexity.  The only additional complexity would be the creation of the VPS by the encoder, and the parsing of the VPS by the decoder.

This proposal maintains the existing two byte length of the NAL unit header both for a single layer design, and for future scalable and 3DV extensions.  Maintaining a fixed length header is desirable for simplifying MANE operation.   Keeping the NAL unit header as short as possible is obviously advantageous.

It is anticipated that most scalable or 3DV extensions will not make full use of the maximum number of layers in each of the possible dimensions.  JCTVC-H0386 provides a possible VPS design for a scalable extension combining the dependency_id, quality_id, view_id, and depth_flag into a single layer_id lookup table for bitrate efficiency, keeping the NAL unit header short.  An industry forum aimed at a particular application may choose to restrict the layer_id values to a direct mapping of a subset of those parameters.  For example, a scalable video conferencing system may choose to allocate the 5 layer_id bits as 2 bits for dependency_id and 3 bits for quality_id.

The VPS provides sequence-level cross-layer information, and is expected to not change frequently.  The encoder can set the VPS parameters to indicate the worst-case scalability capabilities that will be used in the coded sequence, in terms of number of layers, types of layers, and number of layer dependencies.

# Patent rights declaration(s)

Vidyo Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).