| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0388 |
| --- | --- |

| Title: | High level syntax hooks for future extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Wonkap Jang, Danny Hong, Stephan Wenger 433 Hackensack Ave Hackensack, NJ 07601 USA Ye-Kui Wang, Ying Chen Qualcomm USA | Tel: Email: | +1 201 478 6245 {jill,wonkap,danny,stephan}@ vidyo.com {yekuiw,cheny}@qualcomm.com |
| Source: | Vidyo, Qualcomm |  |  |

Adopted text modifications below.

- NAL unit header syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| reserved_one_4bits | u(4) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 2 |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

- NAL unit header semantics

temporal_id specifies a temporal identifier for the NAL unit. The value of temporal_id shall be the same for all NAL units of an access unit.  When an access unit contains any NAL unit with nal_unit_type equal to 4 or 5, temporal_id shall be equal to 0. When nal_unit_type equal 7 (SPS), temporal_id shall be equal to 0.

reserved_one_5bits shall be equal to '00001'.  When reserved_one_5bits not equal to ‘00001’in the NAL unit, it shall be discarded by the decoding process.

- PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| chroma_cb_qp_offset | se(v) |
| chroma_cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| tile_info_present_flag | u(1) |
| … |  |

- PPS semantics

output_flag_present_flag equal to 0 specifies that output_flag is not present in the slice header. output_flag_present_flag equal to 1 specifies that output_flag is present in the slice header.

- slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| output_flag | u(1) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| … |  |

- slice header semantics

output_flag affects the decoded picture output and removal processes as specified in Annex Error: Reference source not found.