| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA 01-10 February, 2012 | Document: JCTVC-H0391 M23266 |
| --- | --- |

| Title: | AHG15: Slice-Level Control of In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ranga Ramanujam Srinivasan Chaitanya Ghone | Tel: Email: Tel: Email: | ramanujam@ti.com csghone@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Current draft of HEVC and HM5.0 code supports turning off the loop-filter across all slices in a picture. However, for better video quality, it is desirable to turn off loop-filter across few slices only. One such scenario is “Gradual Decoder Refresh”. Here, the video data is “refreshed” (compression is done independent of availability of previous frames) one slice at a time. In this case, it is desirable to turn off loop-filter only at the slice which is next to a “refreshed” slice, so that “refreshed” slice does not depend on “non-refreshed” data. Loop-filtering across slices should be allowed for all other data so as to not compromise on video quality. This is not possible as per current draft of HEVC. This contribution proposes a syntax change in the HEVC standard to allow for control of loop-filter to be made available at each slice.

# Introduction

In current working-draft and HM 5.0 code, there is no Slice-level control to enable/disable In-Loop Filtering across slice-boundary. There is a specific need for this for the implementation of Gradual Decoder Refresh. This contribution gives change in syntax required for Slice-Level Control of In-Loop Filtering in HEVC.

# Proposed Change in Syntax

## Current Syntax

Summary of In-Loop filter related syntax present in HM5.0 is given in Table 1.

| Loop Filter Control Syntax | Header | Values | Details |
| --- | --- | --- | --- |
| sample_adaptive_offset_enabled_flag | SPS | 0/1 | Disable/Enable |
| adaptive_loop_filter_enabled_flag | SPS | 0/1 | Disable/Enable |
| loop_filter_across_slice_flag | SPS | 0/1 | Disable/Enable |
| aps_sample_adaptive_offset_flag | APS | 0/1 | Disable/Enable |
| aps_adaptive_loop_filter_flag | APS | 0/1 | Disable/Enable |
| disable_deblocking_filter_flag | Slice Header |  |  |

Table 1: Current In-Loop Filter Syntax

### Sequence Parameter Set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| .. | .. |
| .. |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| .. |  |
| } |  |

### Sequence parameter set RBSP semantics

loop_filter_across_slice_flag equal to 1 specifies the in-loop filtering operations are performed across slice boundary; otherwise, the in-loop operations are slice-independent and not across slice boundary. The in-loop filtering operations include de-blocking filter, sample adaptive offset, and adaptive loop filter

### Slice-Header RBSP syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| .. |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta |  |
| disable_deblocking_filter_flag | u(1) |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| .. |  |

### Slice-Header RBSP semantics

disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall be disabled and disable_deblocking_filter_flag equal to 0 specifies that the operation of the de-blocking filter shall be enabled.

## Proposed Syntax

loop_filter_across_slice_flag in SPS and disable_deblocking_filter_flag in Slice-Header are removed and two new syntax elements, inloop_filter_control_present_flag in SPS and inloop_filter_idc in Slice-Header are added. Proposed syntax is listed in Table 2.

| Loop Filter Control Syntax | Header | Values |  |
| --- | --- | --- | --- |
| sample_adaptive_offset_enabled_flag | SPS | 0/1 | Disable/Enable |
| adaptive_loop_filter_enabled_flag | SPS | 0/1 | Disable/Enable |
| aps_sample_adaptive_offset_flag | APS | 0/1 | Disable/Enable |
| aps_adaptive_loop_filter_flag | APS | 0/1 | Disable/Enable |
| inloop_filter_control_present_flag | SPS | 0/1 | Disable/Enable. Indicates if inloop_filter_idc is present in slice-header |
| inloop_filter_idc | Slice Header | 0,1, 2, 3 |  |

Table 2: Proposed In-Loop Filter Syntax

inloop_filter_idc is present in Slice-Header only when inloop_filter_control_present_flag is 1 in SPS. If not present, value of inloop_filter_idc is considered as 0. Further explanation of inloop_filter_idc is given in Table 3.

| inloop_filter_idc | Details |
| --- | --- |
| 0 | De-block Enabled, SAO/ALF enabled based on signaling in SPS |
| 1 | De-Block, SAO and ALF are disabled |
| 2 | Filtering (De-Block, SAO and ALF) disabled on Slice-Boundary for that Slice |
| 3 | Disable De-Block Filter SAO/ALF enabled based on signaling in SPS |

Table 3: inloop_filter_idc details

### Sequence Parameter Set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| … | … |
| … | … |
| chroma_pred_from_luma_enabled_flag | u(1) |
| inloop_filter_control_present_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| … | … |
| } |  |

### Sequence parameter set RBSP semantics

| inloop_filter_control_present_flag enables/disables the slice-level control of in-loop filtering operations. |  |
| --- | --- |
| 0 | Specifies that inloop_filter_idc is not present in slice-header. The value of inloop_filter_idc is taken to be 0 in this case. |
| 1 | Specifies that inloop_filter_idc is present in slice-header |

### Slice-Header RBSP syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| .. |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta |  |
| if(inloop_filter_control_present_flag ) |  |
| inloop_filter_idc | ue(v) |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| .. |  |

### Slice-Header RBSP semantics

| inloop_filter_idc controls in-loop filter operations for the current slice. In-loop filtering operations include de-blocking filter, sample adaptive offset, and adaptive loop filter. |  |
| --- | --- |
| 0 | De-blocking filter is enabled. Sample-Adaptive Offset(SAO) and Adaptive Loop Filter(ALF) are enabled based on sample_adaptive_offset_enabled_flag and adaptive_loop_filter_enabled_flag |
| 1 | All in-loop filtering is disabled |
| 2 | In-loop filter operations are slice-independent (No filtering shall occur across slices). SAO/ALF operations shall be turned on/off as per sample_adaptive_offset_enabled_flag and adaptive_loop_filter_enabled_flag, but shall not work across slices. |
| 3 | De-blocking filter is disabled. SAO and ALF are enabled based on sample_adaptive_offset_enabled_flag and adaptive_loop_filter_enabled_flag. |

# Test Settings and Conditions

The simulations of this document have used HM5.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

For this proposal, the meaning of LoopFilterDisable parameter in the encoder configuration file has been changed and an additional parameter called DeblockDisable is added to provide equivalent functionality.  Details of this change are tabulated in Table 4 below.

| Parameter | HM 5.0 Definition | New Definition (Modified HM5.0) |
| --- | --- | --- |
| LoopFilterDisable | Disable Deblock Filter | Disable all inloop filters (Deblock, SAO and ALF) |
| DeblockDisable | Not Present | Disable Deblock Filter |

Table 4: Configuration Parameter Definition Changes

The test configuration changes for low-delay configurations when compared to the base settings are listed in Table 5 below.

| Parameter | Base Value for original HM5.0 | Value for original HM5.0 (As per old definition) | Value for modified HM5.0 (As per new definition) |
| --- | --- | --- | --- |
| LoopFilterDisable | 0 | 1 | 0 |
| DeblockDisable | NA | NA | 1 |

Table 5: Test Parameter Changes

# Experimental Results

The simulation results using HM5.0 indicate zero change in BDRATE for all configurations explained in the test-conditions in section 3.

# Conclusions

The proposed syntax change provides the flexibility for slice-level control of In-Loop filter without any loss in coding efficiency.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).