| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA 01-10 February, 2012 | Document: JCTVC-H0392 M23267 |
| --- | --- |

| Title: | AHG 15: Enable picture-level CU/TU size signaling in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naveen Srinivasamurthy Keshava Prasad Yashwant Dutt Minhua Zhou | Tel: Email: Tel: Email: | snaveen@ti.com prasadk@ti.com yashwant.dutt@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents a technique to allow flexibility in signaling Max CU and TU size in bitstream. CU and TU tree size is indicated in the syntax through sequence parameter set. This means that inter and intra frames need to have the same minimum and maximum CU and TU size. Often, in DSP and HWA implementation, one may like to use different CU and TU tree for inter and intra frames for performance and implementation reasons. In this submission, it is proposed that the syntax elements which control CU and TU tree to be conditionally sent in PPS also.

# Introduction

Currently, the CU and TU tree depths are signaled in SPS. This means that the maxCU size and maxTU size has to be same for the whole sequence. The syntax elements which represent the CU and TU tree limit in the sequence are represented below. [Hatched with yellow (excerpts from working draft (JCTVC-G1103-v2.zip))].

[Excerpts from WD]

7.3.2.1	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| : |  |
| : |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| : |  |
| : |  |

Table 1: Sequence parameter set

Corresponding semantics for the above is stated below. [Excerpts from WD]

7.4.2.1	Sequence parameter set RBSP semantics

log2_min_coding_block_size_minus3 specifies the minimum size of a coding block. The variable Log2MinCUSize is set equal to log2_min_coding_block_size_minus3 + 3. log2_diff_max_min_coding_block_size specifies the difference between the maximum and minimum coding block size. The variable Log2MaxCUSize is set equal to log2_min_coding_block_size_minus 3 + 3 + log2_diff_max_min_coding_block_size. log2_min_transform_block_size_minus2 specifies the minimum size of a transform block. The variable Log2MinTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2. log2_diff_max_min_transform_block_size specifies the difference between the maximum and minimum transform size. The variable Log2MaxTrafoSize is set equal to log2_min_transform_block_size_minus 2 + 2 + log2_diff_max_min_transform_block_size. The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Log2MaxCUSize.

This means that the CU and TU tree sizes (min to max) is fixed for all pictures of the sequence. While doing implementation in DSP or HWA, one may consider having different CU and TU tree for intra and inter pictures for various implementation and performance reasons. In current syntax structure, this flexibility is not there. MaxCU size or MaxTU size once chosen has to be same for all intra and inter pictures.

# Proposed Solution

If we make the syntax indicating the CU and TU tree conditionally duplicated in PPS, it will solve the problem. This is because we can have different PPS for different pictures. Hence, this will allow inter and intra pictures to have different Max CU and TU size.

This will help in the below cases

- Allow flexibility to codec implementation which are based on DSP or HWA

- Allow better control on codec delays as Max CU size can be controlled separately for intra and inter pictures

The above proposal will just introduce 1 extra bit in the whole sequence and will have no impact on coding gains.

# Changes in Standard

Following are the changes required in the signaling of PPS.

Syntax modification in the working draft

7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| : |  |
| : |  |
| cu_tu_size_present_flag | u(1) |
| if (cu_tu_size_present_flag == 1) |  |
| { |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| } |  |
| : |  |
| : |  |

Table 2: Picture parameter set

Semantics modification in the working draft

7.4.2.2	Picture parameter set RBSP semantics

cu_tu_size_present_flag specifies if log2_min_coding_block_size_minus3, log2_diff_max_min_coding_block_size, log2_min_transform_block_size_minus2 and log2_diff_max_min_transform_block_size are present in the PPS or not. If the value is set to 1, the slice should override the value it gets in SPS and use the value coming in PPS instead. If the value is set to 0, the corresponding sizes coming in SPS should be used. log2_min_coding_block_size_minus3 specifies the minimum size of a coding block. The variable Log2MinCUSize is set equal to log2_min_coding_block_size_minus3 + 3. log2_diff_max_min_coding_block_size specifies the difference between the maximum and minimum coding block size. The variable Log2MaxCUSize is set equal to log2_min_coding_block_size_minus 3 + 3 + log2_diff_max_min_coding_block_size. log2_min_transform_block_size_minus2 specifies the minimum size of a transform block. The variable Log2MinTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2. log2_diff_max_min_transform_block_size specifies the difference between the maximum and minimum transform size. The variable Log2MaxTrafoSize is set equal to log2_min_transform_block_size_minus 2 + 2 + log2_diff_max_min_transform_block_size. The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Log2MaxCUSize.

This results in adding 1 extra bit in PPS in case a codec implementation does not plan to use this feature.

# Test Settings and Conditions

<NA>

# Experimental Results

<NA>

# Conclusions

The proposed duplication of syntax elements related to CU and TU tree signaling allows flexibility for better implementation of HEVC without compromising on coding gain.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).