| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0396 |
| --- | --- |

| Title: | CE10 Subtest 3: Deblocking Filter Simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the simplification of the filters used for in-loop deblocking. In the first part, the luma weak filter delta is simplified, saving one multiplication. In addition, the second part proposes to replace the HM5 strong filter with the current HM5 weak filter, because the strength of both filters is close. Together with a modified strong/weak filter decision, the line memory requirement is reduced from four lines to three. Further simplification is offered by unification of the luma weak filter and chroma filter deltas. Overall, the BD-rate results, subjective quality, and execution times are similar to the HM5 anchor.

# Introduction

The following provides an overview of the luma and chroma deblocking filters in HM5 [1]:

- Luma strong filter:

p0’ =  ( p2 + 2p1 + 2p0 + 2q0 + q1 + 4 )/8

p1’ =  ( p2 + p1 + p0 + q0 + 2 )/4

p2’ =  ( 2p3 + 3p2 + p1 + p0 + q0 + 4 )/8

q0’ =  ( p1 + 2p0 + 2q0 + 2q1 + q2 + 4 )/8

q1’ =  ( p0 + q0 + q1 + q2 + 2 )/4

q2’ = ( p0 + q0 + q1 + 3q2 + 2q3 + 4 )/8

- Luma weak filter:

 =  ( 9(q0-p0) - 3(q1-p1) + 8 )/16

 = Clip( -tc, tc,  )          (tc is threshold)

p0’ = p0 + 

q0’ = q0 - 

p = Clip( - tc/2, tc/2, ( (p2 + p0 + 1)/2 - p1 + )/2 )

p1’ = p1 + p      (modification of p1 depends on condition)

q = Clip( - tc/2, tc/2, ( (q2 + q0 + 1)/2 - q1 - )/2 )

q1’ = q1 + q      (modification of q1 depends on condition)

- swi=2p2i−2p1i+p0i+q0i−2q1i+q2i<β4 and p3i−p0i+q0i−q3i<β8 and p0i−q0i<5⋅tc+12Strong/weak filter decision for luma edge:

  - IF (sw0 AND sw3) THEN “Strong filter” ELSE “Weak filter” for 4-sample edge segment (i=0…3)

  - IF (sw4 AND sw7) THEN “Strong filter” ELSE “Weak filter” for 4-sample edge segment (i=4…7)

- Chroma filter:

 =  ( 4(q0-p0) - (q1-p1) + 4 )/8

 = Clip( -tc, tc,  )

p0’ = p0 + 

q0’ = q0 - 

# Proposed Simplifications

## Part 1: Luma Weak Filter Delta Simplification

Modified coefficients are proposed for computation of the HM5 weak filter delta, saving one multiplication (×3):

 =  ( 3(q0-p0) - (q1-p1) + 4 )/8

In addition, it is proposed to make the chroma filter delta identical to the simplified luma weak filter delta.

## Part 2: Replace Strong Filter with HM5 Weak Filter

In addition to part 1, it is proposed to replace the HM5 strong filter with the HM5 “weak filter” (modifying two samples on each side of the edge), which has a smoothing strength that is quite close to the HM5 strong filter (see next section). It is also proposed to modify the strong/weak filter decision. Both the modified strong/weak filter decision and the strong filter read three samples on both sides of the edge, instead of four, reducing the line memory buffer requirement from four lines to three lines. In addition, the new strong filter is computationally simpler than the HM5 strong filter.

- Proposed new strong filter, which modifies two samples on each side of the edge:

 = ( 9(q0-p0) - 3(q1-p1) + 8 )/16

 = Clip( -tc, tc,  )

p0’ = p0 + 

q0’ = q0 - 

p = Clip( - tc/2, tc/2, ( (p2 + p0 + 1)/2 - p1 + )/2 )

p1’ = p1 + p

q = Clip( - tc/2, tc/2, ( (q2 + q0 + 1)/2 - q1 - )/2 )

q1’ = q1 + q

- Modified strong/weak filter decision, which reads three samples on each side of the edge:

swi=2p2i−2p1i+p0i+q0i−2q1i+q2i<β4 and p2i−p0i+q0i−q2i<β8 and p0i−q0i<5⋅tc+12

  - IF (sw0 AND sw3) THEN “Strong filter” ELSE “Weak filter” for 4-sample edge segment (i=0…3)

  - IF (sw4 AND sw7) THEN “Strong filter” ELSE “Weak filter” for 4-sample edge segment (i=4…7)

# Filter Characteristics

Figure 1(a) depicts a step profile or edge discontinuity to be deblocking filtered either with the HM5 strong filter or the weak filter, depending on the strong/weak decision. Since both sides of the edge are smooth and flat, it is the third part of the strong/weak decision, the edge discontinuity check, which decides between strong and weak filtering. Figure 1(b) depicts the filtering result with the HM5 strong and weak filters. For the weak filter, two samples are modified on both sides of the edge because of the smoothness on both sides of the edge. This comparison demonstrates that there is not much difference in filtering strength between the HM5 strong and weak filters for the step profile. Figure 1(c) illustrates the filtering result with the proposed weak filter, which demonstrates that this filter smooths the edge discontinuity, but does not make it into a linear slope. This characteristic is desirable for a weak filter, because it preserves texture details as is illustrated in section 4.2.

Figure 2(a) also illustrates a step profile or edge discontinuity to be deblocking filtered. Figure 2(b) illustrates the result after applying the HM5 luma weak filter modifying one sample on each side of the edge. It can be observed that the HM5 luma weak filter overshoots the linear slope, which means that the delta value is too large. Figure 2(c) contains the HM5 chroma filtering result. As for the HM5 weak filter, it is observed that the chroma filter overshoots the linear slope and, therefore the chroma delta is also too large. When the delta overshoots the linear slope, there is potentially a new visual artifact. Figure 2(d) illustrates the filtering result for the proposed chroma filter, which does not overshoot the linear slope.

Figure 1 (a) Step profile (edge discontinuity). (b) HM5 strong and weak filtering of step profile. (c) Filtering result with proposed weak filter.

Figure 2 (a) Step profile or edge discontinuity. (b) HM5 luma weak filtering result for step profile in case one sample is modified on each side of the edge. (c) HM5 chroma filtering result for the step profile. (d) Filtering result with proposed chroma filter.

# Results

## Objective Results

The following BD-rate results are obtained according to the common test conditions in JCTVC-G1200. The execution times are measured in a heterogeneous computing environment and are approximate.

### Part 1

Table 1 BD-rate results for Part 1

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.1% | 0.1% | -0.2% | 0.0% | 0.2% |  |  |  |
| Class B | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% |  |  |  |
| Class C | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% |  |  |  |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class E | -0.1% | 0.3% | 0.3% | -0.1% | 0.9% | 0.7% |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% |  |  |  |
|  | -0.2% | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.3% | -0.2% | -0.5% | -0.2% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.1% |
| Class F | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% | 0.3% |  |  |  |
| Class C | -0.1% | -0.1% | -0.2% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | -0.1% | -0.3% | -0.1% | -0.2% | 0.1% | 0.2% |  |  |  |
| Class E | -0.1% | 0.2% | -0.2% | -0.1% | 0.7% | 0.8% |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 0.3% |  |  |  |
|  | -0.2% | 0.0% | -0.1% | -0.1% | 0.2% | 0.3% |  |  |  |
| Class F | -0.1% | -0.2% | -0.5% | 0.0% | -1.0% | -0.6% |  |  |  |
| Enc Time[%] | 97% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 97% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | 0.0% | 0.4% | 0.3% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | -0.3% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class D | -0.1% | 0.1% | -0.8% | 0.0% | -0.1% | 0.6% |  |  |  |
| Class E | -0.1% | -0.3% | -0.4% | 0.6% | 0.4% | 0.4% |  |  |  |
| Overall | -0.1% | 0.0% | -0.4% | 0.3% | 0.2% | 0.3% |  |  |  |
| Class F | 0.1% | 0.2% | 0.1% | 0.2% | -0.3% | -0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

### Part 2

Table 2 BD-rate results for Part 2

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.1% | 0.0% | -0.2% | 0.0% | 0.2% |  |  |  |
| Class B | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% |  |  |  |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class E | -0.1% | 0.3% | 0.3% | 0.0% | 0.9% | 0.7% |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |  |  |  |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% |  |  |  |
| Class F | -0.6% | -0.1% | 0.0% | -0.5% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | -0.3% | -0.4% |
| Class B | -0.1% | -0.1% | 0.0% | 0.0% | 0.2% | 0.1% | -0.1% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class D | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.2% |
|  | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.2% |
| Class F | -0.3% | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% | 0.0% | -0.4% | -0.2% |  |  |  |
| Class C | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | -0.1% | -0.1% | -0.1% | -0.2% | 0.4% | -0.1% |  |  |  |
| Class E | -0.1% | 0.3% | -0.1% | 0.2% | 0.4% | 0.5% |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | -0.1% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | -0.4% | -0.5% | -0.3% | -0.5% | -0.9% | -0.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.2% | 0.7% | 0.0% | -0.6% |  |  |  |
| Class C | 0.0% | 0.0% | -0.2% | 0.3% | 0.0% | 0.2% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | -0.7% | 0.4% |  |  |  |
| Class E | 0.0% | 0.0% | 0.4% | 1.0% | 0.7% | 0.0% |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.5% | 0.0% | 0.0% |  |  |  |
| Class F | -0.3% | 0.0% | -0.1% | -0.3% | -1.0% | -0.3% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

## Subjective Quality

The result of informal subjective testing is that the quality of parts 1 and 2 is in some cases better than the anchor (less smooth, improved texture detail). Overall, the subjective quality is similar to the anchor.

### Part 1

Figures 3-4 illustrate examples of the subjective quality enhancements of part 1 of this proposal.

| - HM5.0 | - Proposal part 1 |
| --- | --- |

Figure 3 Frame 240 from the class B “Kimono” sequence (Low delay P LC, QP=37). Enhanced texture detail is shown.

| - HM5.0 | - Proposal part 1 |
| --- | --- |

Figure 4 Frame 214 from class C “BQMall” sequence (Low delay P LC, QP=37). Improved texture detail and reduction of trailing edge is shown.

### Part 2

Figures 5-6 illustrate examples of the subjective quality enhancements of part 2 of this proposal.

| - HM5.0 | - Proposal part 2 |
| --- | --- |

Figure 5 Frame 240 from the class B “Kimono” sequence (Low delay P LC, QP=37). Enhanced texture detail is demonstrated.

| - HM5.0 | - Proposal part 2 |
| --- | --- |

Figure 6 Frame 600 from class E “Vidyo3” sequence (Low delay P LC, QP=37).

# Conclusion

Part 1 of this contribution addresses the simplification of the weak filter delta. In addition, part 2 proposes to replace the HM5 strong filter with the HM5 weak filter and to modify the strong/weak filter decision, so that the line memory for in-loop deblocking can be reduced from four to three lines. The unification of the luma weak filter delta and the chroma filter delta is also proposed. In summary, this proposal reduces the computational complexity of the filters for deblocking and reduces the required line memory. Overall, the objective results and the subjective quality are similar to the HM5 anchor.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” 7th JCT-VC Meeting, Geneva, CH, Nov. 2011, Doc. JCT-VC G1103_d6

# Working Draft Text Modifications

The following working draft changes are based on [1].

## Part 1: Luma Weak Filter Delta Simplification

“8.6.1.4.5 Filtering process for a luma sample

…..

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4

 = ( 3* ( q0 –  p0 ) – ( q1 – p1 ) + 4 ) >> 3

.....”

“8.6.1.4.6 Filtering process for a chroma sample

…..

When the variable bS is greater than 1, the filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )

 = Clip3( -tC, tC, ( ( 3* ( q0 –  p0 ) – ( q1 – p1 ) + 4 ) >> 3) )

p0’ = Clip1C( p0 +  )

q0’ = Clip1C( q0 -  )

......”

## Part 2: Replace Strong Filter with HM5 Weak Filter

In following WD text changes are in addition to the working draft text changes for Part 1.

“8.6.1.4.5 Filtering process for a luma sample

……

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1

Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3

–	When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 2:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )

p0’ = Clip1Y( p0 +  )

q0’ = Clip1Y( q0 -  )

    - The filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )

pi’ = Clip1Y( p1 + p )

    - The filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )

qi’ = Clip1Y( q1 + q )

…..”

“8.6.1.4.4 Decision process for a luma sample

…..

When the variable bS is not equal to 0, the following applies:

–	If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	If d is less than ( β >> 2 ), | p2 – p0 | + | q0 – q2 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

…...”

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).