| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0397r1 |
| --- | --- |

| Title: | Context reduction of split_transform_flag in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In HM5.0, there are 4 contexts for split_transform_flag. This proposal reduces the number of contexts for split_transform_flag from 4 to 1, which means all contexts share one single context. Experimental results show that BD-rate changes are 0.0% for AI_HE, 0.0% for AI_LC, 0.0% for RA_HE, 0.0% for RA_LC, 0.1% for LD_HE, -0.0% for LD_LC, and 0.1% for RA_HE10.

# Solution

split_transform_flag indicates the splitting of RQT in HM5.0. There are 4 contexts for either Slice I/P/B in working draft, while there are 10 contexts in HM5.0 software. It’s proposed to reduce the number of contexts from 4 to 1 in the working draft.

# Simulation results

The proposed technique is implemented into HM5.0. The simulation is conducted under common test conditions [1]. A summary of results is shown in Table 1, and detailed results are available in “JCTVC-H0397.xls”.

Table 1 Summary of BD-rate results

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.01% | -0.05% | -0.05% | 0.01% | -0.07% | -0.10% |  |  |  |
| Class B | 0.01% | -0.01% | -0.01% | 0.01% | -0.06% | -0.07% |  |  |  |
| Class C | 0.00% | -0.04% | -0.03% | 0.00% | -0.05% | -0.08% |  |  |  |
| Class D | 0.00% | 0.05% | -0.04% | 0.00% | -0.02% | -0.02% |  |  |  |
| Class E | 0.00% | -0.08% | -0.04% | 0.00% | -0.16% | -0.14% |  |  |  |
| Overall | 0.00% | -0.02% | -0.03% | 0.01% | -0.07% | -0.08% |  |  |  |
|  | 0.00% | -0.02% | -0.02% | 0.00% | -0.07% | -0.08% |  |  |  |
| Class F | -0.01% | 0.05% | 0.03% | 0.01% | 0.15% | -0.10% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.11% | 0.03% | -0.22% | 0.06% | 0.18% | 0.03% | 0.05% | 0.04% | 0.24% |
| Class B | 0.04% | -0.02% | 0.04% | 0.04% | 0.00% | -0.17% | 0.08% | -0.03% | -0.21% |
| Class C | 0.02% | -0.14% | 0.03% | -0.01% | 0.07% | -0.15% |  |  |  |
| Class D | 0.00% | -0.13% | 0.09% | 0.02% | 0.42% | -0.11% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.03% | -0.07% | 0.02% | 0.02% | 0.15% | -0.12% | 0.07% | 0.00% | -0.01% |
|  | 0.03% | -0.09% | -0.01% | 0.02% | 0.13% | -0.14% | 0.07% | 0.00% | 0.02% |
| Class F | -0.05% | -0.19% | -0.16% | 0.06% | 0.14% | -0.04% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | 0.31% | 0.48% | 0.03% | -0.11% | 0.07% |  |  |  |
| Class C | 0.05% | -0.17% | 0.03% | -0.01% | 0.10% | 0.18% |  |  |  |
| Class D | 0.06% | 0.19% | 0.08% | -0.05% | 1.09% | 0.06% |  |  |  |
| Class E | 0.16% | 0.94% | -0.39% | -0.09% | -1.00% | -0.54% |  |  |  |
| Overall | 0.08% | 0.28% | 0.11% | -0.02% | 0.08% | -0.02% |  |  |  |
|  | 0.07% | 0.31% | 0.18% | -0.02% | 0.02% | -0.01% |  |  |  |
| Class F | 0.14% | -0.08% | 0.01% | -0.06% | -0.57% | -0.89% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

# Conclusion

This proposal reduced context number of split_transform_flag from 4 to 1 and the degradation in coding efficiency is negligible. We propose to adopt this proposal into HEVC.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

The changes in WD are shown in highlighted texts as follows.

## WD change 1

Table 6-2 – Values of variable initValue for split_transform_flag ctxIdx

| Initialisation variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 119 100 | 162 55 | 148 86 | 100 | 119 | 71 | 86 | 55 | 119 | 102 | 86 | 86 |

## WD change 2

Table 6-3 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found Table 9-21 | 0..3 0 | 4..7 1 | 8..11 2 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |

## WD change 3

| Table 6-4 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| skip_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| cu_qp_delta | I | as specified in subclause Error: Reference source not found | 2 | Error: Reference source not found | 0 |
|  | P |  | 2 | Error: Reference source not found | 3 |
|  | B |  | 2 | Error: Reference source not found | 6 |
| pred_mode_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| part_mode | I | as specified in subclause Error: Reference source not found | 0 | Error: Reference source not found | 2 |
|  | P |  | 3 | Error: Reference source not found | 3 |
|  | B |  | 3 | Error: Reference source not found | 6 |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mpm_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | as specified in subclause Error: Reference source not found | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| merge_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| merge_idx | P | TU, cMax = MaxNumMergeCand − 1 | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| ref_idx_lc | B | TU, cMax = num_ref_idx_lC_active_minus1 | 2 | Error: Reference source not found | 3 |
| ref_idx_l0 | P | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Error: Reference source not found | 0 |
|  | B |  | 2 | Error: Reference source not found | 3 |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Error: Reference source not found | 3 |
| abs_mvd_greater0_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| abs_mvd_greater1_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | P/B | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_lc_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| mvp_l0_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| mvp_l1_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| split_transform_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 1 |
|  | B |  | 0 | Error: Reference source not found | 8 2 |
| cbf_luma | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 4 |
| cbf_cb, cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| last_significant_coeff_x_prefix | I | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Error: Reference source not found | 0 |
|  | P |  | 8 | Error: Reference source not found | 30 |
|  | B |  | 8 | Error: Reference source not found | 60 |
| last_significant_coeff_y_prefix | I | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 | Error: Reference source not found | 0 |