| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0399r1 |
| --- | --- |

| Title: | Harmonization of coefficient coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal harmonizes coefficient coding for TUs with diagonal scan. The proposed solution has neat design and coding efficiency improvement compared to HM5.0. Experimental results show that coding efficiency gain is: 0.00% for AI_HE, -0.13% for AI_LC, -0.12% for RA_HE, -0.24% for RA_LC, -0.13% for LD_HE, -0.30% for LD_LC and 0.08% for RA_HE10.

# Introduction

In HM5.0, coefficient coding consists of several steps: mode dependent coefficient coding (MDCS), last non-zero coefficient coding, significance map coding and non-zero coefficient level coding. These components vary at different TU sizes as shown in Table 1.

Table 1 Coefficient coding in HM5.0

| TU sizes | Scan | LastPos | Multi-level Sig Map | Sig Map Context | level |
| --- | --- | --- | --- | --- | --- |
| 4x4 | H/V/D | 2D(x,y) | - | Position based (4x4 only) | 16 in 1D |
| 8x8 | H/V/D | 2D(x,y) | - | Position based (8x8 only) | 16 in 1D |
| 16x16/16x4/4x16 32x32/32x8/8x32 | Sub-D | 2D(x,y) | 4x4 sub-block | mask based | 4x4 sub-block |

# Proposed solution

To have a more unified solution in coefficient coding, we propose to harmonize the coefficient coding for the diagonal scan, which means that all the TUs with diagonal scan will have the same coefficient coding.

The proposed solution changes the coefficient coding for 4x4 and 8x8 TUs with diagonal scan.

- TU based diagonal scan is replaced by 4x4 sub-block based diagonal scan, which means sub-block is scanned using diagonal scan and diagonal scan is applied inside each 4x4 sub-block.

- Multi-level significance map coding is applied and all contexts reuse 16x16 and 32x32 cases.

- Significance map context definition uses mask based approach as 16x16 and 32x32 cases.

- Level coding is changed accordingly due to the scan pattern.

The solution is summarized as shown in Table 2.

Table 2 Proposed harmonized coefficient coding

| TU sizes | Scan | LastPos | Multi-level Sig Map | Sig Map Context | level |
| --- | --- | --- | --- | --- | --- |
| 4x4 | H/V | 2D(x,y) | - | Position based (4x4 only) | 16 in 1D |
| 8x8 | H/V | 2D(x,y) | - | Position based (8x8 only) | 16 in 1D |
| 4x4, 8x8 16x16/16x4/4x16 32x32/32x8/8x32 | Sub-D | 2D(x,y) | 4x4 sub-block | mask based | 4x4 sub-block |

# Simulation results

The proposed technique is implemented into HM5.0. The simulation is conducted under common test conditions [1]. A summary of results is shown in Table 3, and detailed results are available in “JCTVC-H0399r1.xls”.

Table 3 Summary of BD-rate results for common conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.15% | -0.01% | 0.08% | 0.00% | 0.14% | 0.16% |  |  |  |
| Class B | 0.07% | 0.17% | 0.10% | -0.06% | 0.06% | 0.16% |  |  |  |
| Class C | -0.11% | 0.05% | 0.04% | -0.23% | -0.11% | -0.11% |  |  |  |
| Class D | -0.14% | 0.05% | -0.06% | -0.27% | -0.14% | -0.13% |  |  |  |
| Class E | 0.10% | 0.26% | 0.23% | -0.03% | 0.28% | 0.24% |  |  |  |
| Overall | 0.00% | 0.11% | 0.07% | -0.13% | 0.03% | 0.05% |  |  |  |
|  | 0.00% | 0.11% | 0.08% | -0.14% | 0.03% | 0.04% |  |  |  |
| Class F | -0.17% | -0.22% | -0.35% | -0.25% | -0.19% | -0.22% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.02% | -0.20% | -0.28% | -0.11% | 0.09% | 0.01% | 0.14% | -0.22% | 0.03% |
| Class B | 0.01% | -0.04% | 0.09% | -0.07% | 0.05% | 0.14% | 0.06% | 0.01% | 0.01% |
| Class C | -0.24% | -0.09% | -0.05% | -0.34% | -0.20% | -0.06% |  |  |  |
| Class D | -0.25% | 0.02% | -0.05% | -0.42% | 0.55% | -0.31% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.06% | -0.03% | -0.24% | 0.12% | -0.05% | 0.10% | -0.09% | 0.02% |
|  | -0.12% | -0.08% | -0.04% | -0.24% | 0.11% | -0.06% | 0.08% | -0.06% | 0.09% |
| Class F | -0.30% | -0.30% | -0.27% | -0.37% | -0.39% | -0.34% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.11% | -0.09% | -0.10% | -0.69% | -0.35% |  |  |  |
| Class C | -0.20% | -0.14% | 0.16% | -0.47% | -0.39% | -0.58% |  |  |  |
| Class D | -0.23% | -0.26% | 0.29% | -0.51% | 0.78% | 0.20% |  |  |  |
| Class E | -0.12% | 0.30% | -0.72% | -0.14% | -0.86% | 0.50% |  |  |  |
| Overall | -0.13% | -0.08% | -0.05% | -0.30% | -0.28% | -0.11% |  |  |  |
|  | -0.13% | -0.04% | -0.03% | -0.30% | -0.24% | -0.15% |  |  |  |
| Class F | -0.41% | -0.39% | -0.71% | -0.39% | -1.31% | -1.03% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |  |  |  |

A summary of results under low QP settings as CE11 subset1[2] is shown in Table 4, and detailed results are available in “JCTVC-H0399_LQP.xls”.

Table 4 Summary of BD-rate results for low QP conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.13% | 0.05% | 0.10% | 0.02% | 0.03% | 0.04% |  |  |  |
| Class B | 0.08% | 0.13% | 0.11% | 0.01% | -0.07% | -0.06% |  |  |  |
| Class C | -0.04% | 0.00% | -0.04% | -0.20% | -0.18% | -0.18% |  |  |  |
| Class D | -0.10% | -0.06% | -0.05% | -0.24% | -0.21% | -0.20% |  |  |  |
| Class E | 0.05% | 0.10% | 0.07% | -0.05% | -0.02% | -0.02% |  |  |  |
| Overall | 0.01% | 0.04% | 0.03% | -0.10% | -0.11% | -0.10% |  |  |  |
|  | 0.01% | 0.05% | 0.04% | -0.10% | -0.11% | -0.10% |  |  |  |
| Class F | -0.07% | -0.18% | -0.25% | -0.27% | -0.19% | -0.18% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| 1 | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.03% | -0.14% | -0.28% | -0.2% | 0.0% | -0.1% | 0.22% | -0.07% | -0.12% |
| Class B | 0.05% | 0.17% | 0.07% | 0.0% | 0.1% | 0.0% | 0.06% | 0.13% | -0.03% |
| Class C | -0.31% | -0.06% | -0.24% | -0.5% | -0.3% | -0.3% |  |  |  |
| Class D | -0.33% | -0.14% | -0.20% | -0.5% | -0.2% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.15% | -0.01% | -0.13% | -0.3% | -0.1% | -0.2% | 0.13% | 0.04% | -0.07% |
|  | -0.14% | -0.01% | -0.14% | -0.3% | -0.1% | -0.2% | 0.15% | 0.04% | -0.09% |
| Class F | -0.40% | -0.28% | -0.43% | -0.4% | -0.4% | -0.3% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.22% | -0.08% | 0.1% | 0.0% | -0.1% |  |  |  |
| Class C | -0.34% | -0.06% | -0.08% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class D | -0.33% | -0.01% | -0.16% | -0.6% | -0.4% | -0.4% |  |  |  |
| Class E | -0.08% | 0.08% | -0.21% | -0.2% | -0.5% | -0.1% |  |  |  |
| Overall | -0.17% | 0.07% | -0.13% | -0.3% | -0.3% | -0.2% |  |  |  |
|  | -0.16% | 0.04% | -0.16% | -0.3% | -0.3% | -0.2% |  |  |  |
| Class F | -0.44% | -0.43% | -0.49% | -0.5% | -0.4% | -0.7% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |

# Conclusion

This proposal harmonized the coefficient coding based on scan. This solution has simpler design and better coding efficiency compared to HM5.0.  We propose to adopt this proposal into HEVC.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

- V. Sze, T. Nguyen, K. Panusopone, Y. Piao, J. Sole, “Description of Core Experiment 11 (CE11): Coefficient scanning and coding”, JCTVC-G1211, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).