| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0400r2 |
| --- | --- |

| Title: | Chroma QP extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Alexis Tourapis 591 Yosemite Drive Milpitas, CA 95035 | Tel: Email: | +1-408-934-3766 alexis.tourapis@magnumsemi.com |
|  | Ehsan Maani 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Ehsan.Maani@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc., Magnum Semiconductor Inc. |  |  |

_____________________________

# Abstract

In HM5.0, Chroma QP can only take values in the range of [0, 39]. This proposal extends Chroma QP to the range of [0, 51]. Such extension enhances the functionality, flexibility and friendliness of HEVC for various potential applications, such as rate control at low bit-rate including the flexibility to obtain the desired performance balance point between luma/chroma, and for handling video sources with different color formats (e.g. RGB).

# Problem statement

The current Chroma QP derivation process in HEVC replicates that of the H.264/AVC specification as shown in Table 1. For QP values in the range of 0 to 29 a linear relationship (QPC = QPY ) is followed, whereas a logarithmic relationship is followed for higher QP values. Chroma QP also saturates at a maximum value of 39 without any consideration of the color format that may be used.  This table actually defines the relationship between Luma and Chroma at different quality levels. However, this topic has not been thoroughly studied and evaluated in JCTVC meetings. Limiting Chroma QP in the range of [0, 39] has several disadvantages.

Table 1 H.264/AVC specification: Table 8-15 – Specification of QPC as a function of qPI

| qPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | =qPI | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

Firstly, problems in the bit consumption for luma and chroma arise when the bit-rate is very low. Experimental results using HM5.0 show that at high QPs (e.g. QP= 38, 42, 46, 50) luma and chroma consume almost the same number of bits for their residual coding, which is significantly different from what is observed for the common test conditions (i.e. QP= 22, 27, 32 and 37). Under common test conditions, the ratio of luma residual bits and chroma residual bits is around 9:1. With the limitation of Chroma QP, rate control algorithms may have difficulty in allocating or finding the desired bit budget balancing point between luma and chroma. In the scenario of low bit-rate applications, rate control algorithms may need to limit chroma bits by increasing Chroma QP, while saturating at 39 for Chroma QP could change such attempts to unachievable tasks.  A second problem is that this property does not allow the codec to properly quantize chroma to the desirable extent.

Another problem for rate control is that the previous observation makes bit-rate much more unpredictable and possibly unstable. In general, for rate control algorithms, one needs to design a model for bit to QP mapping. Quite commonly, algorithms tend to use luma information (distortion) and ignore chroma. As formerly mentioned, chroma bits at low bit-rates can have a severe impact in the total bit-rate. Furthermore, quite commonly the models used follow a quadratic relationship between bitrate and QP. Given the logarithmic behavior of Chroma QP that model may be less accurate.

Video sequences may have color components that might not follow the possibly logarithmic relationship between luma and chroma and their impact on subjective quality that seems to have inspired the chroma QP derivation process. Instead, it may be desirable to provide more flexibility in controlling these QP parameters, handling a more generic content space, but also be able to properly handle other color spaces and formats (e.g. RGB or YCoCg among others). Video coding standards should provide the functionality and flexibility for video sources with different color components or even other color formats (e.g. RGB and YCoCg). When video sources have a different relationship between Luma and Chroma components, assumed by Table 1, the codec may behave unpredictably.

# Solution

In this proposal, we remove the mapping table g_aucChromaScale[52] and set Chroma QP equal to Luma QP such that Chroma QP can take values in the range of [0, 51] as follows.

QPCb = Clip(0, 51, QPY+Cb_QP_offset),                                                (1)

QPCr = Clip(0, 51, QPY+Cr_QP_offset),                                                (2)

where Cb_QP_offset and Cr_QP_offset are the two Chroma QP offset parameters introduced in JCTVC-G509 [1]. With Equation (1) and (2), the codec can achieve the same model as g_aucChromaScale[52] in HM5.0 by controlling the Chroma QP offset parameters.

# Results

## Common test conditions

The proposed technique is implemented into HM5.0. The simulation is conducted using the common test conditions in [1]. A summary of the test results is shown in Table 2, and detailed results are available in “JCTVC-H0400.xls”.

Table 2. Summary of BD-rate results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.0% | 6.0% | 6.2% | -1.2% | 5.9% | 6.2% |
| Class B | -1.1% | 6.2% | 6.1% | -1.3% | 6.4% | 6.6% |
| Class C | -1.1% | 5.9% | 6.1% | -1.4% | 6.0% | 6.1% |
| Class D | -1.1% | 5.6% | 6.1% | -1.2% | 5.8% | 6.0% |
| Class E | -0.8% | 7.0% | 7.1% | -0.8% | 6.6% | 6.7% |
| Overall | -1.0% | 6.1% | 6.3% | -1.2% | 6.2% | 6.3% |
|  | -1.0% | 6.7% | 6.8% | -1.2% | 6.6% | 6.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | 8.4% | 8.6% | -0.8% | 8.6% | 8.5% |
| Class B | -1.1% | 10.5% | 9.7% | -1.2% | 10.8% | 10.0% |
| Class C | -1.2% | 8.9% | 9.1% | -1.3% | 8.9% | 9.2% |
| Class D | -1.0% | 8.6% | 8.9% | -1.0% | 9.2% | 9.2% |
| Class E |  |  |  |  |  |  |
| Overall | -1.1% | 9.3% | 9.2% | -1.1% | 9.6% | 9.4% |
|  | -1.1% | 9.5% | 9.5% | -1.1% | 9.7% | 9.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | 10.1% | 9.7% | -0.9% | 10.2% | 9.5% |
| Class C | -1.0% | 8.9% | 9.5% | -1.1% | 9.2% | 9.8% |
| Class D | -0.8% | 8.9% | 9.3% | -0.9% | 10.0% | 9.8% |
| Class E | -0.4% | 9.6% | 9.0% | -0.2% | 8.6% | 9.2% |
| Overall | -0.8% | 9.4% | 9.4% | -0.8% | 9.6% | 9.6% |
|  | -0.8% | 9.7% | 9.8% | -0.8% | 10.0% | 10.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

## High QP settings

Simulation is also conducted using high QPs, i.e. QP=38, 42, 46, 50. The RD-curve is shown in Figure 1. HM5.0 turns to have very small chroma PSNR difference for two adjacent test QPs and chroma PSNR saturates at certain levels, which confirms the limitation of HM5.0 in quantizing chroma to the desirable extent. Figure 2 shows that luma quality stays when releasing the limitation on Chroma QP, while bit-rate is reduced due to the larger quantization in chroma.Error: Reference source not found

Figure 1 RD curves of Chroma_U at QP=38, 42, 46, 50

Figure 2 RD curves of Luma at QP = 38, 42, 46, 50

# Conclusion

This proposal extended Chroma QP to the range of [0, 51], which simplified the design and enhanced the functionality of the standard. We propose to adopt this proposal into HEVC.

# Reference

- S. Liu and K. Sato, “Support of ChromaQPOffset in HEVC”, JCTVC-G509, Geneva, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Magnum Semiconductor Inc. does not have any current or pending patent rights relating to the technology described in this contribution.