| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0401 |
| --- | --- |

| Title: | Differentiated weighting parameter sets for weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Jun.Xu@am.sony.com |
|  | Alexis Tourapis 591 Yosemite Drive Milpitas, CA 95035 | Tel: Email: | +1-408-934-3766 alexis.tourapis@magnumsemi.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 Ali.Tabatabai@am.sony.com |
| Source: | Sony Electronics Inc., Magnum Semiconductor Inc. |  |  |

_____________________________

# Abstract

This proposal introduces differentiated weighting parameter sets for weighted prediction. Two separate weighting parameter sets serving for uni-prediction and bi-prediction respectively are signaled explicitly in weighted prediction syntax. The overhead for weighting parameter sets is small while weighted prediction can be improved.

# Introduction

Weighted prediction is adopted in HEVC to, primarily, compensate illumination changes that may exist in a video sequence. In HM5.0, only one explicit weighting parameter set can be associated with each picture in the reference lists for weighted prediction.  The same weighting parameter set is used for both uni-prediction and bi-prediction when explicit weighted prediction is invoked. However, one single weighting parameter set for both uni-prediction and bi-prediction cannot guarantee an optimal solution for both types of prediction.

# Solution

To improve the performance of weighted prediction, two weighting parameter sets are employed for each picture in the reference lists. One weighting parameter set is trained and signaled for uni-prediction and the other is trained and signaled for bi-prediction.

To support different weighting parameter sets for uni-prediction and bi-prediction, separate weighting parameter sets are signalled in the syntax for uni-prediction and bi-prediction, respectively. The corresponding changes in syntax and semantics are shown in Section 6.

# Conclusion

This proposal introduced separate weights for uni-prediction and bi-prediction to improve the performance of weighted prediction. The overhead in signaling weights is small. We propose to adopt this proposal into HEVC.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Magnum Semiconductor Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# WD changes

The changes in WD are shown in highlighted texts as follows.

#### Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_uni_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) { |  |
| chroma_log2_uni_weight_denom | ue(v) |
| } |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| luma_uni_weight_l0[ i ] | se(v) |
| luma_uni_offset_l0[ i ] | se(v) |
| } |  |
| if ( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| chroma_uni_weight_l0[ i ][ j ] | se(v) |
| chroma_uni_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| luma_log2_bi_weight_denom | ue(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag ) { |  |
| luma_bi_weight_l0[ i ] | se(v) |
| luma_bi_offset_l0[ i ] | se(v) |
| } |  |
| if ( chroma_format_idc != 0 ) { |  |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| chroma_bi_weight_l0[ i ][ j ] | se(v) |
| chroma_bi_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| luma_uni_weight_l1[ i ] | se(v) |
| luma_uni_offset_l1[ i ] | se(v) |
| luma_bi_weight_l1[ i ] | se(v) |
| luma_bi_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| chroma_uni_weight_l1[ i ][ j ] | se(v) |
| chroma_uni_offset_l1[ i ][ j ] | se(v) |
| chroma_bi_weight_l1[ i ][ j ] | se(v) |
| chroma_bi_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Weighted prediction parameters semantics

luma_log2_uni_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors of uni-prediction. The value of luma_log2_uni_weight_denom shall be in the range of 0 to 7, inclusive.

luma_log2_bi_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors of bi-prediction. The value of luma_log2_bi_weight_denom shall be in the range of 0 to 7, inclusive.

chroma_log2_uni_weight_denom is the base 2 logarithm of the denominator for all chroma weighting factors of uni-prediction.  The value of chroma_log2_uni_weight_denom shall be in the range of 0 to 7, inclusive.

chroma_log2_bi_weight_denom is the base 2 logarithm of the denominator for all chroma weighting factors of bi-prediction.  The value of chroma_log2_bi_weight_denom shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

luma_uni_weight_l0[ i ] is the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ] in uni-prediction.  When luma_weight_l0_flag is equal to 1, the value of luma_uni_weight_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_uni_weight_l0[ i ] shall be inferred to be equal to 2luma_log2_uni_weight_denom for RefPicList0[ i ].

luma_bi_weight_l0[ i ] is the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ] in bi-prediction.  When luma_weight_l0_flag is equal to 1, the value of luma_bi_weight_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_bi_weight_l0[ i ] shall be inferred to be equal to 2luma_log2_bi_weight_denom for RefPicList0[ i ].

luma_uni_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ] in uni-prediction. The value of luma_uni_offset_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_uni_offset_l0[ i ] shall be inferred as equal to 0 for RefPicList0[ i ].

luma_bi_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ] in bi-prediction. The value of luma_bi_offset_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_bi_offset_l0[ i ] shall be inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

chroma_uni_weight_l0[ i ][ j ] is the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] in uni-prediction with j equal to 0 for Cb and j equal to 1 for Cr.  When chroma_weight_l0_flag is equal to 1, the value of chroma_uni_weight_l0[ i ][ j ] shall be in the range of –128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, chroma_uni_weight_l0[ i ][ j ] shall be inferred to be equal to 2chroma_log2_uni_weight_denom for RefPicList0[ i ].

chroma_bi_weight_l0[ i ][ j ] is the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] in bi-prediction with j equal to 0 for Cb and j equal to 1 for Cr.  When chroma_weight_l0_flag is equal to 1, the value of chroma_bi_weight_l0[ i ][ j ] shall be in the range of –128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, chroma_bi_weight_l0[ i ][ j ] shall be inferred to be equal to 2chroma_log2_bi_weight_denom for RefPicList0[ i ].

chroma_uni_offset_l0[ i ][ j ] is the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] in uni-prediction with j equal to 0 for Cb and j equal to 1 for Cr. The value of chroma_uni_offset_l0[ i ][ j ] shall be in the range of -127 to 128, inclusive.  When chroma_weight_l0_flag is equal to 0, chroma_uni_offset_l0[ i ][ j ] shall be inferred to be equal to 0 for RefPicList0[ i ].

chroma_bi_offset_l0[ i ][ j ] is the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] in bi-prediction with j equal to 0 for Cb and j equal to 1 for Cr. The value of chroma_bi_offset_l0[ i ][ j ] shall be in the range of -127 to 128, inclusive.  When chroma_weight_l0_flag is equal to 0, chroma_bi_offset_l0[ i ][ j ] shall be inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, luma_uni_weight_l1, luma_bi_weight_l1, luma_uni_offset_l1, luma_bi_offset_l1, chroma_weight_l1_flag, chroma_uni_weight_l1, chroma_bi_weight_l1, chroma_uni_offset_l1, chroma_bi_offset_l1 have the same semantics as luma_weight_l0_flag, luma_uni_weight_l0, luma_bi_weight_l0, luma_uni_offset_l0, luma_bi_offset_l0, chroma_weight_l0_flag, chroma_uni_weight_l0, chroma_bi_weight_l0, chroma_uni_offset_l0, chroma_bi_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

###### Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction uni_logWDC, uni_logWDC, uni_w0C, uni_w1C, uni_o0C, uni_o1C, bi_logWDC, bi_logWDC, bi_w0C, bi_w1C, bi_o0C, bi_o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * uni_w0C + 2 uni_logWDC − 1 ) >> uni_logWDC ) + uni_ o0C ) 	 (8-1)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * uni_w0C + uni_o0C ) 	(8-2)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * uni_w1C + 2 uni_logWDC − 1 ) >> uni_logWDC ) + uni_o1C ) 	(8-3)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * uni_w1C + uni_o1C ) 	(8-4)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples [ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * ( bi_w0C + bi_w1C ) + 2 bi_logWDC ) 
			 >> (bi_logWDC + 1 ) ) + ( (bi_o0C + bi_o1C + 1 ) >> 1 ) )	(8-5)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * bi_w0C + predSamplesL1 [ x, y ] * bi_w1C + 2 bi_logWDC ) 
			 >> (bi_logWDC + 1 ) ) + ( (bi_o0C + bi_o1C + 1 ) >> 1 ) )	(8-6)

Where the variables uni_logWDC, uni_logWDC, uni_w0C, uni_w1C, uni_o0C, uni_o1C, bi_logWDC, bi_logWDC, bi_w0C, bi_w1C, bi_o0C, bi_o1C  are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

logWDc = 5+shift1		(8-7)

o0C = 0				(8-8)

o1C = 0				(8-9)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( -128, 127, PicOrderCnt( currPoc ) – RefPicOrderCnt( currPic, refIdxL0, L0 ) )	(8-10)

td = Clip3( -128, 127, RefPicOrderCnt( currPoc, refIdxL1, L1 ) 
		 – RefPicOrderCnt( currPoc, refIdxL0, L0 )	(8-11)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-12)

WeightScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-13)

The variables w0C and w1C are derived as follows.

–	If RefPicOrderCnt( currPic, refIdxL0, L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ) or ( WeightScaleFactor >> 2 ) < -64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

w0C=32				(8-199)

w1C=32				(8-14)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)		(8-15)

w1C = WeightScaleFactor >> 2		(8-16)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice or weighted_bipred_idc equal to 1 in B-slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

uni_logWDc = luma_log2_ uni_weight_denom+ shift	(8-17-1)

uni_w0C = luma_ uni_weight_l0[refIdxL0]		(8-18-1)

uni_w1C = luma_ uni_weight_l1[refIdxL1]		(8-19-1)

uni_o0C = luma_ uni_offset_l0[refIdxL0] * ( 1 << ( BitDepthY – 8 ) )	(8-20-1)

uni_o1C = luma_ uni_offset_l1[refIdxL1] * ( 1 << ( BitDepthY – 8 ) )	(8-21-1)

bi_logWDc = luma_log2_ bi_weight_denom+ shift	(8-224-2)

bi_w0C = luma_ bi_weight_l0[refIdxL0]		(8-23-2)

bi_w1C = luma_ bi_weight_l1[refIdxL1]		(8-24-2)

bi_o0C = luma_ bi_offset_l0[refIdxL0] * ( 1 << ( BitDepthY – 8 ) )	(8-25-2)

bi_o1C = luma_ bi_offset_l1[refIdxL1] * ( 1 << ( BitDepthY – 8 ) )	(8-26-2)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

uni_logWDc = chroma_log2_ uni_weight_denom+ shift1	(8-27-1)

uni_w0C = chroma_ uni_weight_l0[refIdxL0][ iCbCr ]	(8-28-1)

uni_w1C = chroma_ uni_weight_l1[refIdxL1][ iCbCr ]	(8-29-1)

uni_o0C = chroma_ uni_offset_l0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-30-1)

uni_o1C = chroma_ uni_offset_l1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-31-1)

bi_logWDc = chroma_log2_ bi_weight_denom+ shift1	(8-32-2)

bi_w0C = chroma_ bi_weight_l0[refIdxL0][ iCbCr ]	(8-33-2)

bi_w1C = chroma_ bi_weight_l1[refIdxL1][ iCbCr ]	(8-34-2)

bi_o0C = chroma_ bi_offset_l0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-35-2)

bi_o1C = chroma_ bi_offset_l1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-36-2)