| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0401r2 |
| --- | --- |

| Title: | Differentiated weighting parameter sets for weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Jun.Xu@am.sony.com |
|  | Alexis Tourapis 591 Yosemite Drive Milpitas, CA 95035 | Tel: Email: | +1-408-934-3766 alexis.tourapis@magnumsemi.com |
|  | Ehsan Maani 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Ehsan.Maani@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 Ali.Tabatabai@am.sony.com |
| Source: | Sony Electronics Inc., Magnum Semiconductor Inc. |  |  |

_____________________________

# Abstract

This proposal introduces differentiated weighting parameter sets for weighted prediction. Two separate weighting parameter sets serving for uni-prediction and bi-prediction respectively are signaled explicitly in weighted prediction syntax. The overhead for weighting parameter sets is small while weighted prediction can be improved.

# Introduction

Weighted prediction is adopted in HEVC to, primarily, compensate illumination changes that may exist in a video sequence. In HM5.0, only one explicit weighting parameter set can be associated with each picture in the reference lists for weighted prediction.  The same weighting parameter set is used for both uni-prediction and bi-prediction when explicit weighted prediction is invoked. However, one single weighting parameter set for both uni-prediction and bi-prediction cannot guarantee an optimal solution for both types of prediction.

# Weighting parameter sets

# To improve the performance of weighted prediction, two weighting parameter sets are employed for each picture in the reference lists. One weighting parameter set is signaled for uni-prediction and the other is signaled for bi-prediction.

To support different weighting parameter sets for uni-prediction and bi-prediction, separate weighting parameter sets are signalled in the syntax for uni-prediction and bi-prediction, respectively. To achieve efficient signalling, bi-prediction weighting factor is encoded using prediction from uni-prediction weighting factor. The corresponding changes in syntax and semantics are shown in Section 8.

# Weighted Prediction for Chroma

Current working draft incorporates a prediction for the chroma offset value. Thus, the chroma offset value ChromaOffsetL0[ i ][ j ] is obtained by

shift = 1 << ( BitDepthC – 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom )  shift )

where delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr. ChromaWeightL0 is the chroma explicit weight and ChromaLog2WeightDenom is the base 2 logarithm of the denominator for chroma weighting factors.

However, depending on the format of the input signal, this prediction may not be effective. Therefore, we propose to add a flag, namely chroma_offset_pred_flag, to allow prediction from zero similarly to the luma component. In other words, if the chroma_offset_pred_flag is off then the value ChromaOffsetL0[ i ][ j ] is computed according to

ChromaOffsetL0[ i ][ j ] = delta_chroma_offset_l0[i][j]

otherwise the same process as incorporated in the current working draft is used.

# Conclusion

This proposal introduced separate weights for uni-prediction and bi-prediction to improve the performance of weighted prediction. The overhead in signaling weights is small. We propose to adopt this proposal into HEVC.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Magnum Semiconductor Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# WD changes

The changes in WD are shown in highlighted texts as follows.

#### Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |  |
| --- | --- | --- |
| luma_log2_uni_weight_denom | ue(v) |  |
| if( chroma_format_idc != 0 ){ |  |  |
| delta_chroma_log2_uni_weight_denom | se(v) |  |
| chroma_offset_pred_flag | u(1) |  |
| } |  |  |
| if ( slice_type = = P ¦ ¦ ( slice_type = = B && ref_pic_list_combination_flag = = 0 ) ) { |  |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |  |
| luma_uni_weight_l0_flag[i] | u(1) |  |
| if( luma_uni_weight_l0_flag[i] ) { |  |  |
| delta_luma_uni_weight_l0[ i ] | se(v) |  |
| luma_uni_offset_l0[ i ] | se(v) |  |
| } |  |  |
| if ( chroma_format_idc != 0 ) { |  |  |
| chroma_uni_weight_l0_flag[i] | u(1) |  |
| if( chroma_uni_weight_l0_flag[i] ) |  |  |
| for( j =0; j < 2; j++ ) { |  |  |
| delta_chroma_uni_weight_l0[ i ][ j ] | se(v) |  |
| delta_chroma_uni_offset_l0[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( slice_type = = B && ref_pic_list_combination_flag = = 0 ) { |  |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |  |
| luma_uni_weight_l1_flag[i] | u(1) |  |
| if( luma_uni_weight_l1_flag[i] ) { |  |  |
| delta_luma_uni_weight_l1[ i ] | se(v) |  |
| luma_uni_offset_l1[ i ] | se(v) |  |
| } |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| chroma_uni_weight_l1_flag[i] | u(1) |  |
| if( chroma_uni_weight_l1_flag[i] ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| delta_chroma_uni_weight_l1[ i ][ j ] | se(v) |  |
| delta_chroma_uni_offset_l1[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( slice_type = = B ) { |  |  |
| luma_log2_bi_weight_denom | ue(v) |  |
| if( chroma_format_idc != 0 ) |  |  |
| delta_chroma_log2_bi_weight_denom | se(v) |  |
| if ( ref_pic_list_combination_flag = = 0 ) { |  |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |  |
| luma_bi_weight_l0_flag[i] | u(1) |  |
| if( luma_bi_weight_l0_flag[i] ) { |  |  |
| delta_luma_bi_weight_l0[ i ] | se(v) |  |
| luma_bi_offset_l0[ i ] | se(v) |  |
| } |  |  |
| if ( chroma_format_idc != 0 ) { |  |  |
| chroma_bi_weight_l0_flag[i] | u(1) |  |
| if( chroma_bi_weight_l0_flag[i] ) |  |  |
| for( j =0; j < 2; j++ ) { |  |  |
| delta_chroma_bi_weight_l0[ i ][ j ] | se(v) |  |
| delta_chroma_bi_offset_l0[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |  |
| luma_bi_weight_l1_flag[i] | u(1) |  |
| if( luma_bi_weight_l1_flag[i] ) { |  |  |
| delta_luma_bi_weight_l1[ i ] | se(v) |  |
| luma_bi_offset_l1[ i ] | se(v) |  |
| } |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| chroma_bi_weight_l1_flag[i] | u(1) |  |
| if( chroma_bi_weight_l1_flag[i] ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| delta_chroma_bi_weight_l1[ i ][ j ] | se(v) |  |
| delta_chroma_bi_offset_l1[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |  |
| luma_uni_weight_lc_flag[i] | u(1) |  |
| if( luma_uni_weight_lc_flag[i] ) { |  |  |
| delta_luma_uni_weight_lc[ i ] | se(v) |  |
| luma_uni_offset_lc[ i ] | se(v) |  |
| } |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| chroma_uni_weight_lc_flag[i] | u(1) |  |
| if( chroma_uni_weight_lc_flag[i] ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| delta_chroma_uni_weight_lc[ i ][ j ] | se(v) |  |
| delta_chroma_uni_offset_lc[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |  |
| luma_bi_weight_lc_flag[i] | u(1) |  |
| if( luma_bi_weight_lc_flag[i] ) { |  |  |
| delta_luma_bi_weight_lc[ i ] | se(v) |  |
| luma_bi_offset_lc[ i ] | se(v) |  |
| } |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| chroma_bi_weight_lc_flag[i] | u(1) |  |
| if( chroma_bi_weight_lc_flag[i] ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| delta_chroma_bi_weight_lc[ i ][ j ] | se(v) |  |
| delta_chroma_bi_offset_lc[ i ][ j ] | se(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Weighted prediction parameters semantics

luma_log2_uni_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors of uni-prediction. The value of luma_log2_uni_weight_denom shall be in the range of 0 to 7, inclusive.

luma_log2_bi_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors of bi-prediction. The value of luma_log2_bi_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_uni_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2UniWeightDenom is specified by luma_log2_uni_weight_denom + delta_chroma_log2_uni_weight_denom and it shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_bi_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2BiWeightDenom is specified by luma_log2_bi_weight_denom + delta_chroma_log2_bi_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_uni_weight_l0_flag[i] equal to 1 specifies that weighting factor for the luma component of list 0 uni-prediction using RefPicList0[ i ] is present. luma_uni_weight_l0_flag[i] equal to 0 specifies that the weighting factor is not present.

luma_bi_weight_l0_flag[i] equal to 1 specifies that weighting factor for the luma component of list 0 bi-prediction using RefPicList0[ i ] is present. luma_bi_weight_l0_flag[i] equal to 0 specifies that the weighting factor is not present.

delta_luma_uni_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 uni-prediction using RefPicList0[ i ].

The variable LumaUniWeightL0[ i ] is specified by (1 << luma_log2_uni_weight_denom ) + delta_luma_uni_weight_l0[ i ]. When luma_uni_weight_l0_flag[i] is equal to 1, the value of LumaUniWeightL0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_uni_weight_l0_flag[i] is equal to 0, LumaUniWeightL0[ i ] shall be inferred to be equal to 2luma_log2_uni_weight_denom for RefPicList0[ i ].

delta_luma_bi_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 bi-prediction using RefPicList0[ i ].

The variable LumaBiWeightL0[ i ] is specified by LumaUniWeightL0[ i ] + delta_luma_bi_weight_l0[ i ]. When luma_bi_weight_l0_flag[i] is equal to 1, the value of LumaBiWeightL0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_bi_weight_l0_flag[i] is equal to 0, LumaBiWeightL0[ i ] shall be inferred to be equal to 2luma_log2_bi_weight_denom for RefPicList0[ i ].

luma_uni_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 uni-prediction using RefPicList0[ i ]. The value of luma_uni_offset_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_uni_weight_l0_flag[i] is equal to 0, luma_uni_offset_l0[ i ] shall be inferred as equal to 0 for RefPicList0[ i ].

luma_bi_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 bi-prediction using RefPicList0[ i ]. The value of luma_bi_offset_l0[ i ] shall be in the range of –128 to 127, inclusive.  When luma_bi_weight_l0_flag[i] is equal to 0, luma_bi_offset_l0[ i ] shall be inferred as equal to 0 for RefPicList0[ i ].

chroma_uni_weight_l0_flag[i] equal to 1 specifies that weighting factor for the chroma prediction values of list 0 uni-prediction is present. chroma_uni_weight_l0_flag[i] equal to 0 specifies that these weighting factor is not present.

chroma_bi_weight_l0_flag[i] equal to 1 specifies that weighting factor for the chroma prediction values of list 0 bi-prediction is present. chroma_bi_weight_l0_flag[i] equal to 0 specifies that these weighting factor is not present.

delta_chroma_uni_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 uni-prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaUniWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2UniWeightDenom ) + delta_chroma_uni_weight_l0[ i ][ j ]. When chroma_uni_weight_l0_flag[i] is equal to 1, the value of ChromaUniWeightL0[ i ][ j ] shall be in the range of –128 to 127, inclusive.  When chroma_uni_weight_l0_flag[i] is equal to 0, ChromaUniWeightL0[ i ][ j ] shall be inferred to be equal to 2ChromaLog2UniWeightDenom for RefPicList0[ i ].

delta_chroma_bi_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 bi-prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaBiWeightL0[ i ][ j ] is specified by ChromaUniWeightL0[ i ][ j ]  + delta_chroma_bi_weight_l0[ i ][ j ]. When chroma_bi_weight_l0_flag[i] is equal to 1, the value of ChromaBiWeightL0[ i ][ j ] shall be in the range of –128 to 127, inclusive.  When chroma_bi_weight_l0_flag[i] is equal to 0, ChromaBiWeightL0[ i ][ j ] shall be inferred to be equal to 2ChromaLog2BiWeightDenom for RefPicList0[ i ].

delta_chroma_uni_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 uni-prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaUniOffsetL0[ i ][ j ] is specified as follows:

If chroma_offset_pred_flag is equal to 1

shift = 1 << ( BitDepthC – 1 )

ChromaUniOffsetL0[ i ][ j ] = (delta_chroma_uni_offset_l0[i][j] – 
				( (shift*ChromaUniWeightL0[ i ][ j ]) >> ChromaLog2UniWeightDenom ) + shift )	 (7-1)

Otherwise

ChromaUniOffsetL0[ i ][ j ] = delta_chroma_uni_offset_l0[i][j]

The variable ChromaUniOffsetL0[ i ][ j ] shall be in the range of -127 to 128, inclusive. When chroma_uni_weight_l0_flag[i] is equal to 0, ChromaUniOffsetL0[ i ][ j ] shall be inferred to be equal to 0 for RefPicList0[ i ].

delta_chroma_bi_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 bi-prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaBiOffsetL0[ i ][ j ] is specified as follows:

If chroma_offset_pred_flag is equal to 1

shift = 1 << ( BitDepthC – 1 )

ChromaBiOffsetL0[ i ][ j ] = (delta_chroma_bi_offset_l0[i][j] – 
				( (shift*ChromaBiWeightL0[ i ][ j ]) >> ChromaLog2BiWeightDenom ) + shift )	 (7-2)

Otherwise

ChromaBiOffsetL0[ i ][ j ] = delta_chroma_bi_offset_l0[i][j]

The variable ChromaBiOffsetL0[ i ][ j ] shall be in the range of -127 to 128, inclusive. When chroma_bi_weight_l0_flag[i] is equal to 0, ChromaBiOffsetL0[ i ][ j ] shall be inferred to be equal to 0 for RefPicList0[ i ].

luma_uni_weight_l1_flag, delta_luma_uni_weight_l1, luma_uni_offset_l1, chroma_uni_weight_l1_flag, delta_chroma_uni_weight_l1, delta_chroma_uni_offset_l1, luma_bi_weight_l1_flag, delta_luma_bi_weight_l1, luma_bi_offset_l1, chroma_bi_weight_l1_flag, delta_chroma_bi_weight_l1, delta_chroma_bi_offset_l1 have the same semantics as luma_uni_weight_l0_flag, delta_luma_uni_weight_l0, luma_uni_offset_l0, chroma_uni_weight_l0_flag, delta_chroma_uni_weight_l0, delta_chroma_uni_offset_l0, luma_bi_weight_l0_flag, delta_luma_bi_weight_l0, luma_bi_offset_l0, chroma_bi_weight_l0_flag, delta_chroma_bi_weight_l0, delta_chroma_bi_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

luma_uni_weight_lc_flag, delta_luma_uni_weight_lc, luma_uni_offset_lc, chroma_uni_weight_lc_flag, delta_chroma_uni_weight_lc, delta_chroma_uni_offset_lc, luma_bi_weight_lc_flag, delta_luma_bi_weight_lc, luma_bi_offset_lc, chroma_bi_weight_lc_flag, delta_chroma_bi_weight_lc, delta_chroma_bi_offset_lc have the same semantics as luma_uni_weight_l0_flag, delta_luma_uni_weight_l0, luma_uni_offset_l0, chroma_uni_weight_l0_flag, delta_chroma_uni_weight_l0, delta_chroma_uni_offset_l0, luma_bi_weight_l0_flag, delta_luma_bi_weight_l0, luma_bi_offset_l0, chroma_bi_weight_l0_flag, delta_chroma_bi_weight_l0, delta_chroma_bi_offset_l0, respectively. [Ed. (WJ): the use of terms list combination / List combination here may not be suitable]

###### Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction uni_logWDC, uni_logWDC, uni_w0C, uni_w1C, uni_o0C, uni_o1C, bi_logWDC, bi_logWDC, bi_w0C, bi_w1C, bi_o0C, bi_o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * uni_w0C + 2 uni_logWDC − 1 ) >> uni_logWDC ) + uni_ o0C ) 	 (8-3)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * uni_w0C + uni_o0C ) 	(8-4)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * uni_w1C + 2 uni_logWDC − 1 ) >> uni_logWDC ) + uni_o1C ) 	(8-5)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * uni_w1C + uni_o1C ) 	(8-6)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * ( bi_w0C + bi_w1C ) + 
			 ( (bi_o0C + bi_o1C + 1 ) << bi_logWDC ) ) >> (bi_logWDC + 1 ) )	(8-7)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * bi_w0C + predSamplesL1 [ x, y ] * bi_w1C + 
			 ( (bi_o0C + bi_o1C + 1 ) << logWDC ) ) >> (bi_logWDC + 1 ) )	(8-8)

Where the variables uni_logWDC, uni_logWDC, uni_w0C, uni_w1C, uni_o0C, uni_o1C, bi_logWDC, bi_logWDC, bi_w0C, bi_w1C, bi_o0C, bi_o1C  are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

uni_logWDc = 5+shift1		(8-9-1)

bi_logWDc = 5+shift1		(8-10-2)

uni_o0C = 0		(8-11-2)

bi_o0C = 0			(8-12-2)

uni_o1C = 0		(8-13-1)

bi_o1C = 0			(8-14-2)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-15)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 
		 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-16)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-17)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-18)

The variables uni_w0C, uni_w1C bi_w0C and bi_w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

uni_w0C=32		(8-199-1)

bi_w0C=32			(8-199-2)

uni_w1C=32		(8-19-1)

bi_w1C=32			(8-20-2)

–	Otherwise;

uni_w0C = 64 – (WeightScaleFactor >> 2)		(8-21-1)

bi_w0C = 64 – (WeightScaleFactor >> 2)		(8-22-2)

uni_w1C = WeightScaleFactor >> 2		(8-23-1)

bi_w1C = WeightScaleFactor >> 2		(8-24-2)

–	Otherwise, if both weighted_bipred_idc and ref_pic_list_combination_flag are equal to 1 in B slice, combined explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

uni_logWDc = luma_log2_uni_weight_denom+ shift1	(8-25-1)

bi_logWDc = luma_log2_bi_weight_denom+ shift1	(8-26-2)

uni_w0C = LumaUniWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ]	(8-27-1)

bi_w0C = LumaBiWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ]	(8-28-2)

uni_w1C = LumaUniWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ]	(8-29-1)

bi_w1C = LumaBiWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ]	(8-30-2)

uni_o0C = luma_uni_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL0 ] ] * ( 1 << ( BitDepthY – 8 ) )	(8-31-1)

bi_o0C = luma_bi_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL0 ] ] * ( 1 << ( BitDepthY – 8 ) )	(8-32-2)

uni_o1C = luma_uni_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL1 ] ] * ( 1 << ( BitDepthY – 8 ) )	(8-33-1)

bi_o1C = luma_bi_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL1 ] ] * ( 1 << ( BitDepthY – 8 ) )	(8-34-2)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

uni_logWDc = ChromaLog2UniWeightDenom + shift1	(8-35-1)

bi_logWDc = ChromaLog2BiWeightDenom + shift1	(8-36-2)

uni_w0C = ChromaUniWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ]	(8-37-1)

bi_w0C = ChromaBiWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ]	(8-38-2)

uni_w1C = ChromaUniWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ]	(8-39-1)

bi_w1C = ChromaBiWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ]	(8-40-2)

uni_o0C = ChromaUniOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-41-1)

bi_o0C = ChromaBiOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-42-2)

uni_o1C = ChromaUniOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-43-1)

bi_o1C = ChromaBiOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-44-2)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice or ref_pic_list_combination_flag is equal to 0 in B-slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

uni_logWDc = luma_log2_uni_weight_denom+ shift1	(8-45-1)

bi_logWDc = luma_log2_bi_weight_denom+ shift1	(8-46-2)

uni_w0C = LumaUniWeightL0[refIdxL0]		(8-47-1)

bi_w0C = LumaBiWeightL0[refIdxL0]		(8-48-2)

uni_w1C = LumaUniWeightL1[refIdxL1]		(8-49-1)

bi_w1C = LumaBiWeightL1[refIdxL1]		(8-50-2)

uni_o0C = luma_uni_offset_l0[refIdxL0] * ( 1 << ( BitDepthY – 8 ) )	(8-51-1)

bi_o0C = luma_bi_offset_l0[refIdxL0] * ( 1 << ( BitDepthY – 8 ) )	(8-52-2)

uni_o1C = luma_uni_offset_l1[refIdxL1] * ( 1 << ( BitDepthY – 8 ) )	(8-53-1)

bi_o1C = luma_bi_offset_l1[refIdxL1] * ( 1 << ( BitDepthY – 8 ) )	(8-54-2)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

uni_logWDc = ChromaLog2UniWeightDenom + shift1	(8-55-1)

bi_logWDc = ChromaLog2BiWeightDenom + shift1	(8-56-2)

uni_w0C = ChromaUniWeightL0[refIdxL0][ iCbCr ]	(8-57-1)

bi_w0C = ChromaBiWeightL0[refIdxL0][ iCbCr ]	(8-58-2)

uni_w1C = ChromaUniWeightL1[refIdxL1][ iCbCr ]	(8-59-1)

bi_w1C = ChromaBiWeightL1[refIdxL1][ iCbCr ]	(8-60-2)

uni_o0C = ChromaUniWeightL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-61-1)

bi_o0C = ChromaBiWeightL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-62-2)

uni_o1C = ChromaUniOffsetL1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-63-1)

bi_o1C = ChromaBiOffsetL1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC – 8 ) )	(8-64-2)