| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0404 |
| --- | --- |

| Title: | Context Set Selection for Coefficient Level Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He and Gaëlle Martin-Cocher 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

A refined context set selection algorithm for coefficient level coding is proposed. The proposed algorithm uses information from neighbouring coefficient groups and therefore, is more consistent with multi-level significant coefficient flag coding, while providing a small improvement in BD-rate. On average, under common test conditions against HM-5.0 anchor, the proposal improves the BD-rate by 0.1% for the AI-HE configuration and suffers no loss in all other configurations.

# Problem

The HEVC current working draft [2] specifies that the context set for coefficient level coding is dependent on the number of transform coefficients with absolute value greater than one in the previous scan set.  With the adoption of JCTVC-G323 [5], the previous scan set is precisely the previous coefficient group in reverse sub-block scan order.  The basic premise of the existing context selection algorithm is to derive information about the magnitudes of transform coefficients from the magnitudes of recently processed or nearby transform coefficients.  Unfortunately, the previously processed scan set does not always translate into a nearby scan set.  For example, when moving from diagonal to another, coefficient groups are generally not close to each other.  Consider the 32x32 TU illustrated below, partitioned into coefficient groups.  The highlighted coefficient groups are consecutive in scan order, but are far away from each other geometrically.  Hence, one would expect the magnitudes of their transform coefficients to be weakly correlated.

# Proposal

## Context set selection based on neighbouring coefficient groups

In general, the context set selection algorithm for coefficient level coding works as follows.  After the context set is initialized for the current scan set, it is incremented by 1 if f > 1 and incremented by 2 if f > 3, where f is a function of the number of transform coefficients with absolute value greater than one in the previous scan set(s).     

In JCTVC-G644 [7], which was adopted, the right and lower neighbouring coefficient groups are used to determine the context for the significance of the current coefficient group.  A similar idea is proposed here: use the right and lower neighbouring coefficient groups to determine the context for the coefficient levels in the current coefficient group, as show n in the figure below, where the red-shaded block denotes the current coefficient group, and the green-shaded blocks denote its right and lower neighbours.

Let R and L be the number of transform coefficients with absolute value greater than one in the right coefficient group and lower coefficient group, respectively.  If the right or lower coefficient group falls outside the boundary of the TU, R or L is assumed to be 0, respectively.  Then, f = f(R,L),  where

f(R, L) = Q(R) + Q(L)

and

Q(k) = 0 if k = 0,

Q(k) = 1 if 0 < k <= 3, and

Q(k) = 2 otherwise

The proposed context set selection algorithm is more intuitively satisfying and natural than the existing algorithm in the HEVC current working draft [2] because it addresses the problem described in Section 1.  Moreover, it results in a more consistent, harmonized design with significant coefficient flag coding.

# Results

In this section, we report experimental results of the proposal (integrated into HM-5.0) against HM-5.0 anchor under common test conditions [4].

Table 3.1 below shows that the BD-rates of the proposal against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).  Note that the encoding time results of the proposal and the HM-5.0 anchor are obtained on the same computing platform; and the decoding time results of the proposal and the HM-5.0 anchor are obtained on the same computer.

Table 3.1: BD-rate vs. HM-5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.2% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | 0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.3% |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.2% | 0.3% |
| Class E | 0.0% | 0.5% | -0.7% | 0.0% | -0.6% | 0.3% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.1% |
| Class F | -0.1% | -0.1% | -0.4% | 0.0% | 0.0% | -0.6% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Note that the decoding timing results will be updated shortly.

# Conclusion

The proposed context set selection algorithm is a refinement of the existing algorithm.  It uses information from geometrically neighbouring scan sets (as opposed to previous scan sets), unifies concepts used in significant coefficient flag coding and provides 0.1% improvement in BD-rate for the AI-HE configuration.

# Change to the HEVC text spec.

## Proposed changes to “9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag”

Inputs to this process are the color component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-1)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  3		(9-2)

- When the subset i is not the first one to be processed in this subclause, the following applies.

- If the TU is 4x4 or 8x8, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause Error: Reference source not found for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-3)

- When ( numGreater1  >>  1 ) is greater than 3 and cIdx is equal to 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-4)

- Otherwise, the TU is 16x16 or 32x32, and the following applies.

- The variable Q_sum is set equal to the sum of the Q_numGreater1 variables that have been derived for the subsets immediately to the right of subset i and immediately below subset i.  If either the right or lower subsets do not exist (i.e., fall outside the boundary of the TU), their respective Q_numGreater1 variables are assumed to be 0.

- When  Q_sum is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-5)

- When Q_sum is greater than 3, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-6)

- The variable greater1Ctx is set equal to 1.

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater1Ctx is set equal to the variable greater1Ctx that has been derived during the last invocation of this subclause.

- When greater1Ctx is greater than 0, the variable lastGreater1Flag is set equal to the syntax element coeff_abs_level_greater1_flag that has been used during the last invocation of this subclause and greater1Ctx is modifed as follows.

- If lastGreater1Flag is equal to 1, greater1Ctx is set equal to 0.

- Otherwise (lastGreater1Flag is equal to 0), greater1Ctx is incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )	(9-7)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  24

## Proposed changes to “9.3.3.1.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag”

Inputs to this process are the color component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntex elements coeff_abs_level_greater2_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-8)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  3		(9-9)

- If the TU is 16x16 or 32x32, a separate instance of the variable Q_numGreater1 is maintained for each subset.

- The variable numGreater1 for the first subset is set equal to 0.

- When the subset i is not the first one to be processed in this subclause, the following applies.

- If the TU is 4x4 or 8x8, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause for the subset i + 1 and is modifed as follows.

numGreater1  =  numGreater1  >>  1		(9-10)

- When numGreater1 is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-11)

- When numGreater1 is greater than 3 and cIdx is equal to 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-12)

- Otherwise, the TU is 16x16 or 32x32, and the following applies.

- The variable Q_sum is set equal to the sum of the Q_numGreater1 variables that have been derived for the subsets immediately to the right of subset i and immediately below subset i.  If either the right or lower subsets do not exist (i.e., fall outside the boundary of the TU), their respective Q_numGreater1 variables are assumed to be 0.

- When  Q_sum is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-13)

- When Q_sum is greater than 3, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-14)

- The variable greater2Ctx is set equal to 0.

- Otherwise (coeff_abs_level_greater2_flag[ n ] is not the first to be parsed within the current subset i), for the derivation of ctxSet and greater2Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater2Ctx is set equal to the variable greater2Ctx that has been derived during the last invocation of this subclause incremented by 1.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause incremented by 1.

- If the TU is 16x16 or 32x32 and coeff_abs_level_greater2_flag[ n ] is last to be parsed within the current subset i, Q_numGreater1 for subset i is set equal to 0 if numGreater1 = 0, 1 if 0 < numGreater1 <=3, and 2 otherwise.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  3 )  +  Min( 2, greater2Ctx )	(9-15)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

# ctxIdxInc =  ctxIdxInc  +  18

# References

[1] D. Marpe, H. Schwarz, and T. Wiegand, “Context-based adaptive binary arithmetic coding in the h.264/AVC video compression standard,” IEEE Transactions on Circuits and Systems for Video Technology, 13(7):620–636, Jul. 2003.

[2] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-G1103.

[3] T. Nguyen, T. Winken, D. Marpe et al., “Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and PIPE”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting, Daegu, January 2011.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

[5] J. Sole, R. Joshi, and M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” JCTVC-G323.

[6] J. Sole, “Cross-check of RIM's multi-level significant maps for large transform units (JCTVC-G644),” JCTVC-G1001.

[7] N. Nguyen, T. Ji, D. He, G. Martin-Cocher and L. Song, “Multiple Level Significance Maps for Large Transform Units”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 7th Meeting, Geneva, November 2011.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).