| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0405 |
| --- | --- |

| Title: | Bypass coding in chroma Intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC] |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ehsan Maani, Ali Tabatabai 1730 N 1st ST. San Jose, CA 95112 | Tel: Email: | (408) 352 4710 ehsan.maani@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Current HM5.0 applies bypass coding in luma Intra mode coding but not in the coding of chroma Intra modes. This proposal extends bypass coding to chroma Intra mode coding. Using this approach 3 out of 5 chroma Intra bins are bypass coded. Based on common test condition, a small performance gain is observed for the chroma channels especially in the low complexity configuration.

# Proposed method

Current HM5.0 recognizes the following 6 modes for chroma Intra prediction: DM, LM, Planar, Ver, Hor, and DC. These modes are signalled using truncated unary codes as shown in Table 1 below.

Table 1. Intra chroma modes and codewords.

| Mode | Codeword |
| --- | --- |
| DM | 0 |
| LM | 01 |
| Planar | 11 0 |
| Ver | 11 10 |
| Hor | 11 110 |
| DC | 11 111 |

The first bin uses a separate context while the rest of the bins share one context. In the proposed method, however, there is no change in the context of the first bin. The second bin also uses a separate context. The rest of the bins will be coded using the bypass mode in CABAC.

Using this approach the last 3 bins will be coded using the bypass mode which reduces the codec's complexity especially at the decoder.

# Results

Based on the common test condition, the following results were obtained for the proposed modifications:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.3% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).