| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0407 |
| --- | --- |

| Title: | Intra mode coding using logical mode numbering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC] |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ehsan Maani, Ali Tabatabai 1730 N 1st ST. San Jose, CA 95112 | Tel: Email: | (408) 352 4710 ehsan.maani@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document proposes to use logical mode numbers for both performing prediction and coding in the bit stream. Logical mode numbers are sorted based on angle (with respect to an arbitrary reference) and are therefore more meaningful which result in a better design and removal of several otherwise necessary look-up tables. In addition to many simplifications that results from this type of numbering, simulations show there is a small coding efficiency benefit. Furthermore, by using this type of naming, adding an additional MPM is very efficient since no additional tables are required. Using 3 MPMs achieves a larger gain of 0.25% and 0.31% for the AI_HE and AI_LC cases, respectively.

# Introduction

Currently there are two types Intra mode names (numbering) used in HM5.0: the logical and bit stream mode numbers. The logical mode numbers are more meaningful and are directly used in the prediction module to produce the Intra prediction. Bit stream mode numbers are, however, only used to signal the mode to the decoder. Bit stream mode numbers are inherited from AVC and considered to be sorted in their frequency of occurrences. However, statistics collected from HEVC does not show any bias towards any of the angular directions (not including Planar and DC) and as a result bypass coding is used for the remaining modes. Therefore, logical mode numbers are enough to describe the directions and also can be directly coded into the bit stream. Using of logical mode numbers everywhere in the software has multiple advantages:

- Better design: mode numbers are meaningful. Given an Intra mode number, its direction angle can be determined with simple calculations without any look-up tables. In addition, it can be deducted that adjacent mode numbers mean adjacent prediction directions.

- Simple mapping: 4x4 PU sizes do not support all Intra prediction modes as in other PU sizes. Therefore, for MPM calculation of 4x4 PUs, a mapping is needed. With logical mode numbers a simple operation -such as clearing the least significant bit- is enough to do the mapping. Currently, to avoid mapping Planar mode is used for 4x4 PU where clearly an angular mode should be used.

- Several look-up tables can be removed. Intra smoothing,  mode dependent scan, and mode dependant transform do not need large tables to figure out the smoothing, scan index, or transform type (DCT/DST). Instead, a simple mode range checking can determine the criteria for which different smoothing filters, scans, or transforms can be applied. Furthermore, there is no need for look up tables to convert bit stream mode numbers to logical numbers and vice versa.

- Since the mode numbers indicate their angle, design of a fast mode decision for a real time encoder may be much simpler.  A lower resolution of the directions can be tested first and based on the result, only modes close to the best one can be tested in the second stage. This is analogous to fast motion estimation.

- Some proposals such as the ones involving more than two MPMs, use direction which are adjacent to the neighbouring Intra mode. Such proposals can be greatly simplified since the adjacent directions are computed by just incrementing/decrementing the neighbouring mode number by one or two (depending on PU size).

# Proposed Method

The following Intra mode numbering is proposed: 0: Planar, 1: DC, and the rest of the angular directions are shown in Figure 1. As illustrated in the figure, the index of the scan can also be easily derived by calculating the distance of the mode from vertical and horizontal directions. Therefore, the look-up table aucIntraDirToScanIdx[MAX_CU_DEPTH][NUM_INTRA_MODE] can be removed.

Figure 1. Logical (and bit stream) Intra mode numbers.

Similarly, in Intra smoothing, the distance of the direction from both horizontal and vertical can be computed. Then the minimum of these distances is compared against a pre-determined value stored in a much smaller look-up table (of size 5 rather than 5x36 in current HM). If the minimum distance is larger than the value of the table for the corresponding block size then smoothing is applied. Therefore, the table can be reduced from m_aucIntraFilter[5][ NUM_INTRA_MODE] to m_aucIntraFilter[5] = {10, 7, 1, 0 , 10}. The value of 10 is large enough to prevent smoothing to be applied to any Intra mode for that block size.  Using this table, the modes for which smoothing is applied to are exactly the same as HM 5.0. The table size, however, is reduced.

Figure 2. Proposed 3 MPM decoding tree.

The tables for mode dependant transform (DCT/DST) can also be removed in a similar fashion. For 4x4 transform, vertical DST is applied to Planar and modes 11 through 34. Horizontal DST is, on the other hand, applied to Planar and modes 2 through 25. The rest of the modes use the default DCT transform. Hence, tables g_aucDCTDSTMode_Hor and g_aucDCTDSTMode_Vert can be safely removed.

In addition, with the logical mode numbers, the table to map logical and bit stream numbers, i.e., g_aucAngIntraModeOrder[NUM_INTRA_MODE],  is no longer needed.

One final point should be made with regards to 4x4 PU sizes. For 4x4 PUs only even mode angular numbers are allowed. This results in 17 angular modes plus planar and DC modes (total 19 modes). To code the mode number in the bit stream, the mode number is mapped to a smaller number at the last stage (just before coding into the bit stream) according to (iDir>>1) + (iDir>0) to compensate for odd mode numbers which are not allowed (blocked). After parsing at the decoder, for 4x4 PUs the parsed number is mapped back to the actual mode number considering that odd angular mode numbers are not allowed for 4x4 PUs.

# Extension to 3 MPMs

In HM 5.0, for 8x8, 16x16, and 32x32 PU sizes, there are 35 Intra modes. For 4x4 PUs however, 18 modes are considered. For a  full symmetric approach however, 19 modes should be considered for 4x4 PUs. With the full symmetric approach, there are 19 and 35  modes for 4x4 and 8x8/16x16/32x32 PUs, respectively, which corresponds to 3 modes in addition to a full power of 2. Consequentially, if we allow for 3 MPMs, there will be exactly 16 and 32 remaining modes for  4x4 and 8x8/16x16/32x32 PUs, respectively. This eliminates the use of any escape code that is currently needed in the software design. Figure 2 shows the the decoding tree of the proposed 3 MPM. It should be noted that the introduction of a third MPM has no additional complexity or memory/context requirements. The 3 MPMs are derived using simple operations without any look-up tables as seen in the table below:

| Neighbouring modes condition | MPMs (three MPMs) |
| --- | --- |
| iLeftIntraDir == iAboveIntraDir | iLeftIntraDir, iLeftIntraDir-, iLeftIntraDir+ |
| iLeftIntraDir != iAboveIntraDir | iLeftIntraDir, iAboveIntraDir, Planar/DC |

is 2 for 4x4 PUs and 1 for the rest of PU sizes.

# Results

The changes explained above have been implemented in HM5.0. For regularity, 19 luma Intra prediction modes are assumed for 4x4 PUs as mentioned in the previous section. This additional mode, however, has no additional gain (as verified by other contributions) and is just added for symmetry and better design. Table 1 shows the BD-rate performance of the proposed Intra mode renaming. The small gain is due to better mapping from 35 mode to 19 modes in 4x4 PUs as explained in previous section.

Table 1. Results for renaming Intra modes (2 MPM derived the same as HM5.0)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class F | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

If 3MPM is used the results is improved. Note that no additional context is defined even for the case of 3 MPMs. Table below shows the BD-rate performance of the proposed 3-MPM method under the common test condition.

Table 2. Results for Intra mode name modification + 3MPM.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.29% | -0.13% | 0.00% | -0.38% | -0.15% | -0.09% |
| Class B | -0.20% | -0.07% | -0.06% | -0.28% | -0.07% | -0.06% |
| Class C | -0.25% | -0.13% | -0.16% | -0.32% | -0.17% | -0.18% |
| Class D | -0.21% | -0.07% | -0.09% | -0.24% | -0.10% | -0.13% |
| Class E | -0.33% | -0.25% | -0.05% | -0.42% | -0.13% | -0.15% |
| Overall | -0.25% | -0.12% | -0.08% | -0.31% | -0.12% | -0.12% |
|  | -0.25% | -0.13% | -0.08% | -0.31% | -0.12% | -0.13% |
| Class F | -0.33% | -0.35% | -0.28% | -0.33% | -0.38% | -0.45% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).